We are using the following MotorControl SDK solution:

C:\ti\c2000\C2000Ware_MotorControl_SDK_5_04_00_00\solutions\tidm_02010_dmpfc

Currently, we are working in DMC_LEVEL_2 and running a compressor PMSM motor successfully.

Our motor parameters are configured as below:

#define USER_MOTOR1_TYPE                    MOTOR_TYPE_PM

#define USER_MOTOR1_NUM_POLE_PAIRS          (4)

#define USER_MOTOR1_Rr_Ohm                  (0.0)

#define USER_MOTOR1_Rs_Ohm                  (1.335f)

#define USER_MOTOR1_Ls_d_H                  (0.00895f)

#define USER_MOTOR1_Ls_q_H                  (0.01541f)

#define USER_MOTOR1_RATED_FLUX_VpHz         (0.5415546)

#define USER_MOTOR1_MAGNETIZING_CURRENT_A   (NULL)

#define USER_MOTOR1_RES_EST_CURRENT_A       (3.5)

#define USER_MOTOR1_IND_EST_CURRENT_A       (-3.0)

#define USER_MOTOR1_MAX_CURRENT_A           (9.0)

#define USER_MOTOR1_FLUX_EXC_FREQ_Hz        (20.0)

#define USER_MOTOR1_NUM_ENC_SLOTS           (2500.0)

#define USER_MOTOR1_INERTIA_Kgm2            (0.000315)

#define USER_MOTOR1_RATED_VOLTAGE_V         (200.0)

#define USER_MOTOR1_RATED_SPEED_KRPM        (3.0)

#define USER_MOTOR1_FREQ_MIN_Hz             (30.0)

#define USER_MOTOR1_FREQ_MAX_Hz             (360.0)

#define USER_MOTOR1_FREQ_LOW_Hz             (48.0)

#define USER_MOTOR1_FREQ_HIGH_Hz            (480.0f)

#define USER_MOTOR1_VOLT_MIN_V              (33.0)

#define USER_MOTOR1_VOLT_MAX_V              (220.0)

#define USER_MOTOR1_ALIGN_DELTA_A           (0.01f)

#define USER_MOTOR1_ALIGN_CURRENT_A         (2.0f)

#define USER_MOTOR1_STARTUP_CURRENT_A       (3.5)

#define USER_MOTOR1_OVER_CURRENT_A          (16.0)

#define USER_MOTOR1_LOCK_CURRENT_A          (1.0f)

#define USER_MOTOR1_SPEED_START_Hz          (25.0)

#define USER_MOTOR1_SPEED_FORCE_Hz          (20.0)

#define USER_MOTOR1_ACCEL_START_Hz          (10.0)

#define USER_MOTOR1_ACCEL_START_Hzps        (10.0f)

#define USER_MOTOR1_ACCEL_MAX_Hzps          (15.0)

// eSMO sliding mode gains

#define USER_MOTOR1_KSLIDE_MAX              (1.2f)

#define USER_MOTOR1_KSLIDE_MIN              (0.15f)

// PLL gain window and scale

#define USER_MOTOR1_PLL_KP_MAX              (10.0f)

#define USER_MOTOR1_PLL_KP_MIN              (2.0f)

#define USER_MOTOR1_PLL_KP_SF               (5.0f)

// BEMF detection & filtering

#define USER_MOTOR1_BEMF_THRESHOLD          (0.5f)

#define USER_MOTOR1_BEMF_KSLF_FC_Hz         (1.0f)

#define USER_MOTOR1_BEMF_KSLF_FC_SF         (1.0f)

// Angle/Speed filter

#define USER_MOTOR1_THETA_OFFSET_SF         (1.0f)

#define USER_MOTOR1_SPEED_LPF_FC_Hz         (400.0f)

// Vibration compensation

#define USER_MOTOR1_VIBCOMPA_ALPHA          (0.98f)

#define USER_MOTOR1_VIBCOMPA_GAIN           (0.0f)

#define USER_MOTOR1_VIBCOMPA_INDEX_DELTA    (10)

Current behavior:

- Up to speed_ref = 50

  - Phase current waveform is good

  - Generated angle (angleGenM1_rad) and estimated angle are matching properly

  - Motor rotation is smooth and stable

- Figure 1: Estimated angle good at lower speed

- When increasing speed_ref beyond around 65

  - Motor still rotates properly

  - Phase current and generated angle still appear good

  - But the estimated angle from ESMO becomes distorted/noisy as shown in the attached figure.

- Figure 2: Estimated angle distorted at higher speed.

Because of this issue, we are unable to confidently move to DMC_LEVEL_3.

We would like guidance on the following:

- Which ESMO parameters mainly affect estimated angle quality at higher speed?

- Which parameters should be tuned first to improve the estimated angle waveform?

- Is this issue mainly related to:

  - Kslide tuning.

  - PLL gains

  - BEMF filter coefficients

  - Current sensing noise

  - Voltage scaling

  - ADC sampling timing

  - Motor parameter mismatch (Rs/Ls/Flux)?

- Is the current value of USER_MOTOR1_BEMF_KSLF_FC_Hz = 1.0f too low for higher speed operation?

- What is the recommended tuning sequence before moving to DMC_LEVEL_3?

- Which internal ESMO variables should be monitored in CCS graph/watch window for debugging?