# How the ADCs are set up

I set up the ADCs using the following routine. This is called during initialization before the motor control ISR is enabled.

/***********************************************************

* Select the channels to convert and the

* start of conversion (SOC) flags

***********************************************************/

void SetupADC_Channels(void)

{

EALLOW;

// It is better to use the analog inputs ADCINA2, ADCINA4, ADCINB2 and ADCIN14

// since they are connected to the CMPSS and trip module.

// Analog signals that are sampled (Suggested connections)

// Ifb-SV  ADC A2

// Ifb-SW  ADC A4

// Ifb-SU  ADC B2

// Vfb-Bus ADC 14

// ACQPS is set to 31 based on the existing code. It seems there is

// intra-channel crosstalk between channels in the same ADC block.

// For example, ADCA2 and ADCA4 have crosstalk. But ADCB2 is not

// affected. ACQPS has a bearing on that.

// Configure the SOC0 on ADC a an b

// Shunt Motor Currents (SV) @  ADCIN14 (A2 is a better choice)

// ********************************

Ph_A_ADC.ADCSOC0CTL.bit.CHSEL     = Ph_A_ADC_Ch;    // SOC0 will convert pin ADCIN14 (Ph A)

Ph_A_ADC.ADCSOC0CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // 31 sample window in SYSCLK cycles

Ph_A_ADC.ADCSOC0CTL.bit.TRIGSEL   = 0;    // software trigger.

// Configure the post processing block (PPB) to eliminate subtraction related calculation

Ph_A_ADC.ADCPPB1CONFIG.bit.CONFIG = 0;    // PPB 1 is associated with SOC0

Ph_A_ADC.ADCPPB1OFFCAL.bit.OFFCAL = 0;    // Write zero to this. The ADC result in PPB

// is calculated by subtracting this value from

// the original ADC result.

Ph_A_ADC.ADCEVTSEL.bit.PPB3TRIPHI = 0;      // Setting this flag causes the ADCAEVT3 to be asserted when PPB's high limit is violated.

Ph_A_ADC.ADCEVTSEL.bit.PPB3TRIPLO = 0;      // Setting this flag causes the ADCAEVT3 to be asserted when PPB's low  limit is violated.

Ph_A_ADC.ADCINTSEL1N2.bit.INT1SEL = 0;    // EOC0 sets the interrupt flag INT1

Ph_A_ADC.ADCINTSEL1N2.bit.INT1E = 1;      // Enable interrupt 1

// Shunt Motor Currents (SW) @ ADCIN15 (A4 is a better choice)

// ********************************

Ph_B_ADC.ADCSOC1CTL.bit.CHSEL     = Ph_B_ADC_Ch;    // SOC1 will convert pin ADCINA2 (Ph B)

Ph_B_ADC.ADCSOC1CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

Ph_B_ADC.ADCSOC1CTL.bit.TRIGSEL   = 0;    // software trigger.

// Configure PPB to eliminate subtraction related calculation

Ph_B_ADC.ADCPPB2CONFIG.bit.CONFIG = 1;    // PPB 2 is associated with SOC1

Ph_B_ADC.ADCPPB2OFFCAL.bit.OFFCAL = 0;    // Write zero to this. The ADC result in PPB

// is calculated by subtracting this value from

// the original ADC result.

Ph_B_ADC.ADCEVTSEL.bit.PPB4TRIPHI = 0;      // Setting this flag causes the ADCBEVT4 to be asserted when PPB's high limit is violated.

Ph_B_ADC.ADCEVTSEL.bit.PPB4TRIPLO = 0;      // Setting this flag causes the ADCBEVT4 to be asserted when PPB's low  limit is violated.

Ph_B_ADC.ADCINTSEL1N2.bit.INT2SEL = 1;    // EOC1 sets the interrupt flag INT2

Ph_B_ADC.ADCINTSEL1N2.bit.INT2E = 1;      // Enable interrupt 1

// Shunt Motor Currents (SU) @ B0 (B2 is a better choice)

// ***************************

Ph_C_ADC.ADCSOC0CTL.bit.CHSEL     = Ph_C_ADC_Ch;    // SOC0 will convert pin B2, (Ph C)

Ph_C_ADC.ADCSOC0CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

Ph_C_ADC.ADCSOC0CTL.bit.TRIGSEL   = 0;    // software trigger.

// Configure PPB to eliminate subtraction related calculation

Ph_C_ADC.ADCPPB1CONFIG.bit.CONFIG = 0;    // PPB is associated with SOC0

Ph_C_ADC.ADCPPB1OFFCAL.bit.OFFCAL = 0;    // Write zero to this. The ADC result in PPB

// is calculated by subtracting this value from

// the original ADC result.

Ph_C_ADC.ADCEVTSEL.bit.PPB2TRIPHI = 0;    // Setting this flag causes the ADCCEVT2 to be asserted when PPB's high limit is violated.

Ph_C_ADC.ADCEVTSEL.bit.PPB2TRIPLO = 0;    // Setting this flag causes the ADCCEVT2 to be asserted when PPB's low  limit is violated.

Ph_C_ADC.ADCINTSEL1N2.bit.INT1SEL = 0;    // EOC0 sets the interrupt flag INT1

Ph_C_ADC.ADCINTSEL1N2.bit.INT1E = 1;      // Enable interrupt 1

// Bus Voltage Feedback at A1 (not used)

// **************************************

VMot_ADC.ADCSOC11CTL.bit.CHSEL    = VMot_ADC_Ch;     // SOC3 will convert pin A1

VMot_ADC.ADCSOC11CTL.bit.ACQPS    = ADC_SAMPLE_WINDOW;    // sample window in SYSCLK cycles

VMot_ADC.ADCSOC11CTL.bit.TRIGSEL  = 0;    // software trigger.

// All the following ADCs have to be software triggered.

// SOCs should be higher than 1. Trigger should be issues within the motor ISR

// after the current conversion is complete.

// Bus Current I-Total @ ADCINA4 on Terumo board

// ***************************

Itotal_ADC.ADCSOC10CTL.bit.CHSEL     = Itotal_ADC_Ch;    // SOC2 will convert pin A4. Results will be in ADCRESULT2

Itotal_ADC.ADCSOC10CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

Itotal_ADC.ADCSOC10CTL.bit.TRIGSEL   = 0;     // software triggered

//Itotal_ADC.ADCPPB3CONFIG.bit.CONFIG = 2;    // PPB 3 is associated with SOC2

//Itotal_ADC.ADCPPB3OFFCAL.bit.OFFCAL = 0;    // Write zero to this. The ADC result in PPB

// is calculated by subtracting this value from

// the original ADC result.

// phase voltage VA_ADC @ ADCINA4 on Terumo board

// ***************************

VA_ADC.ADCSOC4CTL.bit.CHSEL     = VA_ADC_Ch;    // SOC4 will convert pin A3. Results will be in ADCRESULT3

VA_ADC.ADCSOC4CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

VA_ADC.ADCSOC4CTL.bit.TRIGSEL   = 0;    // software triggered

// phase voltage VB_ADC @ ADCINA3 on Terumo board

// ***************************

VB_ADC.ADCSOC5CTL.bit.CHSEL     = VB_ADC_Ch;    // SOC5 will convert pin A15. Results will be in ADCRESULT5

VB_ADC.ADCSOC5CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

VB_ADC.ADCSOC5CTL.bit.TRIGSEL   = 0;    // software triggered

// phase voltage VA_ADC @ ADCINA2 on Terumo board

// ***************************

VC_ADC.ADCSOC5CTL.bit.CHSEL     = VC_ADC_Ch;    // SOC1 will convert pin B0. Results will be in ADCRESULT1

VC_ADC.ADCSOC5CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

VC_ADC.ADCSOC5CTL.bit.TRIGSEL   = 0;    // software triggered

// Motor temperature C_DEG_ADC @ ADCINB3 on Terumo board

// ***************************

C_DEG_ADC.ADCSOC2CTL.bit.CHSEL     = C_DEG_ADC_Ch;    // SOC2 will convert pin B3. Results will be in ADCRESULT2

C_DEG_ADC.ADCSOC2CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

C_DEG_ADC.ADCSOC2CTL.bit.TRIGSEL   = 0;    // software triggered

// 5V sense V_1p2_ADC @ ADCINB4 on Terumo board

// ***************************

V_1p2_ADC.ADCSOC3CTL.bit.CHSEL     = V_1p2_ADC_Ch;   // SOC3 will convert pin B4. Results will be in ADCRESULT3

V_1p2_ADC.ADCSOC3CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

V_1p2_ADC.ADCSOC3CTL.bit.TRIGSEL   = 0;    // software triggered

// 3.3V sense V_3p3_ADC @ ADCINB5 on Terumo board

// ***************************

V_3p3_ADC.ADCSOC4CTL.bit.CHSEL     = V_3p3_ADC_Ch;    // SOC4 will convert pin B5. Results will be in ADCRESULT4

V_3p3_ADC.ADCSOC4CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

V_3p3_ADC.ADCSOC4CTL.bit.TRIGSEL   = 0;    // software triggered

// Processor temperature sense PROC_TEMP_ADC @ ADCIN13 on all boards

// ***************************

PROC_TEMP_ADC.ADCSOC6CTL.bit.CHSEL     = PROC_TEMP_ADC_Ch;    // SOC6 will convert pin A13. Results will be in ADCRESULT6

PROC_TEMP_ADC.ADCSOC6CTL.bit.ACQPS     = ADC_SAMPLE_WINDOW;   // sample window in SYSCLK cycles

PROC_TEMP_ADC.ADCSOC6CTL.bit.TRIGSEL   = 0;    // software triggered

/*

Advisory ADC: Random Conversion Errors

Revision(s) Affected:  B (REVID Address: (*0x5D00C) = 0x02)

Details:

The ADC may have errors at a rate as high as 1 in 106.5 ADC conversions in 12-bit mode

and as high as 1 in 108.75 conversions in 16-bit mode. When a conversion error occurs, it

will be a significant random jump in the digital output of the ADC without a corresponding

change in the ADC input voltage, otherwise known as a "sparkle code". The magnitude

of this jump will typically be in the range of 20 LSBs to 200 LSBs; however, larger or

smaller jumps may occur.

Workaround(s): For the revisions affected, the error rate will be lower than 1 error

in 10.0E+14.5 ADC conversions for both 12-bit mode and 16-bit mode when all of the following

configurations are used:

- The S+H duration is at least 320 ns

- ADCCLK is 40 MHz or less

- ADCCLK prescale is a whole number: /1.0, /2.0, /3.0, /4.0, /5.0, /6.0, /7.0, or /8.0

- The value of 0x7000 is written to memory locations 0x0000 743F, 0x0000 74BF,

0x0000 753F, and 0x0000 75BF (writing this value is only valid when the ADCCLK

prescale is a whole number).

*/

// ******************************************************

// static analog trim for all ADCs (A, B, C and D)

// *******************************************************

adc_cal=(int16*)0x0000743F;

*adc_cal=0x7000;

adc_cal=(int16*)0x000074BF;

*adc_cal=0x7000;

adc_cal=(int16*)0x0000753F;

*adc_cal=0x7000;

adc_cal=(int16*)0x000075BF;

*adc_cal=0x7000;

EDIS;

}

I also have the following routine to enable the ADC PPB trip-zone. This is called after the ADC offset is calibrated.

/*****************************************************************

* Enable ADC PPB trip-zone

*****************************************************************/

void enableADC_PPB_Tripzone(void)

{

Ph_A_ADC.ADCPPB3CONFIG.bit.CONFIG = 0;    // PPB 3 is associated with SOC0

Ph_A_ADC.ADCPPB3OFFCAL.bit.OFFCAL = 0;

Ph_A_ADC.ADCPPB3TRIPHI.bit.LIMITHI = ADC_PPB_HI_LIMIT;  // Hi limit of ADC PPB output

Ph_A_ADC.ADCPPB3TRIPHI.bit.HSIGN   = 0;                 // sign bit

Ph_A_ADC.ADCPPB3TRIPLO.bit.LIMITLO = ADC_PPB_LO_LIMIT;  // Lo limit of ADC PPB output

Ph_A_ADC.ADCPPB3TRIPLO.bit.LSIGN   = 0;                 // sign bit

Ph_A_ADC.ADCEVTSEL.bit.PPB3TRIPHI = 1;      // Setting this flag causes the ADCAEVT3 to be asserted when PPB's high limit is violated.

Ph_A_ADC.ADCEVTSEL.bit.PPB3TRIPLO = 1;      // Setting this flag causes the ADCAEVT3 to be asserted when PPB's low  limit is violated.

Ph_B_ADC.ADCPPB4CONFIG.bit.CONFIG = 1;    // PPB 4 is associated with SOC1

Ph_B_ADC.ADCPPB4OFFCAL.bit.OFFCAL = 0;

Ph_B_ADC.ADCPPB4TRIPHI.bit.LIMITHI = ADC_PPB_HI_LIMIT;  // Hi limit of ADC PPB output

Ph_B_ADC.ADCPPB4TRIPHI.bit.HSIGN   = 0;                 // sign bit

Ph_B_ADC.ADCPPB4TRIPLO.bit.LIMITLO = ADC_PPB_LO_LIMIT;  // Lo limit of ADC PPB output

Ph_B_ADC.ADCPPB4TRIPLO.bit.LSIGN   = 0;                 // sign bit

Ph_B_ADC.ADCEVTSEL.bit.PPB4TRIPHI = 1;      // Setting this flag causes the ADCBEVT4 to be asserted when PPB's high limit is violated.

Ph_B_ADC.ADCEVTSEL.bit.PPB4TRIPLO = 1;      // Setting this flag causes the ADCBEVT4 to be asserted when PPB's low  limit is violated.

Ph_C_ADC.ADCPPB2CONFIG.bit.CONFIG = 0;    // PPB 2 is associated with SOC0

Ph_C_ADC.ADCPPB2OFFCAL.bit.OFFCAL = 0;

Ph_C_ADC.ADCPPB2TRIPHI.bit.LIMITHI = ADC_PPB_HI_LIMIT;  // Hi limit of ADC PPB output

Ph_C_ADC.ADCPPB2TRIPHI.bit.HSIGN   = 0;                 // sign bit

Ph_C_ADC.ADCPPB2TRIPLO.bit.LIMITLO = ADC_PPB_LO_LIMIT;  // Lo limit of ADC PPB output

Ph_C_ADC.ADCPPB2TRIPLO.bit.LSIGN   = 0;                 // sign bit

Ph_C_ADC.ADCEVTSEL.bit.PPB2TRIPHI = 1;    // Setting this flag causes the ADCCEVT2 to be asserted when PPB's high limit is violated.

Ph_C_ADC.ADCEVTSEL.bit.PPB2TRIPLO = 1;    // Setting this flag causes the ADCCEVT2 to be asserted when PPB's low  limit is violated.

}

# How the trip-zones are set up

/*******************************************************************************

*

* Description:

*    This file contains routines to set up the trip zones.

*

*******************************************************************************/

//----------------------------------------------------------------------------------

//  Copyright Texas Instruments (C) 2004-2015

//----------------------------------------------------------------------------------

#include "Faults.h"

#include <math.h>

#include <stdint.h>

#include <stdbool.h>

#include <limits.h>

#include "Terumo_PM_Servo_F2837x.h"

#include "type.h"

#include "Terumo_HW.h"

#include "F2837xS_device.h"

#include "TripZone.h"

#include "Watchdog.h"

#include "MsgEvents.h"

// ****************************************************************************

// ****************************************************************************

// DMC Protection Against Over Current Protection

// ****************************************************************************

// ****************************************************************************

#if TRIP_ZONE_ACTIVE

//definitions for selecting DACH reference

#define REFERENCE_VDDA     0

//definitions for COMPH input selection

#define NEGIN_DAC          0

#define NEGIN_PIN          1

//definitions for CTRIPH/CTRIPOUTH output selection

#define CTRIP_ASYNCH       0

#define CTRIP_SYNCH        1

#define CTRIP_FILTER       2

#define CTRIP_LATCH        3

#endif

// Faults set by motor ISR

bool   gADC_Fault = false;

Uint16 gADC_ConversionError = 0;

bool   gHALL_Fault = false;

bool   gMotorDirectionFA = false;

bool   gMotorEncoderFA = false;

bool   gMotorErraticFA = false;

bool   gMotorIndexFA = false;

bool   gMotorVibratingFA = false;

bool     gElectThetaRationalityFA = false;

float32  gEncoderFaultPos = 0.0;

bool     gEncoderDirtyFA = false;

Uint32   gEncoderDirtyValue = 0;

bool   gPWM_Tripped = false;

#if TRIP_ZONE_ACTIVE

Uint16 gPWM_TrippedByQEP = 0;

Uint16 gPWM_TrippedByClkFail = 0;

Uint16 gPWM_TrippedByOverCurrnt = 0;

Uint16 gPWM_TrippedByADC_PPB = 0;

Uint16 gPWM_TrippedByECC_Err = 0;

Uint16 gTZ_FilterThresh      = 18;

float32  gTZ_CurrentLimit = INSTANTANEOUS_CURRENT_LIMIT;

// CMPSS parameters for Over Current Protection

static Uint16  TZ_FilterClkPrescale = 20,

TZ_FilterSampleWindow     = 30,

SHUNT_curHi = SHUNT(INSTANTANEOUS_CURRENT_LIMIT),

SHUNT_curLo = SHUNT(INSTANTANEOUS_CURRENT_LIMIT);

static Uint16 TOTAL_curHi = TOTAL(INSTANTANEOUS_TOTAL_CURRENT_LIMIT),

TOTAL_curLo = TOTAL(INSTANTANEOUS_TOTAL_CURRENT_LIMIT);

Uint16 ClearPWM_Trip = 0;

float32 TZ_ItotalCurrentLimit = INSTANTANEOUS_TOTAL_CURRENT_LIMIT;

void  Modify_CMPSS_DigitalFilter(volatile struct CMPSS_REGS *v, Uint16 curHi, Uint16 curLo);

void  Config_CMPSS_DigitalFilter(volatile struct CMPSS_REGS *v, int16 Hi, int16 Lo);

void ClearTripZoneRegisterFalgs(void);

//---------------------------------------------------

// Configure the analog comparators

// Inputs to the analog comparators come from ADC

// input pins.

//---------------------------------------------------

/* Eight Comparator Subsystems (CMPSS)

* Each CMPSS has: Two analog comparators, Two programmable 12-bit DACs

*                 Two digital filters and Ramp generator

*

* Digital filter used to remove spurious trip signals (majority vote)

* Ramp generator used peak current mode control

* Ability to synchronize with PWMSYNC event. */

/* The signal path is as follows:

|\

CMPINxP-------------------------------------------->|+\                |-->CTRIPH

|                                          |  |--->COMPHINV-->|

|  DACHVALS--->12 Bit DAC-->COMPHSOURCE--->|-/       =0       |-->CTRIPOUTH

|                                          |/

|

|

|                                          |\

|----------------------------------------->|+\                |-->CTRIPL

|  |--->COMPHINV-->|

DACHVALS--->12 Bit DAC-->COMPLSOURCE--->|-/        =1      |-->CTRIPOUTL

|/

CTRIPH and CTRIPL are fed into ePWMXBAR. Then ePWMXBAR is used to route

them to Trip In 4.

*/

void Config_CMPSS_DigitalFilter(volatile struct CMPSS_REGS *v, int16 Hi, int16 Lo)

{

EALLOW;

// Set up COMPCTL register

v->COMPCTL.bit.COMPDACE    = 1;             // Enable CMPSS

v->COMPCTL.bit.COMPLSOURCE = NEGIN_DAC;     // Negative signal of the comparator is from internal DAC for COMP-L

v->COMPCTL.bit.COMPHSOURCE = NEGIN_DAC;     // Negative signal of the comparator is from internal DAC for COMP-H

v->COMPCTL.bit.COMPHINV    = 0;             // COMP-H output is NOT inverted

v->COMPCTL.bit.COMPLINV    = 1;             // COMP-L output is inverted

v->COMPCTL.bit.ASYNCHEN    = 0;             // Disable aynch COMP-H ouput

v->COMPCTL.bit.ASYNCLEN    = 0;             // Disable aynch COMP-L ouput

/* Low comparator CTRIPOUTL source select.

0 Asynchronous comparator output drives CTRIPOUTL

1 Synchronous comparator output drives CTRIPOUTL

2 Output of digital filter drives CTRIPOUTL

3 Latched output of digital filter drives CTRIPOUTL */

if(gTZ_FilterThresh > 0)

{

v->COMPCTL.bit.CTRIPHSEL    = CTRIP_FILTER; // Dig filter output ==> CTRIPH

v->COMPCTL.bit.CTRIPOUTHSEL = CTRIP_FILTER; // Dig filter output ==> CTRIPOUTH

v->COMPCTL.bit.CTRIPLSEL    = CTRIP_FILTER; // Dig filter output ==> CTRIPL

v->COMPCTL.bit.CTRIPOUTLSEL = CTRIP_FILTER; // Dig filter output ==> CTRIPOUTL

}

else

{

v->COMPCTL.bit.CTRIPHSEL    = CTRIP_ASYNCH; // Dig filter output ==> CTRIPH

v->COMPCTL.bit.CTRIPOUTHSEL = CTRIP_ASYNCH; // Dig filter output ==> CTRIPOUTH

v->COMPCTL.bit.CTRIPLSEL    = CTRIP_ASYNCH; // Dig filter output ==> CTRIPL

v->COMPCTL.bit.CTRIPOUTLSEL = CTRIP_ASYNCH; // Dig filter output ==> CTRIPOUTL

}

// Set up COMPHYSCTL register

v->COMPHYSCTL.bit.COMPHYS   = 2; // COMP hysteresis set to 2x typical value

// set up COMPDACCTL register

v->COMPDACCTL.bit.SELREF    = 0; // VDDA is REF for CMPSS DACs

v->COMPDACCTL.bit.SWLOADSEL = 0; // DAC updated on sysclock

v->COMPDACCTL.bit.DACSOURCE = 0; // Ramp bypassed

EDIS;

// Load DACs - High and Low

v->DACHVALS.bit.DACVAL = Hi;     // Set DAC-H to allowed MAX +ve current

v->DACLVALS.bit.DACVAL = Lo;     // Set DAC-L to allowed MAX -ve current

EALLOW;

// digital filter settings - HIGH side

v->CTRIPHFILCLKCTL.bit.CLKPRESCALE = TZ_FilterClkPrescale;                // set time between samples, max : 1023

v->CTRIPHFILCTL.bit.SAMPWIN        = TZ_FilterSampleWindow;               // # of samples in window, max : 31

v->CTRIPHFILCTL.bit.THRESH         = gTZ_FilterThresh;                    // recommended : gTZ_FilterThresh > TZ_FilterSampleWindow/2

v->CTRIPHFILCTL.bit.FILINIT        = 1;                                   // Init samples to filter input value

// digital filter settings - LOW side

v->CTRIPLFILCLKCTL.bit.CLKPRESCALE = TZ_FilterClkPrescale;                // set time between samples, max : 1023

v->CTRIPLFILCTL.bit.SAMPWIN        = TZ_FilterSampleWindow;               // # of samples in window, max : 31

v->CTRIPLFILCTL.bit.THRESH         = gTZ_FilterThresh;                    // recommended : gTZ_FilterThresh > TZ_FilterSampleWindow/2

v->CTRIPLFILCTL.bit.FILINIT        = 1;                                   // Init samples to filter input value

// Clear the status register for latched comparator events

v->COMPSTSCLR.bit.HLATCHCLR = 1;

v->COMPSTSCLR.bit.LLATCHCLR = 1;

EDIS;

return;

}

/*********************************************************************

* F28377S is capable of tripping the PWM outputs when overcurrent

* is detected. It does so without the intervention of the cpu.

* This routine sets up the CMPSS and PWM-xbar

*

* This is the part 1 of this routine. The following trip-zone

* triggers are not set up here since they cause the PWM to trip

* after power on test. They are set up in the part 2 of the routine.

*

*  PWM_Ph_A.DCBHTRIPSEL.bit.TRIPINPUT14 = 1;          // ECCDBLERR

*  PWM_Ph_A.TZDCSEL.bit.DCBEVT1         = TZ_DCBH_HI; // 010: DCBH = high, DCBL = don't care

*  PWM_Ph_B.DCBHTRIPSEL.bit.TRIPINPUT14 = 1;          // ECCDBLERR

*  PWM_Ph_B.TZDCSEL.bit.DCBEVT1         = TZ_DCBH_HI; // 010: DCBH = high, DCBL = don't care

*  PWM_Ph_C.DCBHTRIPSEL.bit.TRIPINPUT14 = 1;          // ECCDBLERR

*  PWM_Ph_C.TZDCSEL.bit.DCBEVT1         = TZ_DCBH_HI; // 010: DCBH = high, DCBL = don't care

*

*********************************************************************/

void InitOverCurrentTrip_Pt1(void)

{

// F28377S 100 pin package has only CMPIN1P, CMPIN2P, CMPIN3P and CMPIN4P

// So, it is better to use the analog inputs ADCINA2, ADCINA4, ADCINB2 and

// ADCIN14 respectively. Configure GPIO used for Trip Mechanism

// ADC inputs are connected to the CMPSS inputs as well.

// Shunt Current phase V(ADCINA14 = CMPIN4P), phase W(ADCINA2 = CMPIN1P),

// phase U(ADCINB2 = CMPIN3P), and total current(ADCIN4 = CMPIN2P)

// High Low Compare event trips

SHUNT_curHi = ADC_MID_POINT + SHUNT(gTZ_CurrentLimit);

SHUNT_curLo = ADC_MID_POINT - SHUNT(gTZ_CurrentLimit);

TOTAL_curHi = ADC_MID_POINT + TOTAL(TZ_ItotalCurrentLimit);

TOTAL_curLo = ADC_MID_POINT - TOTAL(TZ_ItotalCurrentLimit);

// CMPIN1P is connected to Cmpss1Regs.

// CMPIN2P is connected to Cmpss3Regs.

// CMPIN3P is connected to Cmpss4Regs.

Config_CMPSS_DigitalFilter(&Ph_C_CMPSS, SHUNT_curHi, SHUNT_curLo);     //Enable CMPSS3 - Shunt U (Phase C)

Config_CMPSS_DigitalFilter(&Ph_B_CMPSS, SHUNT_curHi, SHUNT_curLo);     //Enable CMPSS1 - Shunt W (Phase B)

Config_CMPSS_DigitalFilter(&Ph_A_CMPSS, SHUNT_curHi, SHUNT_curLo);     //Enable CMPSS4 - Shunt V (Phase A)

Config_CMPSS_DigitalFilter(&I_TOTAL_CMPSS, TOTAL_curHi, TOTAL_curLo);  //Enable CMPSS2 - I_Total

// CMPSS's CTRIPH and CTRIPL outputs are connected to the ePWM-xbar.

// Configure TRIP 4 to OR the High and Low trips from CMPSS comparators 1, 3, 4 and 2.

// Trip 4 is used since that is the first trip signal coming out of ePWM-xbar.

EALLOW;

// Clear everything first

EPwmXbarRegs.TRIP4MUX0TO15CFG.all  = 0x0000;

EPwmXbarRegs.TRIP4MUX16TO31CFG.all = 0x0000;

// For CMPSS's (CTRIPL OR CTRIPH) conditions, the muxes used are as follows.

// For CMPSS1.CTRIPH_OR_CTRIPL - mux 0

// For CMPSS2.CTRIPH_OR_CTRIPL - mux 2

// For CMPSS3.CTRIPH_OR_CTRIPL - mux 4

// For CMPSS4.CTRIPH_OR_CTRIPL - mux 6

// For CMPSS5.CTRIPH_OR_CTRIPL - mux 8

// For CMPSS6.CTRIPH_OR_CTRIPL - mux 10

// For all these muxes, the mux control input should be set to 1

// in order to route the signals to Trip4.

// Select the control inputs of the above muxes.

EPwmXbarRegs.TRIP4MUX0TO15CFG.bit.MUX6   = 1;  //cmpss4 - tripH or tripL

EPwmXbarRegs.TRIP4MUX0TO15CFG.bit.MUX4   = 1;  //cmpss3 - tripH or tripL

EPwmXbarRegs.TRIP4MUX0TO15CFG.bit.MUX0   = 1;  //cmpss1 - tripH or tripL

EPwmXbarRegs.TRIP4MUX0TO15CFG.bit.MUX2   = 1;  //cmpss2 - tripH or tripL

// Disable all the muxes first

EPwmXbarRegs.TRIP4MUXENABLE.all = 0x0000;

// Enable Mux 0, 4 and 6 to generate TRIP4 signal.

EPwmXbarRegs.TRIP4MUXENABLE.bit.MUX6  = 1;

EPwmXbarRegs.TRIP4MUXENABLE.bit.MUX4  = 1;

EPwmXbarRegs.TRIP4MUXENABLE.bit.MUX0  = 1;

EPwmXbarRegs.TRIP4MUXENABLE.bit.MUX2  = 1;

// Route the trip signals to GPIO using output xbar

// The following configures only output 1.

// That means all trip signals are ORed and output to

// just one GPIO pin.

OutputXbarRegs.OUTPUT1MUX0TO15CFG.all = 0;

OutputXbarRegs.OUTPUT1MUX16TO31CFG.all = 0;

OutputXbarRegs.OUTPUT1MUXENABLE.all = 0;

// Route the CMPSS1.CTRIPOUTH_OR_CTRIPOUTL

// CMPSS3.CTRIPOUTH_OR_CTRIPOUTL and

// CMPSS6.CTRIPOUTH_OR_CTRIPOUTL to MUX0, 2, 4 and 6 of OutputXbar1

OutputXbarRegs.OUTPUT1MUX0TO15CFG.bit.MUX0  = 1;

OutputXbarRegs.OUTPUT1MUX0TO15CFG.bit.MUX4  = 1;

OutputXbarRegs.OUTPUT1MUX0TO15CFG.bit.MUX6  = 1;

OutputXbarRegs.OUTPUT1MUX0TO15CFG.bit.MUX2  = 1;

OutputXbarRegs.OUTPUT1MUXENABLE.bit.MUX0  = 1;

OutputXbarRegs.OUTPUT1MUXENABLE.bit.MUX4  = 1;

OutputXbarRegs.OUTPUT1MUXENABLE.bit.MUX6  = 1;

OutputXbarRegs.OUTPUT1MUXENABLE.bit.MUX2  = 1;

OutputXbarRegs.OUTPUTLATCHENABLE.bit.OUTPUT1 = 0;

OutputXbarRegs.OUTPUTLATCHCLR.bit.OUTPUT1 = 1;

OutputXbarRegs.OUTPUTINV.bit.OUTPUT1 = 0;

// Route OutputXbar1 to GPIO2

// GPIO2 in Terumo is not connected to anything.

// So, GPIO2 indicates the state of the Trip 4

GpioCtrlRegs.GPAGMUX1.bit.GPIO2 =  0;

GpioCtrlRegs.GPAPUD.bit.GPIO2 =   1;  // pull up disabled // in Terumo only

GpioCtrlRegs.GPAMUX1.bit.GPIO2 =  1;

GpioCtrlRegs.GPADIR.bit.GPIO2 =   1;

GpioCtrlRegs.GPAGMUX1.bit.GPIO2 =  1;

/**********************************************/

/* ADC Post Processing Block limit violations */

/* These assert TRIP5                         */

/**********************************************/

// Clear everything first

EPwmXbarRegs.TRIP5MUX0TO15CFG.all  = 0x0000;

EPwmXbarRegs.TRIP5MUX16TO31CFG.all = 0x0000;

#if 1

// Set up PWM X-Bar to channel ADC limit violations in ADC PPB 3, 4 and 2

// (i.e. ADC Post Processing Block 3, 4 and 2) into TRIP5.

EPwmXbarRegs.TRIP5MUX0TO15CFG.bit.MUX4   = 2;  //ADCAEVT3

EPwmXbarRegs.TRIP5MUX0TO15CFG.bit.MUX6   = 2;  //ADCAEVT4

EPwmXbarRegs.TRIP5MUX0TO15CFG.bit.MUX10  = 2;  //ADCBEVT2

#endif

// Disable all the muxes first

EPwmXbarRegs.TRIP5MUXENABLE.all = 0x0000;

#if 1

// Enable Mux 4, 6 and 10 to generate TRIP5 signal.

EPwmXbarRegs.TRIP5MUXENABLE.bit.MUX4  = 1;

EPwmXbarRegs.TRIP5MUXENABLE.bit.MUX6  = 1;

EPwmXbarRegs.TRIP5MUXENABLE.bit.MUX10 = 1;

#endif

/* Signal path from here is as follows:

* TRIPIN4 is connected to Digital Compare Filter A via DCTRIPSEL.bit.DCAHCOMPSEL.

* The output of the filter (DCAEVT1) is connected to the PWMxRegs' trip via

* DCACTL. The trip signals will cause the PWMA output to be tripped.    */

// Set the ePWM7, ePWM8 and ePWM9 to be tripped by TRIP4 if there is over-current.

// One-shot mode is used so that PWM outputs will remain low

// once they are tripped.

PWM_Ph_A.DCTRIPSEL.bit.DCAHCOMPSEL = 15; //Trip combination input (all trip inputs selected by DCAHTRIPSEL register ORed together)

PWM_Ph_A.DCAHTRIPSEL.bit.TRIPINPUT4  = 1;          // Overcurrent trip

PWM_Ph_A.DCAHTRIPSEL.bit.TRIPINPUT5  = 1;          // TRIPIN5 - This is asserted when the ADC OPPB has limit violations.

PWM_Ph_A.TZDCSEL.bit.DCAEVT1       = TZ_DCBH_HI;   // 010: DCAH = high, DCAL = don't care

PWM_Ph_A.DCACTL.bit.EVT1SRCSEL     = DC_EVT1;      // DCAEVT1 Source Signal Select, 0: Source Is DCAEVT1 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_A.DCACTL.bit.EVT1FRCSYNCSEL = 0;            // DCAEVT1 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

//with EPWMCLK

PWM_Ph_A.TZDCSEL.bit.DCAEVT2       = TZ_DCBH_HI;   // 010: DCAH = high, DCAL = don't care

PWM_Ph_A.DCACTL.bit.EVT2SRCSEL     = DC_EVT2;      // DCAEVT2 Source Signal Select, 0: Source Is DCAEVT2 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_A.DCACTL.bit.EVT2FRCSYNCSEL = 0;            // DCAEVT2 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

// with EPWMCLK

PWM_Ph_B.DCTRIPSEL.bit.DCAHCOMPSEL = 15; //Trip combination input (all trip inputs selected by DCAHTRIPSEL register ORed together)

PWM_Ph_B.DCAHTRIPSEL.bit.TRIPINPUT4  = 1;          // Overcurrent trip

PWM_Ph_B.DCAHTRIPSEL.bit.TRIPINPUT5  = 1;          // TRIPIN5 - This is asserted when the ADC OPPB has limit violations.

PWM_Ph_B.TZDCSEL.bit.DCAEVT1       = TZ_DCBH_HI;

PWM_Ph_B.DCACTL.bit.EVT1SRCSEL     = DC_EVT1;

PWM_Ph_B.DCACTL.bit.EVT1FRCSYNCSEL = 0;

PWM_Ph_B.TZDCSEL.bit.DCAEVT2       = TZ_DCBH_HI;   // 010: DCAH = high, DCAL = don't care

PWM_Ph_B.DCACTL.bit.EVT2SRCSEL     = DC_EVT2;      // DCAEVT2 Source Signal Select, 0: Source Is DCAEVT2 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_B.DCACTL.bit.EVT2FRCSYNCSEL = 0;            // DCAEVT2 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

// with EPWMCLK

PWM_Ph_C.DCTRIPSEL.bit.DCAHCOMPSEL = 15; //Trip combination input (all trip inputs selected by DCAHTRIPSEL register ORed together)

PWM_Ph_C.DCAHTRIPSEL.bit.TRIPINPUT4  = 1;          // Overcurrent trip

PWM_Ph_C.DCAHTRIPSEL.bit.TRIPINPUT5  = 1;          // TRIPIN5 - This is asserted when the ADC OPPB has limit violations.

PWM_Ph_C.TZDCSEL.bit.DCAEVT1       = TZ_DCBH_HI;

PWM_Ph_C.DCACTL.bit.EVT1SRCSEL     = DC_EVT1;

PWM_Ph_C.DCACTL.bit.EVT1FRCSYNCSEL = 0;

PWM_Ph_C.TZDCSEL.bit.DCAEVT2       = TZ_DCBH_HI;   // 010: DCAH = high, DCAL = don't care

PWM_Ph_C.DCACTL.bit.EVT2SRCSEL     = DC_EVT2;      // DCAEVT2 Source Signal Select, 0: Source Is DCAEVT2 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_C.DCACTL.bit.EVT2FRCSYNCSEL = 0;            // DCAEVT2 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

// with EPWMCLK

#if 1

/* Signal path here is as follows:

* OR the trip signals 1 to 15 using DCTRIPSEL.bit.DCBHCOMPSEL and connect the

* output to the Digital Compare Filter B. However, not all

* trip signals are input into the OR gate. The trip signals input into to the OR

* gate are selected using DCBHTRIPSEL. The output of the filter (DCBEVT1) is

* connected to the PWMxRegs' trip via DCBCTL. The trip signals will cause the

* PWMB output to be tripped.

*

* Trip inputs, TZ4 and TZ5 are special. They are connected to the PWMxRegs'

* trip directly via TZSEL.     */

// Set the ePWM1, ePWM2 and ePWM 3 to be tripped by all other trip signals.

PWM_Ph_A.DCTRIPSEL.bit.DCBHCOMPSEL   = 15;         // Trip combination input (all trip inputs selected by DCBHTRIPSEL register ORed together)

PWM_Ph_A.DCBHTRIPSEL.bit.TRIPINPUT14 = 1;          // ECCDBLERR - This is enabled separately

PWM_Ph_A.TZDCSEL.bit.DCBEVT1         = TZ_DCBH_HI; // 010: TZ_DCBH_HI = high, TZ_EVT_DISABLE, DCBL = don't care

PWM_Ph_A.DCBCTL.bit.EVT1SRCSEL       = DC_EVT1;    // DCBEVT1 Source Signal Select, 0: Source Is DCBEVT1 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_A.DCBCTL.bit.EVT1FRCSYNCSEL   = 0;          // DCBEVT1 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

// with EPWMCLK

PWM_Ph_A.TZDCSEL.bit.DCBEVT2         = TZ_DCBH_HI; // 010: DCBH = high, DCBL = don't care

PWM_Ph_A.DCBCTL.bit.EVT2SRCSEL       = DC_EVT2;    // DCBEVT2 Source Signal Select, 0: Source Is DCBEVT2 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_A.DCBCTL.bit.EVT2FRCSYNCSEL   = 0;          // DCBEVT2 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

// with EPWMCLK

PWM_Ph_B.DCTRIPSEL.bit.DCBHCOMPSEL   = 15;         // Trip combination input (all trip inputs selected by DCBHTRIPSEL register ORed together)

PWM_Ph_B.DCBHTRIPSEL.bit.TRIPINPUT14 = 1;          // ECCDBLERR - This is enabled separately

PWM_Ph_B.TZDCSEL.bit.DCBEVT1         = TZ_DCBH_HI; // 010: DCBH = high, DCBL = don't care

PWM_Ph_B.DCBCTL.bit.EVT1SRCSEL       = DC_EVT1;    // DCBEVT1 Source Signal Select, 0: Source Is DCBEVT1 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_B.DCBCTL.bit.EVT1FRCSYNCSEL   = 0;          // DCBEVT1 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

// with EPWMCLK

PWM_Ph_B.TZDCSEL.bit.DCBEVT2         = TZ_DCBH_HI; // 010: DCBH = high, DCBL = don't care

PWM_Ph_B.DCBCTL.bit.EVT2SRCSEL       = DC_EVT2;    // DCBEVT2 Source Signal Select, 0: Source Is DCBEVT2 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_B.DCBCTL.bit.EVT2FRCSYNCSEL   = 0;          // DCBEVT2 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

// with EPWMCLK

PWM_Ph_C.DCTRIPSEL.bit.DCBHCOMPSEL   = 15;         // Trip combination input (all trip inputs selected by DCBHTRIPSEL register ORed together)

PWM_Ph_C.DCBHTRIPSEL.bit.TRIPINPUT14 = 1;          // ECCDBLERR - This is enabled separately

PWM_Ph_C.TZDCSEL.bit.DCBEVT1         = TZ_DCBH_HI; // 010: DCBH = high, DCBL = don't care

PWM_Ph_C.DCBCTL.bit.EVT1SRCSEL       = DC_EVT1;    // DCBEVT1 Source Signal Select, 0: Source Is DCBEVT1 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_C.DCBCTL.bit.EVT1FRCSYNCSEL   = 0;          // DCBEVT1 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

// with EPWMCLK

PWM_Ph_C.TZDCSEL.bit.DCBEVT2         = TZ_DCBH_HI; // 010: DCBH = high, DCBL = don't care

PWM_Ph_C.DCBCTL.bit.EVT2SRCSEL       = DC_EVT2;    // DCBEVT2 Source Signal Select, 0: Source Is DCBEVT2 Signal, 1: Source Is DCEVTFILT Signal

PWM_Ph_C.DCBCTL.bit.EVT2FRCSYNCSEL   = 0;          // DCBEVT2 Force Synchronization Signal Select, 0: Source is passed through asynchronously, 1: Source is synchronized

// with EPWMCLK

PWM_Ph_A.TZSEL.bit.CBC6 = 0x1; // Emulator Stop is derived from TZ6 input (This is enabling cycle by cycle disablement for TZ6 input

PWM_Ph_B.TZSEL.bit.CBC6 = 0x1; // Emulator Stop is derived from TZ6 input (This is enabling cycle by cycle disablement for TZ6 input

PWM_Ph_C.TZSEL.bit.CBC6 = 0x1; // Emulator Stop is derived from TZ6 input (This is enabling cycle by cycle disablement for TZ6 input

// Set the method to clear cycle by cycle trip flags

PWM_Ph_A.TZCLR.bit.CBCPULSE = 0;

PWM_Ph_B.TZCLR.bit.CBCPULSE = 0;

PWM_Ph_C.TZCLR.bit.CBCPULSE = 0;

#endif

EDIS;

// What do we want the OST/CBC events to do?

// TZA events can force EPWMxA

// TZB events can force EPWMxB

SetTripZoneOutputAction(TRIPZONE_OUTPUT_MODE_ON_RESET);

// Clear any spurious OV trip

ClearTripZoneRegisterFalgs();

}

/**********************************************************************

* This is the part 2 of this routine. The following trip-zone

* triggers are set up after PWM ISR is enabled. Otherwise, they cause

* the PWM to trip after power on test.

***********************************************************************/

void InitOverCurrentTrip_Pt2(void)

{

EALLOW;

PWM_Ph_A.TZSEL.bit.DCAEVT2         = 1;            // Enable DCAEVT2 as the trip source (cycle by cycle)

PWM_Ph_B.TZSEL.bit.DCAEVT2         = 1;            // Enable DCAEVT2 as the trip source (cycle by cycle)

PWM_Ph_C.TZSEL.bit.DCAEVT2         = 1;            // Enable DCAEVT2 as the trip source (cycle by cycle)

PWM_Ph_A.TZSEL.bit.DCBEVT2         = 0;            // Enable DCBEVT2 as the trip source (cycle by cycle)

PWM_Ph_B.TZSEL.bit.DCBEVT2         = 0;            // Enable DCBEVT2 as the trip source (cycle by cycle)

PWM_Ph_C.TZSEL.bit.DCBEVT2         = 0;            // Enable DCBEVT2 as the trip source (cycle by cycle)

PWM_Ph_A.TZSEL.bit.DCAEVT1         = 0;            // Enable DCAEVT1 as the trip source (one-shot)

PWM_Ph_B.TZSEL.bit.DCAEVT1         = 0;            // Enable DCAEVT1 as the trip source (one-shot)

PWM_Ph_C.TZSEL.bit.DCAEVT1         = 0;            // Enable DCAEVT1 as the trip source (one-shot)

PWM_Ph_A.TZSEL.bit.DCBEVT1         = 1;            // Enable DCBEVT1 as the trip source (one-shot)

PWM_Ph_B.TZSEL.bit.DCBEVT1         = 1;            // Enable DCBEVT1 as the trip source (one-shot)

PWM_Ph_C.TZSEL.bit.DCBEVT1         = 1;            // Enable DCBEVT1 as the trip source (one-shot)

PWM_Ph_A.TZSEL.bit.CBC4            = 0;            // Enable TZ4 as the trip source (this is the eQEP error)

PWM_Ph_A.TZSEL.bit.CBC5            = 0;            // Enable TZ5 as the trip source (this is the clock failure)

PWM_Ph_B.TZSEL.bit.CBC4            = 0;            // Enable TZ4 as the trip source (this is the eQEP error)

PWM_Ph_B.TZSEL.bit.CBC5            = 0;            // Enable TZ5 as the trip source (this is the clock failure)

PWM_Ph_C.TZSEL.bit.CBC4            = 0;            // Enable TZ4 as the trip source (this is the eQEP error)

PWM_Ph_C.TZSEL.bit.CBC5            = 0;            // Enable TZ5 as the trip source (this is the clock failure)

PWM_Ph_A.TZSEL.bit.OSHT4           = 0;            // Enable TZ4 as the trip source (this is the eQEP error)

PWM_Ph_A.TZSEL.bit.OSHT5           = 1;            // Enable TZ5 as the trip source (this is the clock failure)

PWM_Ph_B.TZSEL.bit.OSHT4           = 0;            // Enable TZ4 as the trip source (this is the eQEP error)

PWM_Ph_B.TZSEL.bit.OSHT5           = 1;            // Enable TZ5 as the trip source (this is the clock failure)

PWM_Ph_C.TZSEL.bit.OSHT4           = 0;            // Enable TZ4 as the trip source (this is the eQEP error)

PWM_Ph_C.TZSEL.bit.OSHT5           = 1;            // Enable TZ5 as the trip source (this is the clock failure)

EDIS;

// Clear any spurious OV trip

ClearTripZoneRegisterFalgs();

}

#endif

/*******************************************\

| Periodic                                  |

\*******************************************/

//---------------------------------------------------

// Report the faults that caused the inverter to trip

//---------------------------------------------------

//---------------------------------------------------------

// Also check if there is a fatal fault. If so, stop the motor.

// These faults are fatal faults that do not trip the

// inverter or the gate drive.

//---------------------------------------------------------

void ReportTripFlags(TimerId timerId)

{

#if TRIP_ZONE_ACTIVE

//**********************************************************************

// Check if a tripping has occurred.

//**********************************************************************

CheckPWMTripFlag();

if(gPWM_TrippedByQEP) ReportFail(&PWMTripQEPErr, gPWM_TrippedByQEP);

else ReportPass(&PWMTripQEPErr, gPWM_TrippedByQEP);

if(gPWM_TrippedByClkFail) ReportFail(&PWMTripClockFault, gPWM_TrippedByClkFail);

else ReportPass(&PWMTripClockFault, gPWM_TrippedByClkFail);

if(gPWM_TrippedByOverCurrnt) ReportFail(&PWMTripOverCurrent, gPWM_TrippedByOverCurrnt);

else ReportPass(&PWMTripOverCurrent, gPWM_TrippedByOverCurrnt);

if(gPWM_TrippedByADC_PPB) ReportFail(&PWMTripADC_PPBLimitFault, gPWM_TrippedByADC_PPB);

else ReportPass(&PWMTripADC_PPBLimitFault, gPWM_TrippedByADC_PPB);

if(gPWM_TrippedByECC_Err) ReportFail(&PWMTripECCerr, gPWM_TrippedByECC_Err);

else ReportPass(&PWMTripECCerr, gPWM_TrippedByECC_Err);

#endif

}

/********************************************************************

* SetTripZoneOutputAction

*

* This function sets the action taken at the output when the

* trip zone trips

********************************************************************/

void SetTripZoneOutputAction(Uint16 Action)

{

EALLOW;

PWM_Ph_A.TZCTL2.bit.ETZE = 0; // TZCTL2 Enable. 0: Use trip action from TZCTL (legacy EPWM compatibility), 1: Use trip action defined in TZCTL2,

// TZCTLDCA and TZCTLDCB. Settings in TZCTL are ignored

PWM_Ph_A.TZCTL.bit.TZA = Action; //TZ_NO_CHANGE; // TZ_FORCE_LO; TZ_HIZ; // EPWMxA will go low - Dhammika TZ_FORCE_LO

PWM_Ph_A.TZCTL.bit.TZB = Action; //TZ_NO_CHANGE; // TZ_FORCE_LO; TZ_HIZ; // EPWMxB will go low - Dhammika TZ_NO_CHANGE

PWM_Ph_B.TZCTL2.bit.ETZE = 0; // TZCTL2 Enable. 0: Use trip action from TZCTL (legacy EPWM compatibility), 1: Use trip action defined in TZCTL2,

// TZCTLDCA and TZCTLDCB. Settings in TZCTL are ignored

PWM_Ph_B.TZCTL.bit.TZA = Action; //TZ_NO_CHANGE; // TZ_FORCE_LO; TZ_HIZ; // EPWMxA will go low

PWM_Ph_B.TZCTL.bit.TZB = Action; //TZ_NO_CHANGE; // TZ_FORCE_LO; TZ_HIZ; // EPWMxB will go low

PWM_Ph_B.TZCTL2.bit.ETZE = 0; // TZCTL2 Enable. 0: Use trip action from TZCTL (legacy EPWM compatibility), 1: Use trip action defined in TZCTL2,

//TZCTLDCA and TZCTLDCB. Settings in TZCTL are ignored

PWM_Ph_C.TZCTL.bit.TZA = Action; //TZ_NO_CHANGE; // TZ_FORCE_LO; TZ_HIZ; // EPWMxA will go low

PWM_Ph_C.TZCTL.bit.TZB = Action; //TZ_NO_CHANGE; // TZ_FORCE_LO; TZ_HIZ; // EPWMxB will go low

EDIS;

}

/********************************************************************

* ClearTripZone

*

* This function clears the trip zone registers of the PWM

* channels.

********************************************************************/

void ClearTripZone(void)

{

#if TRIP_ZONE_ACTIVE

// clear EPWM trip flags

DELAY_US(1L);

// Clear flags

ClearTripZoneRegisterFalgs();

EALLOW;

if((Ph_C_CMPSS.COMPSTS.bit.COMPHSTS == 0) && (Ph_C_CMPSS.COMPSTS.bit.COMPLSTS == 0) &&

(Ph_B_CMPSS.COMPSTS.bit.COMPHSTS == 0) && (Ph_B_CMPSS.COMPSTS.bit.COMPLSTS == 0) &&

(Ph_A_CMPSS.COMPSTS.bit.COMPHSTS == 0) && (Ph_A_CMPSS.COMPSTS.bit.COMPLSTS == 0) &&

(I_TOTAL_CMPSS.COMPSTS.bit.COMPHSTS == 0) && (I_TOTAL_CMPSS.COMPSTS.bit.COMPLSTS == 0))

{

// clear HLATCH - (not in TRIP gen path)

Ph_C_CMPSS.COMPSTSCLR.bit.HLATCHCLR = 1;

Ph_B_CMPSS.COMPSTSCLR.bit.HLATCHCLR = 1;

Ph_A_CMPSS.COMPSTSCLR.bit.HLATCHCLR = 1;

I_TOTAL_CMPSS.COMPSTSCLR.bit.HLATCHCLR = 1;

// clear LLATCH - (not in TRIP gen path)

Ph_C_CMPSS.COMPSTSCLR.bit.LLATCHCLR = 1;

Ph_B_CMPSS.COMPSTSCLR.bit.LLATCHCLR = 1;

Ph_A_CMPSS.COMPSTSCLR.bit.LLATCHCLR = 1;

I_TOTAL_CMPSS.COMPSTSCLR.bit.LLATCHCLR = 1;

// Clear the epwm-xbar input flags

XbarRegs.XBARCLR1.bit.Ph_C_XBAR_TRIP_IN_H = 1;

XbarRegs.XBARCLR1.bit.Ph_C_XBAR_TRIP_IN_L = 1;

XbarRegs.XBARCLR1.bit.Ph_B_XBAR_TRIP_IN_H = 1;

XbarRegs.XBARCLR1.bit.Ph_B_XBAR_TRIP_IN_L = 1;

XbarRegs.XBARCLR1.bit.Ph_A_XBAR_TRIP_IN_H = 1;

XbarRegs.XBARCLR1.bit.Ph_A_XBAR_TRIP_IN_L = 1;

XbarRegs.XBARCLR1.bit.TOTAL_I_XBAR_TRIP_IN_H = 1;

XbarRegs.XBARCLR1.bit.TOTAL_I_XBAR_TRIP_IN_L = 1;

XbarRegs.XBARCLR1.bit.Ph_C_XBAR_TRIP_OUT_H = 1;

XbarRegs.XBARCLR1.bit.Ph_C_XBAR_TRIP_OUT_L = 1;

XbarRegs.XBARCLR1.bit.Ph_B_XBAR_TRIP_OUT_H = 1;

XbarRegs.XBARCLR1.bit.Ph_B_XBAR_TRIP_OUT_L = 1;

XbarRegs.XBARCLR1.bit.Ph_A_XBAR_TRIP_OUT_H = 1;

XbarRegs.XBARCLR1.bit.Ph_A_XBAR_TRIP_OUT_L = 1;

XbarRegs.XBARCLR1.bit.TOTAL_I_XBAR_TRIP_OUT_H = 1;

XbarRegs.XBARCLR1.bit.TOTAL_I_XBAR_TRIP_OUT_L = 1;

ClearPWM_Trip = 0;

}

// Clear the output xbar flags.

// OutputXbarRegs.OUTPUTLATCHCLR.bit.OUTPUT1 = 1;

EDIS;

#endif

}

/************************************************************

* ClearTripZoneRegisterFalgs clears the trip zone flags

* in TZCLR, TZOSTCLR and TZCBCCLR.

************************************************************/

void ClearTripZoneRegisterFalgs(void)

{

EALLOW;

// clear OST flags

PWM_Ph_A.TZCLR.bit.OST = 1;

PWM_Ph_B.TZCLR.bit.OST = 1;

PWM_Ph_C.TZCLR.bit.OST = 1;

// clear CNC flags

PWM_Ph_A.TZCLR.bit.CBC = 1;

PWM_Ph_B.TZCLR.bit.CBC = 1;

PWM_Ph_C.TZCLR.bit.CBC = 1;

// clear DCAEVT1 flags

PWM_Ph_A.TZCLR.bit.DCAEVT1 = 1;

PWM_Ph_B.TZCLR.bit.DCAEVT1 = 1;

PWM_Ph_C.TZCLR.bit.DCAEVT1 = 1;

PWM_Ph_A.TZCLR.bit.DCBEVT1 = 1;

PWM_Ph_B.TZCLR.bit.DCBEVT1 = 1;

PWM_Ph_C.TZCLR.bit.DCBEVT1 = 1;

// clear DCAEVT2 flags

PWM_Ph_A.TZCLR.bit.DCAEVT2 = 1;

PWM_Ph_B.TZCLR.bit.DCAEVT2 = 1;

PWM_Ph_C.TZCLR.bit.DCAEVT2 = 1;

PWM_Ph_A.TZCLR.bit.DCBEVT2 = 1;

PWM_Ph_B.TZCLR.bit.DCBEVT2 = 1;

PWM_Ph_C.TZCLR.bit.DCBEVT2 = 1;

// Clear TZOSTCLR

PWM_Ph_A.TZOSTCLR.all = 0xFFFF;

PWM_Ph_B.TZOSTCLR.all = 0xFFFF;

PWM_Ph_C.TZOSTCLR.all = 0xFFFF;

// Clear TZCBCCLR

PWM_Ph_A.TZCBCCLR.all = 0xFFFF;

PWM_Ph_B.TZCBCCLR.all = 0xFFFF;

PWM_Ph_C.TZCBCCLR.all = 0xFFFF;

EDIS;

}

#if TRIP_ZONE_ACTIVE

//**********************************************************************

// Check if a tripping has occurred. If so, disable the whole inverter.

// Also extract the error flags.

// This runs in motor control ISR thread

//**********************************************************************

void CheckPWMTripFlag(void)

{

// Check for PWM trip due to over current

if ((PWM_Ph_A.TZFLG.bit.OST ||

PWM_Ph_B.TZFLG.bit.OST ||

PWM_Ph_C.TZFLG.bit.OST) )

{

// if any EPwm's OST is set, force OST on all three to DISABLE inverter

EALLOW;

PWM_Ph_A.TZFRC.bit.OST = 1;

PWM_Ph_B.TZFRC.bit.OST = 1;

PWM_Ph_C.TZFRC.bit.OST = 1;

EDIS;

gPWM_Tripped = true;      // Trip on DMC (halt and IPM fault trip )

// Extract the error flags

EALLOW;

// Overcurrent

gPWM_TrippedByOverCurrnt = (PWM_Ph_A.TZOSTFLG.bit.DCAEVT1 << 2) | (PWM_Ph_B.TZOSTFLG.bit.DCAEVT1 << 1) | (PWM_Ph_C.TZOSTFLG.bit.DCAEVT1);

// ADC post processing block's limit reached.

gPWM_TrippedByADC_PPB = (Ph_A_ADC.ADCEVTSTAT.bit.PPB2TRIPHI << 1) | Ph_A_ADC.ADCEVTSTAT.bit.PPB2TRIPLO;

gPWM_TrippedByADC_PPB = (gPWM_TrippedByADC_PPB << 2) | (Ph_B_ADC.ADCEVTSTAT.bit.PPB2TRIPHI << 1) | Ph_B_ADC.ADCEVTSTAT.bit.PPB2TRIPLO;

gPWM_TrippedByADC_PPB = (gPWM_TrippedByADC_PPB << 2) | (Ph_C_ADC.ADCEVTSTAT.bit.PPB2TRIPHI << 1) | Ph_C_ADC.ADCEVTSTAT.bit.PPB2TRIPLO;

// QEP Error

gPWM_TrippedByQEP = (PWM_Ph_A.TZOSTFLG.bit.OST4 << 2) | (PWM_Ph_B.TZOSTFLG.bit.OST4 << 1) | (PWM_Ph_C.TZOSTFLG.bit.OST4);

EDIS;

}

else

{

gPWM_Tripped = false;

}

// Clock failure

gPWM_TrippedByClkFail = NmiIntruptRegs.NMIFLG.bit.CLOCKFAIL;

// ECC Error

gPWM_TrippedByECC_Err = (NmiIntruptRegs.NMIFLG.bit.FLUNCERR << 1) | NmiIntruptRegs.NMIFLG.bit.RAMUNCERR;

if(gPWM_TrippedByClkFail || gPWM_TrippedByECC_Err || gPWM_Tripped) gPWM_Tripped = true;

else gPWM_Tripped = false;

}

#endif

# Initialization

The main program has the following initializations.

void main(void)

{

// Perform power on test

POR_Faulted =  PerformPOST();

// Initialize System Control:

// PLL, WatchDog, enable Peripheral Clocks

// This function is found in the F28M3Xx_SysCtrl.c file.

InitSysCtrl();

// Init low frequency peripheral clock

InitBasicConst();

// Initialize logging. This should be removed in the final version

InitDataLog();

// Init the PIE control registers and the vector table

SetUpPeripheralInts();

// Configure the GPIOs

ConfigGPIO();

// Set up the timer2

TimersInit();

// Set up the PWM module. PWM 1, 2 and 3 are used for the three phase

// inverter. This routine sets up the PWM period, syncing PWM 2 and 3 to PWM 1

// count direction after new sync etc. It also init the GPIO.

SetupPWM();

initDeadbandComp();

//Configure the ADC and power it up. Calibration of the linearity of the ADC

//is done within this routine.

ConfigureADC();

//Select the channels to convert and start of conversion (SOC) flag

SetupADC_Channels();

// Init the MOSFET gate driver SPI.

// This should be called after initializing system ctrl.

InitGateDrive();

//Link the start of conversions to PWM1 period counter becoming 0.

//Selecting of the right current has to be done at a different place.

LinkPWM_toADC();

// ****************************************************************************

// ****************************************************************************

// Parameter Initialization

// ****************************************************************************

// ****************************************************************************

// Init the global variables

InitGlobals();

//Init the speed and position loop parameters.

InitSpeedPosLoopPara();

// Init the current loop para

InitCurrentLoopPara();

// ****************************************************************************

// ****************************************************************************

// Call HVDMC Protection function

// ****************************************************************************

// ****************************************************************************

#if TRIP_ZONE_ACTIVE

InitOverCurrentTrip_Pt1();

#endif

#if PROTECTION_ACTIVE

if(gEnable_Watchdog)

{

// Enable watchdog and service it.

SysCtl_setWatchdogMode(SYSCTL_WD_MODE_RESET);

SysCtl_enableWatchdog();

SysCtl_serviceWatchdog();

}

#else

SysCtl_setWatchdogMode(SYSCTL_WD_MODE_RESET);

SysCtl_disableWatchdog();

#endif

// Clear the NMI flags and set the NMI for memory errors

MapAndEnableNMI_ISR();

// Map MotorControlISR to PWM1 interrupt.

// Then enable the interrupts

MapAndEnablePWM_ISR();

//Start our Ticker

TimersStart();

SCI_ThreadInit();  // Init SCI-A

EINT;          // Enable Global interrupt INTM

ERTM;          // Disable debug events

// Execute the task manager

// It has the infinite loop

gTaskMgr();

} //END MAIN CODE

# Second part of trip-zone initialization

InitOverCurrentTrip_Pt2() is executed after ADC offsets have been calibrated inside the motor control ISR. Motor control ISR has been modeled based on “\ti\controlSUITE\development_kits\HVMotorCtrl+PfcKit_v2.1”. The code snippet for doing that is:

#if TRIP_ZONE_ACTIVE

// Now that ADC PPB has been set up, enable trip zone

enableADC_PPB_Tripzone();

// Clear trip-zone flags

ClearTripZone();

// Execute that part 2 of trip-zone init.

InitOverCurrentTrip_Pt2();

#endif

Note that the watchdog is currently disabled.

# POST of flash ECC

POST of flash ECC follows the STL example provided by TI.

# Definitions

#define CurrentFeedback_A AdcaResultRegs.ADCRESULT0  /* A14 */

#define CurrentFeedback_B AdcaResultRegs.ADCRESULT1  /* A2 */

#define CurrentFeedback_C AdcbResultRegs.ADCRESULT0  /* B2 */

#define BUS_VOLT          AdcaResultRegs.ADCRESULT11 /* Bus voltage (unsigned) A1*/

#define BUS_AMPS          AdcaResultRegs.ADCRESULT10 /* I total - A4 */

#define PH_A_VOLT         AdcaResultRegs.ADCRESULT4  /* ph voltage (unsigned) - A3 */

#define PH_B_VOLT         AdcaResultRegs.ADCRESULT5  /* ph voltage (unsigned) - A15 */

#define PH_C_VOLT         AdcbResultRegs.ADCRESULT5  /* ph voltage (unsigned) - B0 */

#define MOT_TEMPERATURE   AdcbResultRegs.ADCRESULT2  /* Motor temperature (unsigned) - B3 */

#define _1p2VSENSE        AdcbResultRegs.ADCRESULT3  /* 1.2V sense voltage (unsigned) - B4 */

#define _3p3VSENSE        AdcbResultRegs.ADCRESULT4  /* 3.3V sense voltage (unsigned) - B5 */

#define PROC_TEMPERATURE  AdcaResultRegs.ADCRESULT6

// ADC result after post processing (i.e. after subtracting the offset. Usually the offset is 2048)

#define CurrentFeedback_A_PPB ((int16)AdcaResultRegs.ADCPPB1RESULT.all)  /* Ph_A */

#define CurrentFeedback_B_PPB ((int16)AdcaResultRegs.ADCPPB2RESULT.all)  /* Ph_B */

#define CurrentFeedback_C_PPB ((int16)AdcbResultRegs.ADCPPB1RESULT.all)  /* Ph_C */

// The following is valid only for BOOSTXL and Terumo

#define Ph_A_ADC      (AdcaRegs)  /* A14 */

#define Ph_B_ADC      (AdcaRegs)  /* A2 */

#define Ph_C_ADC      (AdcbRegs)  /* B2 */

#define VMot_ADC      (AdcaRegs)  /* A1 */

#define Itotal_ADC    (AdcaRegs)  /* A4 */

#define VA_ADC        (AdcaRegs)  /* A3 */

#define VB_ADC        (AdcaRegs)  /* A15 */

#define VC_ADC        (AdcbRegs)  /* B0 */

#define C_DEG_ADC     (AdcbRegs)  /* B3 */

#define V_1p2_ADC     (AdcbRegs)  /* B4 */

#define V_3p3_ADC     (AdcbRegs)  /* B5 */

#define PROC_TEMP_ADC (AdcaRegs)  /* ADCIN13 */

#define Ph_A_POST_PROCESSING_BLOCK_REF_VAL ADCPPB1OFFREF

#define Ph_B_POST_PROCESSING_BLOCK_REF_VAL ADCPPB2OFFREF

#define Ph_C_POST_PROCESSING_BLOCK_REF_VAL ADCPPB1OFFREF

// The following is valid only for Terumo

#define Ph_A_ADC_Ch       14  // ADCIN14

#define Ph_B_ADC_Ch       2   // ADCINA2

#define Ph_C_ADC_Ch       2   // ADCINB2

#define VMot_ADC_Ch       1   // ADCINA1

#define Itotal_ADC_Ch     4   // ADCINA4

#define VA_ADC_Ch         3   // ADCINA3

#define VB_ADC_Ch         15  // ADCINA15

#define VC_ADC_Ch         0   // ADCINB0

#define C_DEG_ADC_Ch      3   // ADCINB3

#define V_1p2_ADC_Ch      4   // ADCINB4

#define V_3p3_ADC_Ch      5   // ADCINB5

#define PROC_TEMP_ADC_Ch  13  // ADCIN13

// The following is valid only for BOOSTXL

#define Ph_A_CMPSS    (Cmpss4Regs)

#define Ph_B_CMPSS    (Cmpss1Regs)

#define Ph_C_CMPSS    (Cmpss3Regs)

#define I_TOTAL_CMPSS (Cmpss2Regs)

#define Ph_A_XBAR_TRIP_IN_H    CMPSS4_CTRIPH

#define Ph_A_XBAR_TRIP_OUT_H   CMPSS4_CTRIPOUTH

#define Ph_A_XBAR_TRIP_IN_L    CMPSS4_CTRIPL

#define Ph_A_XBAR_TRIP_OUT_L   CMPSS4_CTRIPOUTL

#define Ph_B_XBAR_TRIP_IN_H    CMPSS1_CTRIPH

#define Ph_B_XBAR_TRIP_OUT_H   CMPSS1_CTRIPOUTH

#define Ph_B_XBAR_TRIP_IN_L    CMPSS1_CTRIPL

#define Ph_B_XBAR_TRIP_OUT_L   CMPSS1_CTRIPOUTL

#define Ph_C_XBAR_TRIP_IN_H    CMPSS3_CTRIPH

#define Ph_C_XBAR_TRIP_OUT_H   CMPSS3_CTRIPOUTH

#define Ph_C_XBAR_TRIP_IN_L    CMPSS3_CTRIPL

#define Ph_C_XBAR_TRIP_OUT_L   CMPSS3_CTRIPOUTL

#define TOTAL_I_XBAR_TRIP_IN_H    CMPSS2_CTRIPH

#define TOTAL_I_XBAR_TRIP_OUT_H   CMPSS2_CTRIPOUTH

#define TOTAL_I_XBAR_TRIP_IN_L    CMPSS2_CTRIPL

#define TOTAL_I_XBAR_TRIP_OUT_L   CMPSS2_CTRIPOUTL