#ifndef _USER_H_

#define _USER_H_

#include "sw/modules/types/src/types.h"

#include "sw/modules/motor/src/32b/motor.h"

#include "sw/modules/est/src/32b/est.h"

#include "sw/modules/est/src/est_states.h"

#include "sw/modules/est/src/est_Flux_states.h"

#include "sw/modules/est/src/est_Ls_states.h"

#include "sw/modules/est/src/est_Rs_states.h"

#include "sw/modules/ctrl/src/32b/ctrl_obj.h"

#include "sw/modules/fast/src/32b/userParams.h"

#ifdef __cplusplus

extern "C" {

#endif

#define USER_IQ_FULL_SCALE_FREQ_Hz        (800.0)

#define USER_IQ_FULL_SCALE_VOLTAGE_V      (800.0

#define USER_ADC_FULL_SCALE_VOLTAGE_V       (765.0)

#define USER_VOLTAGE_SF               ((float_t)((USER_ADC_FULL_SCALE_VOLTAGE_V)/(USER_IQ_FULL_SCALE_VOLTAGE_V)))

#define USER_IQ_FULL_SCALE_CURRENT_A          (62.5)

#define USER_ADC_FULL_SCALE_CURRENT_A        (125)

#define USER_CURRENT_SF               ((float_t)((USER_ADC_FULL_SCALE_CURRENT_A)/(USER_IQ_FULL_SCALE_CURRENT_A)))

#define USER_NUM_CURRENT_SENSORS            (3)

#define USER_NUM_VOLTAGE_SENSORS            (3)

#define   I_A_offset    (1.006788850)

#define   I_B_offset    (1.009521067)

#define   I_C_offset    (1.013614953)

#define   V_A_offset    (0.3385268450)

#define   V_B_offset    (0.3384960294)

#define   V_C_offset    (0.3403902650)

#define USER_SYSTEM_FREQ_MHz             (90.0)

#define USER_PWM_FREQ_kHz                (8.0)

#define USER_MAX_VS_MAG_PU        (2.0/3.0)

#define USER_CTRL_HANDLE_ADDRESS   (0x13C40)

#define USER_EST_HANDLE_ADDRESS    (0x13840)

#define USER_VD_SF                 (0.95)

#define USER_PWM_PERIOD_usec       (1000.0/USER_PWM_FREQ_kHz)

#define USER_ISR_FREQ_Hz           ((float_t)USER_PWM_FREQ_kHz * 1000.0 / (float_t)USER_NUM_PWM_TICKS_PER_ISR_TICK)

#define USER_ISR_PERIOD_usec       (USER_PWM_PERIOD_usec * (float_t)USER_NUM_PWM_TICKS_PER_ISR_TICK)

#define USER_NUM_PWM_TICKS_PER_ISR_TICK        (1)

#define USER_NUM_ISR_TICKS_PER_CTRL_TICK       (1)

#define USER_NUM_CTRL_TICKS_PER_CURRENT_TICK   (1)

#define USER_NUM_CTRL_TICKS_PER_EST_TICK       (1)

#define USER_NUM_CTRL_TICKS_PER_SPEED_TICK  (8)

#define USER_NUM_CTRL_TICKS_PER_TRAJ_TICK   (15)

#define USER_CTRL_FREQ_Hz          (uint_least32_t)(USER_ISR_FREQ_Hz/USER_NUM_ISR_TICKS_PER_CTRL_TICK)

#define USER_EST_FREQ_Hz           (uint_least32_t)(USER_CTRL_FREQ_Hz/USER_NUM_CTRL_TICKS_PER_EST_TICK)

#define USER_TRAJ_FREQ_Hz          (uint_least32_t)(USER_CTRL_FREQ_Hz/USER_NUM_CTRL_TICKS_PER_TRAJ_TICK)

#define USER_CTRL_PERIOD_usec      (USER_ISR_PERIOD_usec * USER_NUM_ISR_TICKS_PER_CTRL_TICK)

#define USER_CTRL_PERIOD_sec       ((float_t)USER_CTRL_PERIOD_usec/(float_t)1000000.0)

#define USER_MAX_NEGATIVE_ID_REF_CURRENT_A     (-0.5 * USER_MOTOR_MAX_CURRENT)

#define USER_ZEROSPEEDLIMIT   (0.5 / USER_IQ_FULL_SCALE_FREQ_Hz)

#define USER_FORCE_ANGLE_FREQ_Hz   (2.0 * USER_ZEROSPEEDLIMIT * USER_IQ_FULL_SCALE_FREQ_Hz)

#define USER_MAX_CURRENT_SLOPE_POWERWARP   (0.3*USER_MOTOR_RES_EST_CURRENT/USER_IQ_FULL_SCALE_CURRENT_A/USER_TRAJ_FREQ_Hz

#define USER_MAX_ACCEL_Hzps                 (20.0)

#define USER_MAX_ACCEL_EST_Hzps           (2.0)

#define USER_MAX_CURRENT_SLOPE           (USER_MOTOR_RES_EST_CURRENT/USER_IQ_FULL_SCALE_CURRENT_A/USER_TRAJ_FREQ_Hz)

#define USER_IDRATED_FRACTION_FOR_RATED_FLUX (1.0)

#define USER_IDRATED_FRACTION_FOR_L_IDENT    (1.0)

#define USER_IDRATED_DELTA                  (0.00002)

#define USER_SPEEDMAX_FRACTION_FOR_L_IDENT  (1.0)

#define USER_FLUX_FRACTION           (1.0)

#define USER_POWERWARP_GAIN                   (1.0)

#define USER_R_OVER_L_EST_FREQ_Hz (200)

#define USER_VOLTAGE_FILTER_POLE_Hz  (615.0)

#define USER_VOLTAGE_FILTER_POLE_rps  (2.0 * MATH_PI * USER_VOLTAGE_FILTER_POLE_Hz)

#define USER_OFFSET_POLE_rps            (20.0)

#define USER_FLUX_POLE_rps              (100.0)

#define USER_DIRECTION_POLE_rps             (6.0)

#define USER_SPEED_POLE_rps           (100.0)

#define USER_DCBUS_POLE_rps           (100.0)

#define   USER_EST_KAPPAQ               (1.5)

#define Estun_EMJ_04APB22           101

#define Anaheim_BLY172S             102

#define My_Motor                    104

#define GE_pump                     105

#define fridge_compressor           106

#define Belt_Drive_Washer_IPM       201

#define Marathon_5K33GN2A           301

#define Marathon_56H17T2011A        302

#define Selni_AHV242N06             303

#define Dayton_3N352C_230           304

#define Dayton_3N352C_460           305

#define Weg_00118ET3E143T_W22_230   306

#define Weg_00118ET3E143T_W22_460   307

#define Oriental_4IK25A_SH_230      308

#define Dayton_2N865T               309

#define SEO_motor					400

#define Fan_motor					401

//#define USER_MOTOR Estun_EMJ_04APB22

//#define USER_MOTOR Anaheim_BLY172S

//#define USER_MOTOR My_Motor

//#define USER_MOTOR GE_pump

//#define USER_MOTOR fridge_compressor

//#define USER_MOTOR Belt_Drive_Washer_IPM

//#define USER_MOTOR Marathon_5K33GN2A

//#define USER_MOTOR Marathon_56H17T2011A

//#define USER_MOTOR Selni_AHV242N06

//#define USER_MOTOR Dayton_3N352C_230

//#define USER_MOTOR Dayton_3N352C_460

//#define USER_MOTOR Weg_00118ET3E143T_W22_230

//#define USER_MOTOR Weg_00118ET3E143T_W22_460

//#define USER_MOTOR Oriental_4IK25A_SH_230

//#define USER_MOTOR Dayton_2N865T

//#define USER_MOTOR SEO_motor

#define USER_MOTOR Fan_motor

#if (USER_MOTOR == Estun_EMJ_04APB22)                  // Name must match the motor #define

#define USER_MOTOR_TYPE                 MOTOR_Type_Pm  // Motor_Type_Pm (All Synchronous: BLDC, PMSM, SMPM, IPM) or Motor_Type_Induction (Asynchronous ACI)

#define USER_MOTOR_NUM_POLE_PAIRS       (4)            // PAIRS, not total poles. Used to calculate user RPM from rotor Hz only

#define USER_MOTOR_Rr                   (NULL)         // Induction motors only, else NULL

//#define USER_MOTOR_Rs                   (2.200221)     // Identified phase to neutral resistance in a Y equivalent circuit (Ohms, float)

//#define USER_MOTOR_Ls_d                 (0.008721023)  // For PM, Identified average stator inductance  (Henry, float)

//#define USER_MOTOR_Ls_q                 (0.008721023)  // For PM, Identified average stator inductance  (Henry, float)

//#define USER_MOTOR_RATED_FLUX           (0.3846985)    // Identified TOTAL flux linkage between the rotor and the stator (V/Hz)

#define USER_MOTOR_Rs                   (2.425783)     // Identified phase to neutral resistance in a Y equivalent circuit (Ohms, float)

#define USER_MOTOR_Ls_d                 (0.008207544)  // For PM, Identified average stator inductance  (Henry, float)

#define USER_MOTOR_Ls_q                 (0.008207544)  // For PM, Identified average stator inductance  (Henry, float)

#define USER_MOTOR_RATED_FLUX           (0.3859799)    // Identified TOTAL flux linkage between the rotor and the stator (V/Hz)

#define USER_MOTOR_MAGNETIZING_CURRENT  (NULL)         // Induction motors only, else NULL

#define USER_MOTOR_RES_EST_CURRENT      (1.0)          // During Motor ID, maximum current (Amperes, float) used for Rs estimation, 10-20% rated current

#define USER_MOTOR_IND_EST_CURRENT      (-1.0)         // During Motor ID, maximum current (negative Amperes, float) used for Ls estimation, use just enough to enable rotation

#define USER_MOTOR_MAX_CURRENT          (3.82)         // CRITICAL: Used during ID and run-time, sets a limit on the maximum current command output of the provided Speed PI Controller to the Iq controller

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (20.0)         // During Motor ID, maximum commanded speed (Hz, float), ~10% rated

#elif (USER_MOTOR == Anaheim_BLY172S)

#define USER_MOTOR_TYPE                 MOTOR_Type_Pm

#define USER_MOTOR_NUM_POLE_PAIRS       (4)

#define USER_MOTOR_Rr                   (NULL)

#define USER_MOTOR_Rs                   (0.4110007)

#define USER_MOTOR_Ls_d                 (0.0007092811)

#define USER_MOTOR_Ls_q                 (0.0007092811)

#define USER_MOTOR_RATED_FLUX           (0.03279636)

#define USER_MOTOR_MAGNETIZING_CURRENT  (NULL)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (-1.0)

#define USER_MOTOR_MAX_CURRENT          (5.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (20.0)

#elif (USER_MOTOR == My_Motor)

#define USER_MOTOR_TYPE                 MOTOR_Type_Pm

#define USER_MOTOR_NUM_POLE_PAIRS       (2)

#define USER_MOTOR_Rr                   (NULL)

#define USER_MOTOR_Rs                   (0.3918252)

#define USER_MOTOR_Ls_d                 (0.00023495)

#define USER_MOTOR_Ls_q                 (0.00023495)

#define USER_MOTOR_RATED_FLUX           (0.03955824)

#define USER_MOTOR_MAGNETIZING_CURRENT  (NULL)

#define USER_MOTOR_RES_EST_CURRENT      (3.0)

#define USER_MOTOR_IND_EST_CURRENT      (-0.5)

#define USER_MOTOR_MAX_CURRENT          (20.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (20.0)

#elif (USER_MOTOR == GE_pump)

#define USER_MOTOR_TYPE                 MOTOR_Type_Pm

#define USER_MOTOR_NUM_POLE_PAIRS       (4)

#define USER_MOTOR_Rr                   (NULL)

#define USER_MOTOR_Rs                   (0.1931403)

#define USER_MOTOR_Ls_d                 (0.0001903657)

#define USER_MOTOR_Ls_q                 (0.0001903657)

#define USER_MOTOR_RATED_FLUX           (0.06034314)

#define USER_MOTOR_MAGNETIZING_CURRENT  (NULL)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (-1.0)

#define USER_MOTOR_MAX_CURRENT          (8.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (20.0)

#elif (USER_MOTOR == fridge_compressor)

#define USER_MOTOR_TYPE                 MOTOR_Type_Pm

#define USER_MOTOR_NUM_POLE_PAIRS       (2)

#define USER_MOTOR_Rr                   (NULL)

#define USER_MOTOR_Rs                   (4.707864)

#define USER_MOTOR_Ls_d                 (0.1728041)

#define USER_MOTOR_Ls_q                 (0.1728041)

#define USER_MOTOR_RATED_FLUX           (0.418879)

#define USER_MOTOR_MAGNETIZING_CURRENT  (NULL)

#define USER_MOTOR_RES_EST_CURRENT      (0.3)

#define USER_MOTOR_IND_EST_CURRENT      (-0.3)

#define USER_MOTOR_MAX_CURRENT          (5.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (40.0)

#elif (USER_MOTOR == Belt_Drive_Washer_IPM)

#define USER_MOTOR_TYPE                 MOTOR_Type_Pm

#define USER_MOTOR_NUM_POLE_PAIRS       (4)

#define USER_MOTOR_Rr                   (NULL)

#define USER_MOTOR_Rs                   (2.70544)

#define USER_MOTOR_Ls_d                 (0.0115)

#define USER_MOTOR_Ls_q                 (0.0135)

#define USER_MOTOR_RATED_FLUX           (0.5022156)

#define USER_MOTOR_MAGNETIZING_CURRENT  (NULL)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (-1.0)

#define USER_MOTOR_MAX_CURRENT          (4.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (20.0)

#elif (USER_MOTOR == Marathon_5K33GN2A)                      // Name must match the motor #define

#define USER_MOTOR_TYPE                 MOTOR_Type_Induction // Motor_Type_Pm (All Synchronous: BLDC, PMSM, SMPM, IPM) or Motor_Type_Induction (Asynchronous ACI)

#define USER_MOTOR_NUM_POLE_PAIRS       (2)                  // PAIRS, not total poles. Used to calculate user RPM from rotor Hz only

#define USER_MOTOR_Rr                   (5.574939)           // Identified phase to neutral in a Y equivalent circuit (Ohms, float)

#define USER_MOTOR_Rs                   (10.71121)           // Identified phase to neutral in a Y equivalent circuit (Ohms, float)

#define USER_MOTOR_Ls_d                 (0.04282236)         // For Induction, Identified average stator inductance  (Henry, float)

#define USER_MOTOR_Ls_q                 USER_MOTOR_Ls_d      // For Induction, Identified average stator inductance  (Henry, float)

#define USER_MOTOR_RATED_FLUX           (0.8165*220.0/60.0)  // sqrt(2/3)* Rated V (line-line) / Rated Freq (Hz)

#define USER_MOTOR_MAGNETIZING_CURRENT  (1.514201)           // Identified magnetizing current for induction motors, else NULL

#define USER_MOTOR_RES_EST_CURRENT      (1.0)                // During Motor ID, maximum current (Amperes, float) used for Rs estimation, 10-20% rated current

#define USER_MOTOR_IND_EST_CURRENT      (NULL)               // not used for induction

#define USER_MOTOR_MAX_CURRENT          (2.0)                // CRITICAL: Used during ID and run-time, sets a limit on the maximum current command output of the provided Speed PI Controller to the Iq controller

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (5.0)                // During Motor ID, maximum commanded speed (Hz, float). Should always use 5 Hz for Induction.

#elif (USER_MOTOR == Marathon_56H17T2011A)

#define USER_MOTOR_TYPE                 MOTOR_Type_Induction

#define USER_MOTOR_NUM_POLE_PAIRS       (2)

#define USER_MOTOR_Rr                   (5.076565)

#define USER_MOTOR_Rs                   (7.777135)

#define USER_MOTOR_Ls_d                 (0.02756402)

#define USER_MOTOR_Ls_q                 USER_MOTOR_Ls_d

#define USER_MOTOR_RATED_FLUX           (0.8165*230.0/60.0)

#define USER_MOTOR_MAGNETIZING_CURRENT  (1.130457)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (NULL)

#define USER_MOTOR_MAX_CURRENT          (5.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (5.0)

#elif (USER_MOTOR == Selni_AHV242N06)

#define USER_MOTOR_TYPE                 MOTOR_Type_Induction

#define USER_MOTOR_NUM_POLE_PAIRS       (1)

#define USER_MOTOR_Rr                   (0.6519378)

#define USER_MOTOR_Rs                   (1.863846)

#define USER_MOTOR_Ls_d                 (0.01710425)

#define USER_MOTOR_Ls_q                 USER_MOTOR_Ls_d

#define USER_MOTOR_RATED_FLUX           (0.8165*190.0/200.0)

#define USER_MOTOR_MAGNETIZING_CURRENT  (1.246659)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (NULL)

#define USER_MOTOR_MAX_CURRENT          (5.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (5.0)

#elif (USER_MOTOR == Dayton_3N352C_230)

#define USER_MOTOR_TYPE                 MOTOR_Type_Induction

#define USER_MOTOR_NUM_POLE_PAIRS       (2)

#define USER_MOTOR_Rr                   (2.064656)

#define USER_MOTOR_Rs                   (2.858907)

#define USER_MOTOR_Ls_d                 (0.02556416)

#define USER_MOTOR_Ls_q                 USER_MOTOR_Ls_d

#define USER_MOTOR_RATED_FLUX           (0.8165*230.0/60.0)

#define USER_MOTOR_MAGNETIZING_CURRENT  (2.938731)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (NULL)

#define USER_MOTOR_MAX_CURRENT          (5.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (5.0)

#elif (USER_MOTOR == Dayton_3N352C_460)

#define USER_MOTOR_TYPE                 MOTOR_Type_Induction

#define USER_MOTOR_NUM_POLE_PAIRS       (2)

#define USER_MOTOR_Rr                   (9.077622)

#define USER_MOTOR_Rs                   (11.106)

#define USER_MOTOR_Ls_d                 (0.0713708)

#define USER_MOTOR_Ls_q                 USER_MOTOR_Ls_d

#define USER_MOTOR_RATED_FLUX           (0.8165*460.0/60.0)

#define USER_MOTOR_MAGNETIZING_CURRENT  (1.462848)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (NULL)

#define USER_MOTOR_MAX_CURRENT          (4.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (5.0)

#elif (USER_MOTOR == Weg_00118ET3E143T_W22_230)

#define USER_MOTOR_TYPE                 MOTOR_Type_Induction

#define USER_MOTOR_NUM_POLE_PAIRS       (2)

#define USER_MOTOR_Rr                   (1.067204)

#define USER_MOTOR_Rs                   (2.406377)

#define USER_MOTOR_Ls_d                 (0.05482391)

#define USER_MOTOR_Ls_q                 USER_MOTOR_Ls_d

#define USER_MOTOR_RATED_FLUX           (0.8165*230.0/60.0)

#define USER_MOTOR_MAGNETIZING_CURRENT  (2.17855)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (NULL)

#define USER_MOTOR_MAX_CURRENT          (6.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (5.0)

#elif (USER_MOTOR == Weg_00118ET3E143T_W22_460)

#define USER_MOTOR_TYPE                 MOTOR_Type_Induction

#define USER_MOTOR_NUM_POLE_PAIRS       (2)

#define USER_MOTOR_Rr                   (2.934001)

#define USER_MOTOR_Rs                   (9.537757)

#define USER_MOTOR_Ls_d                 (0.2160697)

#define USER_MOTOR_Ls_q                 USER_MOTOR_Ls_d

#define USER_MOTOR_RATED_FLUX           (0.8165*460.0/60.0)

#define USER_MOTOR_MAGNETIZING_CURRENT  (1.086798)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (NULL)

#define USER_MOTOR_MAX_CURRENT          (5.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (5.0)

#elif (USER_MOTOR == Oriental_4IK25A_SH_230)

#define USER_MOTOR_TYPE                 MOTOR_Type_Induction

#define USER_MOTOR_NUM_POLE_PAIRS       (2)

#define USER_MOTOR_Rr                   (64.00807)

#define USER_MOTOR_Rs                   (128.2861)

#define USER_MOTOR_Ls_d                 (0.6856454)

#define USER_MOTOR_Ls_q                 USER_MOTOR_Ls_d

#define USER_MOTOR_RATED_FLUX           (0.8165*230.0/60.0)

#define USER_MOTOR_MAGNETIZING_CURRENT  (0.166837)

#define USER_MOTOR_RES_EST_CURRENT      (0.1)

#define USER_MOTOR_IND_EST_CURRENT      (NULL)

#define USER_MOTOR_MAX_CURRENT          (0.3)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (5.0)

#elif (USER_MOTOR == Dayton_2N865T)

#define USER_MOTOR_TYPE                 MOTOR_Type_Induction

#define USER_MOTOR_NUM_POLE_PAIRS       (2)

#define USER_MOTOR_Rr                   (2.540533)

#define USER_MOTOR_Rs                   (4.681129)

#define USER_MOTOR_Ls_d                 (0.02621018)

#define USER_MOTOR_Ls_q                 USER_MOTOR_Ls_d

#define USER_MOTOR_RATED_FLUX           (0.8165*230.0/60.0)

#define USER_MOTOR_MAGNETIZING_CURRENT  (1.838921)

#define USER_MOTOR_RES_EST_CURRENT      (1.0)

#define USER_MOTOR_IND_EST_CURRENT      (NULL)

#define USER_MOTOR_MAX_CURRENT          (5.0)

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (5.0)

#elif (USER_MOTOR == SEO_motor)                  // Name must match the motor #define

#define USER_MOTOR_TYPE                 MOTOR_Type_Pm  // Motor_Type_Pm (All Synchronous: BLDC, PMSM, SMPM, IPM) or Motor_Type_Induction (Asynchronous ACI)

#define USER_MOTOR_NUM_POLE_PAIRS       (3)            // PAIRS, not total poles. Used to calculate user RPM from rotor Hz only

#define USER_MOTOR_Rr                   (NULL)         // Induction motors only, else NULL

#define USER_MOTOR_Rs                   (2.230033)     // Identified phase to neutral resistance in a Y equivalent circuit (Ohms, float)

#define USER_MOTOR_Ls_d                 (0.0104946)  // For PM, Identified average stator inductance  (Henry, float)

#define USER_MOTOR_Ls_q                 (0.0104946)  // For PM, Identified average stator inductance  (Henry, float)

#define USER_MOTOR_RATED_FLUX           (0.2369867)    // Identified TOTAL flux linkage between the rotor and the stator (V/Hz)

#define USER_MOTOR_MAGNETIZING_CURRENT  (NULL)         // Induction motors only, else NULL

#define USER_MOTOR_RES_EST_CURRENT      (1.0)          // During Motor ID, maximum current (Amperes, float) used for Rs estimation, 10-20% rated current

#define USER_MOTOR_IND_EST_CURRENT      (-1.0)         // During Motor ID, maximum current (negative Amperes, float) used for Ls estimation, use just enough to enable rotation

#define USER_MOTOR_MAX_CURRENT          (5.0)         // CRITICAL: Used during ID and run-time, sets a limit on the maximum current command output of the provided Speed PI Controller to the Iq controller

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (15.0)         // During Motor ID, maximum commanded speed (Hz, float), ~10% rated

#elif (USER_MOTOR == Fan_motor)                  // Name must match the motor #define

#define USER_MOTOR_TYPE                 MOTOR_Type_Pm  // Motor_Type_Pm (All Synchronous: BLDC, PMSM, SMPM, IPM) or Motor_Type_Induction (Asynchronous ACI)

#define USER_MOTOR_NUM_POLE_PAIRS       (1)            // PAIRS, not total poles. Used to calculate user RPM from rotor Hz only

#define USER_MOTOR_Rr                   (NULL)         // Induction motors only, else NULL

#define USER_MOTOR_Rs                   (0.1217002)     // Identified phase to neutral resistance in a Y equivalent circuit (Ohms, float)

#define USER_MOTOR_Ls_d                 (0.00001094549)  // For PM, Identified average stator inductance  (Henry, float)

#define USER_MOTOR_Ls_q                 (0.00001094549)  // For PM, Identified average stator inductance  (Henry, float)

#define USER_MOTOR_RATED_FLUX           (0.4343111)    // Identified TOTAL flux linkage between the rotor and the stator (V/Hz)

#define USER_MOTOR_MAGNETIZING_CURRENT  (NULL)         // Induction motors only, else NULL

#define USER_MOTOR_RES_EST_CURRENT      (9.0)          // During Motor ID, maximum current (Amperes, float) used for Rs estimation, 10-20% rated current

#define USER_MOTOR_IND_EST_CURRENT      (-4.5)         // During Motor ID, maximum current (negative Amperes, float) used for Ls estimation, use just enough to enable rotation

#define USER_MOTOR_MAX_CURRENT          (28.0)         // CRITICAL: Used during ID and run-time, sets a limit on the maximum current command output of the provided Speed PI Controller to the Iq controller

#define USER_MOTOR_FLUX_EST_FREQ_Hz     (30.0)         // During Motor ID, maximum commanded speed (Hz, float), ~10% rated

#else

#error No motor type specified

#endif

#ifndef USER_MOTOR

#error Motor is not defined in user.h

#endif

#ifndef USER_MOTOR_TYPE

#error The motor type is not defined in user.h

#endif

#ifndef USER_MOTOR_NUM_POLE_PAIRS

#error Number of motor pole pairs is not defined in user.h

#endif

#ifndef USER_MOTOR_Rr

#error The rotor resistance is not defined in user.h

#endif

#ifndef USER_MOTOR_Rs

#error The stator resistance is not defined in user.h

#endif

#ifndef USER_MOTOR_Ls_d

#error The direct stator inductance is not defined in user.h

#endif

#ifndef USER_MOTOR_Ls_q

#error The quadrature stator inductance is not defined in user.h

#endif

#ifndef USER_MOTOR_RATED_FLUX

#error The rated flux of motor is not defined in user.h

#endif

#ifndef USER_MOTOR_MAGNETIZING_CURRENT

#error The magnetizing current is not defined in user.h

#endif

#ifndef USER_MOTOR_RES_EST_CURRENT

#error The resistance estimation current is not defined in user.h

#endif

#ifndef USER_MOTOR_IND_EST_CURRENT

#error The inductance estimation current is not defined in user.h

#endif

#ifndef USER_MOTOR_MAX_CURRENT

#error The maximum current is not defined in user.h

#endif

#ifndef USER_MOTOR_FLUX_EST_FREQ_Hz

#error The flux estimation frequency is not defined in user.h

#endif

// **************************************************************************

// the functions

//! \brief      Sets the user parameter values

//! \param[in]  pUserParams  The pointer to the user param structure

extern void USER_setParams(USER_Params *pUserParams);

//! \brief      Checks for errors in the user parameter values

//! \param[in]  pUserParams  The pointer to the user param structure

extern void USER_checkForErrors(USER_Params *pUserParams);

//! \brief      Gets the error code in the user parameters

//! \param[in]  pUserParams  The pointer to the user param structure

//! \return     The error code

extern USER_ErrorCode_e USER_getErrorCode(USER_Params *pUserParams);

//! \brief      Sets the error code in the user parameters

//! \param[in]  pUserParams  The pointer to the user param structure

//! \param[in]  errorCode    The error code

extern void USER_setErrorCode(USER_Params *pUserParams,const USER_ErrorCode_e errorCode);

//! \brief      Recalculates Inductances with the correct Q Format

//! \param[in]  handle       The controller (CTRL) handle

extern void USER_softwareUpdate1p6(CTRL_Handle handle);

//! \brief      Updates Id and Iq PI gains

//! \param[in]  handle       The controller (CTRL) handle

extern void USER_calcPIgains(CTRL_Handle handle);

//! \brief      Computes the scale factor needed to convert from torque created by Ld, Lq, Id and Iq, from per unit to Nm

//! \return     The scale factor to convert torque from (Ld - Lq) * Id * Iq from per unit to Nm, in IQ24 format

extern _iq USER_computeTorque_Ls_Id_Iq_pu_to_Nm_sf(void);

//! \brief      Computes the scale factor needed to convert from torque created by flux and Iq, from per unit to Nm

//! \return     The scale factor to convert torque from Flux * Iq from per unit to Nm, in IQ24 format

extern _iq USER_computeTorque_Flux_Iq_pu_to_Nm_sf(void);

//! \brief      Computes the scale factor needed to convert from per unit to Wb

//! \return     The scale factor to convert from flux per unit to flux in Wb, in IQ24 format

extern _iq USER_computeFlux_pu_to_Wb_sf(void);

//! \brief      Computes the scale factor needed to convert from per unit to V/Hz

//! \return     The scale factor to convert from flux per unit to flux in V/Hz, in IQ24 format

extern _iq USER_computeFlux_pu_to_VpHz_sf(void);

//! \brief      Computes Flux in Wb or V/Hz depending on the scale factor sent as parameter

//! \param[in]  handle       The controller (CTRL) handle

//! \param[in]  sf           The scale factor to convert flux from per unit to Wb or V/Hz

//! \return     The flux in Wb or V/Hz depending on the scale factor sent as parameter, in IQ24 format

extern _iq USER_computeFlux(CTRL_Handle handle, const _iq sf);

//! \brief      Computes Torque in Nm

//! \param[in]  handle          The controller (CTRL) handle

//! \param[in]  torque_Flux_sf  The scale factor to convert torque from (Ld - Lq) * Id * Iq from per unit to Nm

//! \param[in]  torque_Ls_sf    The scale factor to convert torque from Flux * Iq from per unit to Nm

//! \return     The torque in Nm, in IQ24 format

extern _iq USER_computeTorque_Nm(CTRL_Handle handle, const _iq torque_Flux_sf, const _iq torque_Ls_sf);

//! \brief      Computes Torque in lbin

//! \param[in]  handle          The controller (CTRL) handle

//! \param[in]  torque_Flux_sf  The scale factor to convert torque from (Ld - Lq) * Id * Iq from per unit to lbin

//! \param[in]  torque_Ls_sf    The scale factor to convert torque from Flux * Iq from per unit to lbin

//! \return     The torque in lbin, in IQ24 format

extern _iq USER_computeTorque_lbin(CTRL_Handle handle, const _iq torque_Flux_sf, const _iq torque_Ls_sf);

#ifdef __cplusplus

}

#endif // extern "C"

//@} // ingroup

#endif // end of _USER_H_ definition