Contents

Summary of Issue:	1

TI Reference Design:	1

Schematic:	2

PCB Layout:	2

BOM:	2

Deviations from Reference Design:	3

Modifications:	3

Observations and Screen Captures After Modifications:	3

B1 Oscope Captures:	4

B2 Oscope Captures:	5

B3 Oscope Captures:	6

Next Steps:	6

Changes for Rev:	6

# Summary of Issue:

Non-systematic Issues to include:

- Avg Vout 3V3, but noise is 800 – 1000mv

- Vout varies with varying load current (outside of 10% of designed Vout)

- One board has a Vout of 3V8 until under load, at which point Vout settles to 3V3 as desired

  - Issue seen only one case. Other cases Vout is ~3V3 and falls ~50mV when load switches on

- Vout regulation is ~2V70, not the designed 3V3

- Spontaneous “Square Wave” in which regulation falls from 3V3 to ~2V5 in a random square wave like pattern

- Some boards had regulator circuits fail completely causing a short-circuit

- Sometimes regulator works very well and seems to be grumpy only the next day

# TI Reference Design:

https://webench.ti.com/appinfo/webench/scripts/SDP.cgi?ID=40C1E499DED81B82

# Schematic:

# PCB Layout:

# BOM:

| Designator | Description | Manufacturer | Manufacturer Part Number |
| --- | --- | --- | --- |
| C11, C12, C13 | MLCC 10 µF, 10 V, 0402 [1005 Metric], ± 20%, X5R, CL Series | Samsung | CL05A106MP5NUNC |
| FB1 | 600 Ohms @ 100MHz 1 Ferrite Bead 0603 (1608 Metric) 500mA 380mOhm | Murata | BLM18AG601SN1D |
| FB2 | Ind Chip Bead 220Ohm 25% 100MHz Ferrite 2A 0603 Paper T/R | Murata | BLM18EG221SN1D |
| L1 | SMD Power Inductor, 2.2 uH, +/- 20%, 3 x 3 x 1.5 mm, -25 to 120 degC | Taiyo Yuden | NR3015T2R2M |
| R9 | RES SMD 10K OHM 1% 1/16W 0402 | Bourns | CR0402-FX-1002GLF |
| R10 | Thick Film Resistors - SMD 562K OHM 1% | Bourns | CR0402-FX-5623GLF |
| R11 | 0402 180K Ohm 1% 1/16W | Bourns | CR0402-FX-1803GLF |
| R12 | 0402 [1005 Metric], 100 kohm, CR Series, 50 V, Thick Film, 62.5 mW | Bourns | CR0402-FX-1003GLF |
| U3 | DC/DC Buck | Texas Instruments | TPS62160DSGR |

# Deviations from Reference Design:

- Use of 0402 input and output caps

- Use of Ferrite Beads at Input/Output

# Modifications:

- 0402 caps > 0805 caps to deal with DC Voltage Bias

- 20uF > 30uF @ output

- 2.2uH Inductor > 3.3uH Inductor

# Observations and Screen Captures After Modifications:

## B1 Oscope Captures:

Figure 1: B1 Power Up

Figure 2: B1 Power Down

## B2 Oscope Captures:

Figure 3: B2 Power Up (Note: ~1000mV noise starts when LED load turns on, ~80mA draw)

Figure 4: Power Down

## B3 Oscope Captures:

Figure 5: B3 Power Up (Note: Starts at 3V3 > 2V5 > high noise @ LED load, ~ 80mA,  turn on)

# Next Steps:

- AGND? See DS layout guidelines

- Swap 3V3 rail with 5V_STBY (Reverse TXS0102 Ports to handle VCCA <= VCCB). Ensure to isolate DC-DC Regulator!

- Characterize the TPS62160EVM

- Hack TPS62160 into system and characterize performance

# Changes for Rev:

- Add Load/Discharge Resistor to Vout (DNP)

- Swap positions of R12 and R11 to reduce number of vias needed for routing. Also implement Power and Analog Ground layout per datasheet recommended layout.