- If you’re using CCS6.1, and the compiler version is higher than 5.1.10. Then you should do some additional actions:

  - In CCS6.1, import the project that converted by GUI.

  - Right click on project ->properties ->General -> Change the compiler version to 5.2.2 or higher than this version. Then click OK

  - As for this compiler only support ‘ti_arm9_abi’, which all of the 16bit and 32bit entry point with underscore.

  - Open load.asm, change the entry point from assembly language to C

Original code:

_c1_:

BL     $c_int00

New code:

_c1_:

BL     _c_int00

  - Configure the interrupt.c, standard_interrupt.c, software_interrupt.c(if your project have), clear_program_flash.c, zero_out_intergrity_word.c(if your project have) as 32bit code state.

Right click on those files one by one, -> properties -> ARM compiler -> processor options

Select the code state as 32bit.

After doing the above actions, your project can be compiled and linked correctly in CCS6.