Customer use LM5160 as fly-buck controller to provide Aux power for IGBT gate driver application, and reported several issues for startup/overcurrent/Cbst drop down.

The system setting is list as below:

- Input Range:  15~18V[Typical: 16.5V]

- Primary Output:  7V @ No load;

- Secondary Output: 24V @ 0.13A;

And the system setting are list as below excel file, the schematic shown as below:

Reported Issues:

- Could not build the output voltage while in no load operation,  test waveform as figure 1:

CH1: Input Current CH2: Input Voltage CH3: EN/UVLO PIN CH4: Secondary output(Setting to 25V, [+14 – (-11)];

- The input current reach up to 4A during start-up, no matter output is ready or not, shown as figure 1& Figure 2;

CH1: Input Current CH2: Input Voltage CH3: EN/UVLO PIN CH4: Secondary output(Setting to 25V, [+14 – (-11)];

- The bootstrap(CBST) capacitor voltage would dropped down, shown as figure 3;

CH3: CBST Voltage

Support Needs:

- Could you kindly review the schematic and the parameters setting, and give comments on issue 1/2/3;

- For Issue1, could you kindly give comments why the no load/light load may not build correctly output?

- While using LM5160 as fly-buck controller,  the Primary-side output should less than ½ of input minimum,   the general understanding is for helping minimize the secondary peak current and improving load regulation,  could you kindly share more information?