# Texas Instruments

# HPA026 (TPS40090EVM-001) Test Procedure

# Rev. B

1.0 GENERAL

1.1 PURPOSE

The purpose of this document is to provide detailed instructions for testing the TPS40090EVM-001 Modules.

1.2 SCOPE

This document covers complete instructions for testing TPS40090EVM-001. Upon completion of this procedure, all units will be categorized as either conforming or non-conforming material and dispositioned accordingly.

1.3 REFERENCE DOCUMENTATION

    - Schematic HPA026 Rev.A

    - HPA026 Assembly DWGs

    - HPA026 user’s guide, SLUU175

1.4 MATERIALS

1.4.1 Test Log: Test Log attached to end of this procedure.

1.5 DEFINITIONS

N/A

2.0 SAFETY

2.1 EYE PROTECTION

Safety Glasses are to be worn while performing all testing on the TPS40090EVM-001.

2.2 GENERAL RISKS

This test must be performed by qualified personnel trained in electronics theory and

understand the risks and hazards of the assembly to be tested.

2.3 ELECTROSTATIC DISCHARGE

ESD precautions must be followed while handling electronic assemblies while

performing this test.

2.4 THERMAL/SHOCK HAZARDS

Precautions should be observed to avoid touching areas of the assembly that may get

hot or present a shock hazard during testing.

3.0 QUALITY

3.1 DATA PRESERVATION

All test data is to be recorded and maintained on the log provided at the end of this document.

3.2 DATA ACCESS

Test data shall be made available upon request by Texas Instruments.

4.0 APPAREL

4.1 ELECTROSTATIC SMOCK

4.2 ELECTROSTATIC GLOVES OR FINGER COTS

4.3 SAFETY GLASSES

4.4 GROUND ESD WRIST STRAP.

5.0 EQUIPMENT

5.1 POWER SUPPLIES

One power supply is needed and is capable of supplying 10.5V – 14VDC @ 4A. Use at least 12 gauge cables to connect the HPA026 (TPS40090EVM-001).

5.2 LOADS

An electronic load or resistor load is needed and is capable of 10A- 50A @1.5V. Use at least gauge 8 cables to connect the HPA026 (TPS40090EVM-001) and load.

5.3 METERS

One Fluke 45 DVM or equivalent is required.

5.5 OSCILLOSCOPE

A minimum 50MHz digital oscilloscope and one voltage probe is required.

6.0 EQUIPMENT SETUP

6.1. Connect test equipment and HPA026 (TPS40090EVM-001) demo board as shown in Figure 1.

Fig.1 Equipment setup for HPA026 (TPS40090EVM-001) demo board

6.2. Set the oscilloscope as follows:

HORIZONTAL SWEEP: 500ns/div.

TRIGGER MODE: auto, falling edge

TRIGGER SOURCE: Ch1

TRIGGER LEVEL: 400uV

CH1: 10mV/div, AC coupled, bandwidth 20MHz

7.0 PROCEDURE

Steady-State operation

Any unit that “Fails”  this test should be marked with the test step failed and then dispositioned according to section 9.

7.1 Setup the equipment as shown in Fig. 1. Use Ch1 to probe J8.

  - Preset the input voltage for  10.5V  at J1. Set electronic load at 10.0A DC.

  - Turn on the input power supply. Apply 10amp load.  Read the input current and output voltage from the meters. The input current should be less than 2.0 A and output voltage is at 1.46~1.55V.

  - Set load to 50 Amps and verify voltage regulation between 1.46V~1.55V’s.

  - Set Vin to 14 Volts and verify voltage regulation between 1.46V~1.55V’s.

  - Set load to 10 Amps and verify voltage regulation between 1.46V~1.55V’s.

  - Measure the output ripple voltage. It should look similar to Fig.2. The peak-peak value

between cursors should be less than 15mV.

  - Turn off the power supply. Leave load set to 10 Amps.

Fig.2 Output voltage ripple

Transient response

  - Change the scale of oscilloscope to 25us/div and 200mV/div.

  - Put a jumper on J3 to connect Pin 1 and Pin 2.

  - Turn on the power supply. The waveform on the scope should look like that in Fig.3.

Measure the total overshoot and undershoot voltage, which should be less than 450mV.

  - Turn off the power supply.

  - Put the jumper back to connect Pin 2 and Pin 3 of J3.

Fig. 3 Load transient test

7.14 Loop back to 7.1 and continue until all units have been tested.

8.0 EQUIPMENT SHUTDOWN

8.1 Verify all the equipments are powered off.

9.0 MATERIAL DISPOSITION & TRANSFER

## 9.1 CONFORMING MATERIAL

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled HPA026 and shipped per the P.O.

## 9.2 NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches or exceeds 2%, contact EVM coordinator for assistance.