# Request for Quote: PCB Fabrication

## PCB Part No: SV600711 Rev. A

Board Size: 2950x1800 mil

No of Layers: 2

Solder Mask: Green

Silkscreen: White, Top and Bottom

Surface Finish: Immersion Gold

Total Thickness: 0.062” +/- 10%

RoHS Certification

Panelization: YES, Scored. Remain in panels for assembly

Surface Mount Pad: Top Only

Top and Bottom Layer: 2 oz copper

Drill Data:

Size				Count		Plated

------------------------------------------

12.99mil (0.32995mm)	 4		PTH

14mil (0.3556mm)		34		PTH

40mil (1.016mm)		 1		PTH

63mil (1.6002mm)		 1		PTH

98.43mil (2.50012mm)	 4		PTH

125.98mil (3.19989mm)	 4		PTH

------------------------------------------

Totals		48