# Texas Instruments

# PWR551(Mother Board)/471(Daughter Board)

# 390V to 12V, 600W Phase Shifted Full Bridge

UCC27714

# Rev. A, 9/17/23

1.0 GENERAL

1.1 PURPOSE

1.1.1 To provide detailed instructions for testing EVM modules

1.2 SCOPE

1.2.1 Covers complete instructions for testing the PWR551 EVM module.

1.3 REFERENCE DOCUMENTATION

    - Users Guide

2.0 SAFETY

2.1 Safety Glasses are to be worn.

2.2 This test must be performed by qualified personnel trained in electronics theory and understand the risks and hazards of the assembly to be tested.

2.3 ESD precautions must be followed while handling electronic assemblies while

performing this test.

2.4 Precautions should be observed to avoid touching areas of the assembly that may get

hot or present a shock hazard during testing.

  - There are High Voltages on this EVM please take the proper safety precautions.

  - Monitor the Input voltage and do not handle the EVM if the output voltage exceeds 50V.

3.0 QUALITY

3.1 Before the unit can be shipped out as good it must have been 100% tested and passed all tests.

3.2 The unit must also be built per specifications detailed in the bill of materials.

4.0 APPAREL

4.1 Electrostatic smock

4.2 Electrostatic Gloves or finger cots

4.3 Safety Glasses

4.4 For safety do not use a ground strap. Use ionizer to reduce ESD damage.

5.0 EQUIPMENT

5.1   500V DC Source Capable of 700W

5.2   2, 0-20V Bias Supplies

5.5   3 Digital Volt Meters

5.6   400 LFM Fan to cool EVM

5.7   Oscilloscope

- Scope Probe with ferrite toroid wrapped around scope probe wire to reduce common mode noise.

5.8   Precision (+/- 0.1%) Shunt Resistor (RSHUNT) of 5A/100mV to measure input current

5.9   50 Volt, Watt Electronic Load Capable of 700W note resistive loads can be used to achieve the same power levels.

- If electronic load is not used the following resistive loads will be required.

- 2.4 ohm load resistance cable of handling 70W

  - Note a resistance within +/- 1% of the specified load resistance can be used for this test.

- 1.2 ohm load resistance cable of handling 140W

  - Note a resistance within +/- 1% of the specified load resistance can be used for this test.

- 0.24 ohm load resistance cable of handling 700W

  - Note a resistance within +/- 1% of the specified load resistance can be used for this test.

6.0 EQUIPMENT SETUP

6.1 Scope Probe needs to be set to ac 100mV/div, 5us/div, with 20MHz bandwidth limiting.

Diagram 1

7.0 PROCEDURE

7.1 Plug HPA471 Daughter Board into the PWR551 Mother Board

# TESTS If Resistor Loads are Used

- Use setup in step 6.0

- Turn on 400LFM fan pointed at heat sinks per diagram 1.

- Connect 2.4 ohm resistive load per diagram 1

- Connect 0 to 20V bias supplies as shown in diagram 1set at 0V.

- Set current limit to 200mA

- Connect 500V DC source to the input.

- Set current limit to 2A

- Connect scope prober, meters and RSHUNT per diagram 1.

- Apply 390V DC to the input of the power converter with the 500V DC source.

- Set the 0 to 20V bias supplies to 12V.

- Output voltage should be between 11.6 and 12.6V (Volt Meter 3).

- Yes = Pass

- No = Fail

- Iin=Input current  should be > 0.175A (Checks DCM Threshold)

- Voltmeter 2 > 3.5mV Device Pass

- Voltmeter 2 < 3.5mV Device Fails

- Set the 0 to 20V bias supplies to 0V.

- Power down the 500V DC and do not proceed until the input voltage drops below 50V (Volt Meter 1) and then disconnect the load from the output.

- Connect 1.2 ohm resistive load per Diagram 1

- Apply 390V DC to the input of the power converter with the 500V DC source.

- Set the 0 to 20V power supplies to 12V.

- Output voltage should be between 11.6 and 12.6V (Volt Meter 3).

- Yes = Pass

- No = Fail

- Iin=Input current  should be < 0.34A (Checks DCM Threshold)

- Voltmeter 2 > 6.8mV Device Pass

- Voltmeter 2 < 6.8mV Device Fails

- Set the 0 to 20V bias supplies to 0V.

- Power down the 500V DC and do not proceed until the input voltage drops below 50V (Volt Meter 1) and then disconnect the load from the output.

- Connect 0.24 ohm resistive load per Diagram 1

- Apply 390V DC to the input of the power converter with the 500V DC source.

- Set the 0 to 20V bias supplies to 12V.

- Output voltage should be between 11.6 and 12.6V (Volt Meter 3).

- Yes = Pass

- No = Fail

- Iin=Input current  should be < 1.7A

- Voltmeter 2 > 34mV Device Pass

- Voltmeter 2 < 34mV Device Fails

- Output Ripple Voltage Should Look Like Figure 1.

- Vripple > 200mV Device Fails

- Power down the input and bias supply do not proceed until the boost voltage drops below 50V and then disconnect the load from the output.

- Mark board as GOOD if DUT if it meets all test requirements without failing.

- Disconnect unit from test setup

# TESTS If Electronic Load is Used

- Use setup in step 6.0

- Turn on 400LFM fan pointed at heat sinks per diagram 1.

- Connect electronic load per diagram 1.

- Set Electronic Load to 2.4 ohm

- Connect 0 to 20V bias supplies as shown in diagram 1set at 0V.

- Set current limit to 200mA

- Connect 500V DC source to the input.

- Set current limit to 2A

- Connect scope prober, meters and RSHUNT per diagram 1.

- Apply 390V DC to the input of the power converter with the 500V DC source.

- Set the 0 to 20V bias supplies to 12V.

- Output voltage should be between 11.6 and 12.6V (Volt Meter 3).

- Yes = Pass

- No = Fail

- Iin=Input current  should be > 0.175A (Checks DCM Threshold)

- Voltmeter 2 > 3.5mV Device Pass

- Voltmeter 2 < 3.5mV Device Fails

- Set Electronic Load to 1.2 ohm

- Output voltage should be between 11.6 and 12.6V (Volt Meter 3).

- Yes = Pass

- No = Fail

- Iin=Input current  should be < 0.34A (Checks DCM Threshold)

- Voltmeter 2 > 6.8mV Device Pass

- Voltmeter 2 < 6.8mV Device Fails

- Set Electronic Load to 0.24 ohm

- Output voltage should be between 11.6 and 12.6V (Volt Meter 3).

- Yes = Pass

- No = Fail

- Iin=Input current  should be < 1.7A

- Voltmeter 2 > 34mV Device Pass

- Voltmeter 2 < 34mV Device Fails

- Output Ripple Voltage Should Look Like Figure 1.

- Vripple > 200mV Device Fails

- Power down the input and bias supply do not proceed until the boost voltage drops below 50V (Volt Meter 1) and then disconnect the load from the output.

- Mark board as GOOD if DUT if it meets all test requirements without failing.

- Disconnect unit from test setup

Figure 1 Vin = 390V RMS, Load = 0.24 ohms

8.0 EQUIPMENT SHUTDOWN

8.1 Power down DC input.

8.2 Monitor output voltage.

8.3 When the output voltage drops below 50V the test operator may disconnect the EVM under

test.

8.4 Turnoff the 12V DC supply.

8.5 Disconnect unit under test

8.6 Unplug daughter board controller HPA471.

9.0 MATERIAL DISPOSITION & TRANSFER

## 9.1 CONFORMING MATERIAL

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled with two labels according to the table below, and shipped per the P.O.

| Description | Label 1 | Label 2 |
| --- | --- | --- |
| Daughter Card | HPA471 | UCC27714 |
| Mother Board | PWR551 | UCC27714 |

## 9.2 NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches or exceeds 5%, contact EVM coordinator for assistance.