Texas Instruments

PWR716 Test Procedure

Rev. A

25 Feb, 2016

# GENERAL

## PURPOSE

The purpose of this document is to provide detailed instructions for testing the modules of Table 1.

Table 1

| TPS54202EVM-716 | PWR716-001 |
| --- | --- |
| TPS54202HEVM-716 | PWR716-002 |
| TPS54302EVM-716 | PWR716-003 |

## SCOPE

This document covers complete instructions for testing PWR716 modules listed above in Table 1.  Upon completion of this procedure, all units will be categorized as either conforming or non-conforming material and dispositioned accordingly.

## REFERENCE DOCUMENTATION

- Schematic PWR716

- PWR716 Assembly DWGs

## MATERIALS

### Test Log: A test log is not required for this procedure.

## DEFINITIONS

N/A

# SAFETY

## EYE PROTECTION

Safety Glasses are to be worn while performing all testing on the PWR716.

## GENERAL RISKS

This test must be performed by qualified personnel trained in electronics theory and understand the risks and hazards of the assembly to be tested.

## ELECTROSTATIC DISCHARGE

ESD precautions must be followed while handling electronic assemblies.

## THERMAL/SHOCK HAZARDS

Precautions should be observed to avoid touching areas of the assembly that may get hot or present a shock hazard during testing.

# QUALITY

## DATA PRESERVATION

Recorded test data is not required for this procedure.

# APPAREL

## ELECTROSTATIC SMOCK

## ELECTROSTATIC GLOVES OR FINGER COTS

## SAFETY GLASSES

## GROUND ESD WRIST STRAP

# EQUIPMENT

## POWER SUPPLIES

A power supply capable of supplying 24 volts @ 2 amps is required to supply Vin.

## LOAD

One electronic load is required. This electronic load must have a resistive load setting.  If this is not available a resistive load may be substituted.  See Table 2 for load requirements.

Table 2

| CIRCUIT UNDER TEST | OUTPUT VOLTAGE | LOAD REQUIREMENT |
| --- | --- | --- |
| PWR716-001 | 5 V output | 2 A or 2.5 ohm, 10 W resistive load |
| PWR716-002 | 5 V output | 2 A or 2.5 ohm, 10 W resistive load |
| PWR716-003 | 5 V output | 3 A or 1.67ohm, 15 W resistive load |

## METERS

Two DC voltmeters are required.

## OSCILLOSCOPES

An oscilloscope and two voltage probes are required.

## FAN

This converter includes components that can get hot to touch.  A small fan capable of between 200 LFM and 400 LFM is recommended (but not required) to reduce component temperatures when operating the evaluation module.

# EQUIPMENT SETUP

## GENERAL

## INPUT SUPPLY

Set the input voltage power supply to 24 V and the current limit to 2 A.  Disable power supply. Connect the input power supply to the J1 connector of the PWR716, with the positive terminal of the supply connected to VIN (J1-1) and the negative terminal of the supply connected to GND (J1-2).

## LOAD

The load should be connected to the PWR716 at the J2 connector with the positive terminal of the load connected to VOUT (J2-1) and the negative terminal of the load connected to GND (J2-2). If using an electronic load, make sure it is set to resistive or constant resistance mode.

## METERS

For the input voltage, connect the positive lead of the first voltmeter to test point TP1, and the negative terminal of the voltmeter to test point TP2. For the output voltage, connect the positive terminal of the second voltmeter to test point TP6, and the negative terminal of the voltmeter to test point TP7.

## OSCILLOSCOPE

The oscilloscope and voltage probes are used to measure the output voltage ripple and switching frequency at steady state condition.  The oscilloscope channel CH1 used for the voltage measurement should be set to AC coupling, 10 mV/div vertical scale, 1us/div time scale, and bandwidth limited to 20 MHz. Oscilloscope channel CH2 should be set to DC coupling, 1 V/div.  Set the oscilloscope to trigger on CH2 at a voltage level of 5V.  The probes will be connected in section 7.3.  The CH1 voltage probe should be used to measure the output voltage ripple as close as possible to C5 or C6. Use as short a ground connection as possible to minimize noise pick up.  The CH2 voltage probe will be connected in sections 7.3.

## EVM SETTINGS

Ensure that the pins of jumper JP1-1 is open (not covered).  This configuration will allow the PWR716 to be ‘ON’ or enabled.

# PROCEDURE

## STEP ONE:  APPLY POWER AND MEASURE 5 VOLT OUTPUT AT NO LOAD

Ensure that the input power supply current limit is set at 2 amps.  Ensure that the load current is set to 0 A, or if using a resistive load ensure it is disconnected.  Using the input power supply, apply 24 V input voltage at J1.  Verify the input voltage at TP1 relative to TP2 is within 23.9 V to 24.1 V.  Verify the output voltage at TP6 relative to TP7 is within 4.85 V to 5.15 V.

Do NOT exceed an input voltage of 28 volts for the PWR716 evaluation module at any time the unit under test is connected to the input supply.

## STEP TWO: SET 5 V LOAD CURRENT AND MEASURE VOUT

Adjust the load current or attach resistive load according to Table 2.  If attaching a resistive load turn off or disable the input voltage source before attaching the resistive load, then turn on or enable the input voltage source. Verify the input voltage measured at TP1 relative to TP2 is within 23.9 V to 24.1 V.  Verify the output voltage at TP6 relative to TP7 is within 4.85 V to 5.15V.

## STEP THREE: 5 V OUTPUT RIPPLE AND FREQUENCY TESTING

Make sure the circuit under test is loaded per the load table above.  Use the CH1 probe to measure the output directly across or as close as possible to C5 or C6 using the shortest ground lead that is practical.  Use the CH2 probe to measure the voltage at the switching node (TP4).  The peak-to-peak voltage measurement should measure only the amplitude of the fundamental frequency ripple, and should neglect any high frequency noise or spikes.  Verify the peak to peak output voltage ripple is less than 50mV.  Use the PH waveform of CH2 to measure the operating frequency.  Frequency is the inverse of period T, or f = 1/T. Verify the operating frequency (001/002) is within the range of 370 kHz to 630 kHz. Verify the operating frequency (003) is within the range of 296 kHz to 504 kHz. Disconnect the probes.

## STEP FOUR: 5 V CIRCUIT ENABLE TESTING

Turn off or disable the input voltage power supply and disconnect the load from the J2 connector.  Turn on or enable the input voltage power supply and verify that the input voltage at TP1 relative to TP2 is within 23.9 V to 24.1 V. Verify the output voltage at TP6 relative to TP7 is within 4.85 V to 5.15 V range.  Install the shunt across JP1-1 and JP1-2.  Verify the output voltage at TP6 relative to TP7 is within -0.120V to +0.120V.  Remove the shunt from JP1-1 and JP1-2.    Verify the output voltage at TP6 relative to TP7 is within 4.85 V to 5.15 V.

Once this measurement is completed, power off the input supply and disconnects the unit under test.

## STEP FIVE: MATERIAL DISPOSITION

All units (passed and failed) shall be dispositioned according to section 9.  Continue testing by going back to step one (section 7.1) until all units have been tested.

# EQUIPMENT SHUTDOWN

N/A

# MATERIAL DISPOSITION & TRANSFER

## CONFORMING MATERIAL

Units that have passed this test procedure shall have the shunts set as in Table 3, be packaged into anti-static ESD approved bags, labeled as indicated in Table 4 and shipped per the P.O.

Table 3

| Jumper | Shunt Position |
| --- | --- |
| JP1 | Cover JP1-1 only |

Table 4

| Label 1 | Label2 |
| --- | --- |
| PWR716-001 | TPS54202DDC |
| PWR716-002 | TPS54202HDDC |
| PWR716-003 | TPS54302DDC |

## NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches or exceeds 5%, contact EVM coordinator for assistance.