# Texas Instruments

EVM Test Procedure

# BSR107: LM60440AQEVM Test Procedure

# Rev. A

10/23/2019

- GENERAL

  - PURPOSE

    - To provide detailed instructions for testing LM60440AQEVM.

  - SCOPE

    - Covers complete instructions for testing LM60440AQEVM.

  - REFERENCE DOCUMENTATION

    - Schematics:

    - Assembly DWGs:

    - Users Guide: SNVU693

  - MATERIALS

- 1x Power supply that can handle 36Vin and up to 4.0A output

- 2x DMM one to sense output voltage, the other used to sense input current

- 1x Electronic load that can handle at least 4A of DC current.

  - DEFINITIONS

    - DMM is reference to Digital Mustimeters

    - UUT is reference to Unit under Test

    - EVM is reference to Evaluation Module assembly, in this case the UUT

    - “Hi Voltage” is defined as DC voltages > 75V and AC voltages > 50 volts (RMS)

- SAFETY

  - Safety Glasses are to be worn.

  - This test must be performed by qualified personnel trained in electronics theory and understand the risks and hazards of the assembly to be tested.

  - ESD precautions must be followed while handling electronic assemblies while performing this test.

  - Precautions should be observed to avoid touching areas of the assembly that may get hot or present a shock hazard during testing.

  - NO ESD wrist strap shall be worn for Hi Voltage testing (=>50 Vrms or =>75 VDC) use Ionizer.

- QUALITY

  - Test data or reports shall be made available upon request by Texas Instruments.

- APPAREL

  - Electrostatic smock

  - Electrostatic Gloves or finger cots

  - Safety Glasses

  - Ground ESD wrist strap

- EQUIPMENT

  - Specify all equipment needed to perform tests

- 1x Power supply that can handle 36V and up to 4A output

- 2x DMM to sense output voltage and input current currents up to 4A should be tolerated by the DMM used to sense current.

- 1x Electronic  load that can handle at least 4A of DC current

- EQUIPMENT SETUP

  - Connect a DMM set as current meter to power supply positive output terminal.  The negative current sense terminal will be connected to the board under test in step 7.4 – This DMM will be used for sensing input current and be referred to as DMM1.

  - Set the second DMM, called DMM2, to operate as a voltage meter.

  - Set power supply voltage to 13V +/-1V. Current limit should be set to 4A or higher.

  - Electronic load setting will be 4A in constant current mode.

- PROCEDURE (loop through this section - once for each board)

  - Place shunt jumper (SH-J1) on jumper (J3) in position 2-3

  - Place shunt jumper (SH-J2) on jumper (J4) in position 2-3

  - Connect the Eload negative input to the board’s TP7 (GND) terminal

  - Connect the Eload positive input to the board’s TP6 (VOUT) terminal

  - Connect Power Supply’s negative output to the board’s J2 (GND) terminal

  - Connect DMM1’s ‘current in’ terminal to the Power Supply’s positive output. Connect DMM1’s negative (current out rather than current in) terminal to the board’s J1(VIN) terminal.

  - Enable power supply which should be set to provide 13V.  If the power supply does not have an output enable, the board under test may be hot plugged into the power supply.

  - Connect the DMM2 negative connection to TP7 (GND) terminal adjacent to the board’s VOUT terminal.

  - Place the DMM2’s positive probe on the CO2 capacitor pad next to the VOUT connector.

  - Verify output voltage on the DMM2. See table below for expected reading of “Output #1.”

  - Apply 4.0A load on the output and wait for at least 5 seconds

  - Verify output voltage on the DMM2. See table below for expected reading of “Output #1”.

  - Disable Eload

  - Change power supply output to 36V

  - Place the DMM2’s positive probe on the CO2 capacitor pad next to the VOUT connector.

  - Verify output voltage on the DMM2. See table below for expected reading of “Output #1”.

  - Change power supply output to 5.5V

  - Place the DMM2’s positive probe on the CO2 capacitor pad next to the VOUT connector.

  - Verify output voltage on the DMM2. See table below for expected reading of “Output #1”.

  - Turn off power supply.

  - Disable power supply – if the supply selected cannot be disabled, remove connection to power supply.

  - Remove all connections from the board under test

  - Repeat section 7 procedure on next board.

|  | No load – 13Vin (step8) |  |  | Full load – 13Vin (step10) |  |  | No load - 36Vin (step14) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MIN | NORM | MAX | MIN | NORM | MAX | MIN | NORM | MAX |
| Output #1 | 4.75 | 5.00V | 5.25 | 4.75 | 5.00V | 5.25 | 4.75 | 5.00V | 5.25 |

| No load - 36Vin (step17) |  |  |
| --- | --- | --- |
| MIN | NORM | MAX |
| 4.75 | 5.00V | 5.25 |

EQUIPMENT SHUTDOWN

  - Turn Eload OFF

  - Turn off Power supply

- MATERIAL DISPOSITION & TRANSFER

  - Jumpers

Confirm shunt jumper (SH-J1) is on jumper (J3) in position 2-3

Confirm shunt jumper (SH-J2) is on jumper (J4) in position 2-3

  - CONFORMING MATERIAL

On the ESD bag, in text clearly visible and readable to the naked eye, add

For evaluation only; not FCC approved for resale.

This can be included on the label below or added as a separate label.

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled per table below and shipped per the P.O.

| Label 1 Assembly Number+Dash Number (if Applicable) | Label 2 EVM Orderable |
| --- | --- |
| BSR107 | LM60440AQEVM |

  - NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches, or exceeds 5%, contact EVM coordinator for assistance.

Change History:

| Date | Changes | Reason | By |
| --- | --- | --- | --- |
| 10/23/2019 | Original |  | Ethan Than |