DS.No :4300315R2 Date : 27.11.2015

- Electrical Drawing

DS.No :4300315R2 Date : 27.11.2015

- Winding Table:

| Sl.No. | Layer | Start | Finish | Description | Details |
| --- | --- | --- | --- | --- | --- |
| 01 | A | -- | -- | 1 Layer of Insulation Tape |  |
| 02 | B | 5 | 6 | 5T, 30SWG | 1 strand of 30SWG wire |
| 03 | C | -- | -- | 2 Layers of Insulation Tape |  |
| 04 | D | 3 | 1 | 60T, 30SWG | 1 strand of 30SWG wire |
| 05 | E | -- | -- | 2 Layers of Insulation Tape |  |
| 06 | F | 7 | 8 | 4T, 30SWG | 3 strands of 30SWG wire |
| 07 | G | -- | 1 | 1 Layer of Insulation Tape Shield as shown in figure 3. Connect shield to pin 1. 1 Layer of Insulation Tape |  |
| 08 | H | 10 | 9 | 2T, 30SWG | 7 strands of 30SWG wire |
| 09 | I | 12 | 11 | 3T,30SWG | 4 strands of 30SWG wire |
| 10 | J | -- | -- | 3 Layers of Insulation Tape |  |
| 11 | K | 16 | 15 | 3T, 30SWG | 2 strands of 30SWG wire |
| 12 | L | 15 | 14 | 3T, 30SWG | 8 strands of 30SWG wire |
| 13 | M | -- | -- | 2 Layers of Insulation Tape |  |
| 14 | -- | Set the core, match the primary inductance between pin 1 and pin 3 |  |  |  |
| 15 | N | External Shield |  | 1T, Copper Foil | Outside of core |