WITHIN THE ZIP FILE 5117_BUK_B.zip ARE THIS FILE, THE GERBER 274X, EXCELLON DRILL FILES AND A FAB DRAWING IN BOTH GERBER AND PDF FORMATS, FOR A BOARD REFERRED TO AS 5117_BUK_B. THE FILES AND DEFINITIONS ARE AS FOLLOWS:

5117_BUK_B.plc		Place, top side silkscreen

5117_BUK_B.stc		soldermask, top (component side) layer

5117_BUK_B.sts		soldermask, bottom (component side) layer*

5117_BUK_B.cmp		top (component side) layer

5117_BUK_B.sol		bottom (solder side) layer*

5117_BUK_B.ly2		first inner layer nearest top layer

5117_BUK_B.ly3		first inner layer nearest bottom layer*

5117_BUK_B.fab		hole positions and dimensions (for reference)

5117_BUK_B_FAB.pdf	hole positions and dimensions (for reference)

5117_BUK_B.drd		excellon drill file

5117_BUK_B.drl		drill tool list

5117_BUK_B.dri		drill station info

* DENOTES MIRRORED FILE

THE BOARD WILL BE 4 LAYER, FR4, 0.068” MAXIMUM THICKNESS, ALL LAYERS 2 OUNCE COPPER, GREEN SOLDERMASK, WHITE SILKSCREEN BOTH SIDES.  IMMERSION GOLD FINISH ON ALL EXPOSED METAL.  NO TESTING REQUIRED.

Grant Smith

National Semiconductor

15210 So. 50th St.

Suite 120, Mail Stop AZ7-N

Phoenix, AZ  85044

Tel: 480-629-2464

email: grant.smith@nsc.com