# Texas Instruments

EVM Test Procedure

# TPS536C7EVM-051 Test Procedure

# Rev. A

5/08/2019

- GENERAL

  - PURPOSE

    - To provide detailed instructions for testing EVM Modules

  - SCOPE

    - Covers complete instructions for testing BMC051

  - REFERENCE DOCUMENTATION

    - Schematic BMC051

    - Assembly DWG BMC051

    - Users Guide

Note: Please do not put references to revision letters of documents in the test procedure. This will only cause extra work when docs change.

  - MATERIALS

    - Test Log attached to end of this procedure.

  - DEFINITIONS

Add definitions of terms as required

    - DMM is reference to Digital Multimeters

    - LED Load is reference to boards with mounted LEDs

    - UUT is reference to Unit under Test

    - EVM is reference to Evaluation Module assembly, in this case the UUT

- SAFETY

  - Safety Glasses are to be worn.

  - This test must be performed by qualified personnel trained in electronics theory and understand the risks and hazards of the assembly to be tested.

  - ESD precautions must be followed while handling electronic assemblies while performing this test.

  - Precautions should be observed to avoid touching areas of the assembly that may get hot or present a shock hazard during testing.

- QUALITY

  - Test data or reports shall be made available upon request by Texas Instruments.

- APPAREL

  - Electrostatic smock

  - Electrostatic Gloves or finger cots

  - Safety Glasses

  - Ground ESD wrist strap

- EQUIPMENT

  - Voltage Source:

- 1 DC input voltage sources are needed (VIN). VIN should be a 0-14V variable DC source capable of supplying 40Adc

5.2 Multimeters:

- 2 DC voltmeters are needed to measure VIN and VOUTA

5.3 Load:

- 1 DC electronic load capable of 50A or greater.

5.4 Recommended Wire Gauge:

- 12VIN to T1 (+) and T2 (-) - The recommended wire size is AWG #10, with the total length of wire less than 4 feet (2 feet input, 2 feet return).

- Load to T3, T4 (+) and T7, T8 (-) (RailA output) – The minimum recommended wire size is 4xAWG #10, with the total length of wire less than 4 feet (2 feet OUTPUT, 2 feet return).

Top - 3D Layout mode

Figure 1: Top layer reference designators

Bottom - 3D Layout mode

Figure 2: Bottom layer reference designators

- EQUIPMENT SETUP

  - Input Connections:

- Prior to connecting the DC input source VIN, it is advisable to limit the source current to 20A maximum.  Make sure VIN is initially set to 0V and connected the DC input source to T1, T2 as shown in Figure 1. (Make sure to torque the screws of T1, T2 snugs with a flat-head screw driver.)

- Make sure Jumpers J2, J3, J5 connected.

- 1-2 pins short of Jumper J10,J11,J12,J13.

  - Output_A Connections:

Connect the load1 to T3, T4, T7 and T8 and set the load to a constant current mode to sink 0Adc before VIN is applied. (Make sure to torque the screws of T3, T4, T7, T8 snugs with a flat-head screw driver.)

  - See Figure 1 for connection.

- PROCEDURE

  - Line/Load regulation

  - Connect voltmeter #1 at TP16 (VIN) and TP29 (GND) to measure the VIN.

  - Connect voltmeter #2 at TP19 (VOUTA) and TP17 (GND) to measure the VOUTA.

  - Make sure the Jumper connections:

J2, J3, J5 – shorted.

J10, J11, J12, J13 – pins1-2 shorted

  - Ensure load is set to constant current mode and to sink 0Adc.

  - Turn ON and then increase VIN from 0V to 12V. Use voltmeter #1 to measure VIN.

  - Switch S1 to ‘ON’ position.

  - Use voltmeter #2 to measure VOUTA (See table 1 for values).

  - Turn ON and then vary the load1 according to Table 1 below; VOUTA should be remaining in load regulation.

  - Vary VIN from 10V to 14V; VOUTA should remain in line regulation.

  - Verify VOUTA according to the table 1 below.

  - Change the load to sink 0Adc on VOUTA.

  - Switch S1 to ‘OFF’ position.

  - Decrease VIN to 0V.

Example: Table 1

| Step | Vin | Load | Vout1 0.88V |
| --- | --- | --- | --- |
| 1 | 10Vdc | 0 | 0.875-0.885 |
| 2 | 10Vdc | 40A | 0.875-0.885 |
| 3 | 12VDC | 0 | 0.875-0.885 |
| 4 | 12VDC | 40A | 0.875-0.885 |
| 5 | 14VDC | 0 | 0.875-0.885 |
| 6 | 14VDC | 40A | 0.875-0.885 |

  - The last step of this procedure should say to loop back to 7.1 and continue until all units have been tested.

  - If jumpers are used always specify final jumper settings for shipment.

- EQUIPMENT SHUTDOWN

Once all the EVM are tested, please turn OFF output loads first, then turn OFF VIN power source (12V source).

- MATERIAL DISPOSITION & TRANSFER

  - CONFORMING MATERIAL

On the ESD bag, in text clearly visible and readable to the naked eye, add

For evaluation only; not FCC approved for resale.

This can be included on the label below or added as a separate label.

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled per table below and shipped per the P.O.

| Label 1 Assembly Number+Dash Number (if Applicable) | Label 2 IC Number |
| --- | --- |
| BMC051 | TPS536C7 |

  - NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches, or exceeds 5%, contact EVM coordinator for assistance.

Change History:

| Date | Changes | Reason | By |
| --- | --- | --- | --- |
| 6/24/2020 | Original |  |  |