Choosing of FET:

- Determine the maximum power dissipation, PD, in the transistor:

Rsense=  (voltage across RSNS,Vcs)/Ireg=125mV/600mA=0.2 Ohm

PD = (VI − VCS – V (BAT)) IREG

=(12-0.125-5.6)0.6=3.825 W

- Calculate the total theta (Junction to ambient thermal resistance) (C/W), needed:= (Tjmax-Tamax)/Pd=(150-40) /3.825=28 deg/W

Now choose PNP package with a theta at least 10% below this value to account for additional thetas other than the device i.e. a package, for instance, has typically a theta of 25C/W is needed.

Choosing of Board Size:

Minimum board size required :

Board Are in this case= (2.37 X 3.825) = 9.06 in2