# Texas Instruments

EVM Test Procedure

# HVP006 (LMG1020EVM) Test Procedure

# Rev. 1

6Nov2017

- GENERAL

  - PURPOSE

    - To provide detailed instructions for testing LMG1020 EVM Modules

  - SCOPE

    - Covers complete instructions for testing LMG1020EVM-006

  - REFERENCE DOCUMENTATION

    - Schematic HVP006A.Sch Doc

    - Assembly DWG HVP006A-Assy.pdf

    - Users Guide

Note: Please do not put references to revision letters of documents in the test procedure. This will only cause extra work when docs change.

  - MATERIALS

    - Test Log attached to end of this procedure.

  - DEFINITIONS

Add definitions of terms as required

    - DMM is reference to Digital Multimeters

    - LED Load is reference to boards with mounted LEDs

    - UUT is reference to Unit under Test

    - EVM is reference to Evaluation Module assembly, in this case the UUT

- SAFETY

  - Safety Glasses are to be worn.

  - This test must be performed by qualified personnel trained in electronics theory and understand the risks and hazards of the assembly to be tested.

  - ESD precautions must be followed while handling electronic assemblies while performing this test.

  - Precautions should be observed to avoid touching areas of the assembly that may get hot or present a shock hazard during testing.

- QUALITY

  - Test data or reports shall be made available upon request by Texas Instruments.

- APPAREL

  - Electrostatic smock

  - Electrostatic Gloves or finger cots

  - Safety Glasses

  - Ground ESD wrist strap

- EQUIPMENT

1xOscilloscope 1MHz Bandwidth minimum

2x 1MHz probes for the oscilloscope

1x dual power supply capable of 30V at minimum (plus 4 cables/wires)

1x function generator capable of 20ns pulse or less  (plus coaxial cable with spring/alligator connectors).

- EQUIPMENT SETUP

  - Test Equipment

##### DC Voltage Source: capable of supplying the input of the EVM up to 30 Vdc as desired.  Capable of supplying 100mA and supports current limiting.

##### DC Bias Source: capable of 6-18 Vdc output at up to 100mA.

##### Oscilloscope: capable of at least 1GHz operation, using oscilloscope probes with a “pigtail” spring ground clip instead of the standard alligator clip.

Recommended use for ground spring clip test points

Set the scope to trigger on Channel 1,

##### Function Generator: single output capable of at least 0-3Vdc pulse signal signal (operating maximum digital input is 5Vdc),1MHz frequency or higher, 50ns minimum pulse or smaller.

- Recommended Test Setup

  - Connect the input and bias supplies and DC electronic load as indicated in Recommended Connection Points and feature descriptionfigure below.

  - Connect the Vbus power supply to the J2 terminal (positive closest to test point TP5), then connect the Vcc power supply to J1 terminal (positive closest to test point TP1).

Recommended Connection Points and feature description

Connect oscilloscope Ch1 probe to the test point marked Vg

Connect oscilloscope Ch2 probe to the test point marked K

- PROCEDURE

  - Power up the Vcc supply (J1) to 7V ( 15V max) and setting the current limit to 0.1A.

  - Set the oscilloscope trigger on channel1:

- Rising input

- 2.5V

- Set the timescale to 20ns/div

- Set the voltage scale to 2V/div

##### Enable the function generator output  with the following settings:

- PWM (or pulse) mode

- 100kHz of frequency

- Pulse width of 100ns

- Rise/fall time of 10ns

- High voltage  of 3V, low voltage of 0V

##### Set the current limit to 0.2A and power up the input supply(J2) and set to 20V.

##### Observe the oscilloscope triggering on a Vg pulse on Ch1 with positive width of 10-20ns. (pass/fail)

##### Observe the oscilloscope  Ch2 the K pulse with  maximum voltage of 20V and negative width of 10-50ns. (pass/fail)

##### Shutdown Procedure

##### Once the desired measurements have been completed, shut down the EVM by following these steps:

##### Turn off the Vbus power supply(J2)

##### Disable the function generator

##### Disable the Vcc supply(J1)

  - loop back to 7.1 and continue until all units have been tested.

.

  - If trouble shooting or repair instructions are going to be included as part of the procedure it should have its own section at the end (Appendix) of the procedure and should not be included in the middle of the standard test procedure. Preferably a word on the step that the unit failed at such as where to look for the problem would be of benefit.

- MATERIAL DISPOSITION & TRANSFER

  - CONFORMING MATERIAL

On the ESD bag, in text clearly visible and readable to the naked eye, add

For evaluation only; not FCC approved for resale.

This can be included on the label below or added as a separate label.

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled per table below and shipped per the P.O.

| Label 1 Assembly Number+Dash Number (if Applicable) | Label 2 EVM orderable number |
| --- | --- |
| HVP006 | LMG1020EVM-006 |

  - NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches, or exceeds 5%, contact  EVM coordinator for assistance.

Change History:

| Date | Changes | Reason | By |
| --- | --- | --- | --- |
| 4/9/2015 | Original |  |  |