# Texas Instruments

# LMG3100EVM-089 Test Procedure

Date: 1-Dec -2023

1.0 GENERAL

1.1 PURPOSE

This document describes a step by step process of testing the LMG3100R017 EVM, medium voltage open loop half bridge module that is configured as either buck model or boost mode.

1.2 SCOPE

This document covers the complete instructions for testing the LMG3100R017 EVM. Please read through this entire document before testing. Rigorously follow instructions.

2.0 SAFETY

- Safety Glasses are to be worn.

- This test must be performed by qualified personnel trained in electronics theory and understand the risks and hazards of the assembly, and also familiar working with high voltage up to 80VDC to be tested.

- ESD precautions must be followed while handling electronic assemblies while performing this test.

- Precautions should be observed to avoid touching areas of the assembly that may get hot or present a shock hazard during testing.

- Board must be placed in a closed plastic safety enclosure when voltages above 50V are applied to the board.

3.0 QUALITY

- Test data or reports shall be made available upon request by Texas Instruments.

4.0 APPAREL

- Electrostatic smock

- Electrostatic Gloves or finger cots

- Safety Glasses

- Ground ESD wrist strap

- IMPORTANT: ESD gear should be used with caution when working around HVDC due to earth ESD grounding creating a current path and shock hazard when HVDC is on.

5.0 EQUIPMENT

Ensure that the following equipment is available for use.

- Power supply, capable of output voltage 6.0V and up to 0.5A

- Power supply, capable of output voltage 60V and up to 1A

- DMM, accurately measuring voltages 0V – 10V or beyond and resistance from 0.15Ω to above 500kΩ

- 2 pairs of probes for the power supply and 1 probe pair for DMM use

- Function generator capable of output square waveforms with 5V, 50% duty cycle at 200kHz

- High voltage safety enclosure provided by Texas Instruments

6.0 EQUIPMENT SETUP STANDALONE

- Set the power supply to 6.0V and the current limit to 0.5A.

- Connect positive lead of 6V (off) power supply to EXTVCC (pin 1) of J3 and negative lead to AGND (pin 2) of J3 on the left side of the board.

Figure 1: LMG3100EVM-089 design 3D picture

Figure 2: LMG3100EVM-089 design J3 pin illustration

7.0 PROCEDURE STANDALONE

The test procedure flow is given in Figure 3, and the details are explained within each step.

LED ON????

VCC power supply test

See Section 9.0

See Section 7.1

Figure 3: Test procedure flow.

    - Test Power Board

      - Enable 6V power supply on EXTVCC and AGND (pin 1 to pin 2 of J3).

      - Current drawn by the power supply has to be less than 100mA.

      - Green LED D1 should glow if the board is powered up with the VCC_5V.

- EQUIPMENT SETUP FOR TESTING IN BUCK MODE

- Connect the 6V power supply as instructed in the previous section.

- Set the function generator to output 200kHz, 50% duty cycle, 5V (low 0V, high 5V) continuous pulse.

- Connect the function generator output to LMG3100EVM function generator PWM input at J5 shown in figure 4. PWM signal is given at pin no. 4 and GND connected to any of the first three pins.

- Connect the 60VDC 1A power supply to LMG3100EVM’s connector J1 located at the top right of the board. 60V high voltage input for buck mode (High Voltage) shown in figure 5.

Figure 4: Connector J5 for connecting PWM signals.

Figure 5: Connection for 60V Vin to the LMG3100EVM board at J1

+Vout

-Vout

-Vin

+Vin

+6V

GND

PWM Gnd Gnd Gnd

+Vout

Figure 7: Connector details on the LMG3100EVM-089 (LEDs marked in yellow)

9.0 TEST PROCEDURE

    - Buck Mode:

      - Turn on +6VDC to the LMG3100EVM. Watch power supply for DC current to settle down after approx. 3 to 5 seconds.

      - Make sure 5V LED is on

      - Turn on the function generator to output the continuous pulse.

      - Set a 60VDC power supply current limit to 1.0A.

      - Enable the 60V voltage supply, and make sure to ramp up gradually from 0V to 60V. As the voltage is ramping, the Vin LED (D2) will turn on and become brighter. The ramping time is suggested 30 secs.

      - Run the test for 2 mins

      - Make sure the current drawn from the power supply is not exceeding 0.2A

      - IMMEDIATELY TURN OFF 60VDC POWER SUPPLY

      - Wait until High Voltage On LED in figure 7 turns off.

      - Turn off the function generator.

      - Turn off the +6VDC power supply.

Note:

a. During high voltage on, the 60V power supply runs with low current <100mA. If the current keep increasing, the test fails.

11.0 MATERIAL DISPOSITION & TRANSFER

11.1	CONFORMING MATERIAL

On the ESD bag in text visible and readable to the naked eye, add

For evaluation only; not FCC approved for resale.

This can be included on the label below or added as a separate label

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled with two labels according to the table below, and shipped per the P.O.

| Label 1 Assembly Number+Dash Number | Label 2 IC Number |
| --- | --- |
| HVP089 | LMG3100R017 |

11.2	NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. Contact the EVM coordinator for assistance if yield loss exceeds 2%.