| Kernel Driver |  |  |  |
| --- | --- | --- | --- |
| (Replace Linux native kernel driver with the newer version) platform/broadcom/sonic-platform-modules-accton/stn-a2/modules/lm25066.c |  |  |  |
| Create Device Instance |  |  |  |
| echo lm5066i 0x58 > /sys/bus/i2c/devices/i2c-20/new_device echo lm5066i 0x58 > /sys/bus/i2c/devices/i2c-21/new_device |  |  |  |
| Path |  | Access | Description |
|  |  |  | Value |
| /sys/class/hwmon/hwmon[8-9]/ |  |  |  |
|  | curr1_average | ro | Average measured input current |
|  |  |  | 0h read_avg_iin |
|  | curr1_input | ro | Measured input current |
|  |  |  | 0h mfr_read_iin |
|  | curr1_label | ro | "iin" |
|  | curr1_max | r/w | Maximum input current |
|  | curr1_max_alarm | r/w | Input current high alarm |
|  |  |  | Default: N/A |
|  | in1_average | ro | Average measured input voltage |
|  |  |  | 0h read_avg_vin |
|  | in1_input | ro | Measured input voltage |
|  |  |  | 0h read_vin |
|  | in1_label | ro | "vin" |
|  |  |  | 10V to 80V |
|  | in1_max | r/w | Maximum input voltage |
|  | in1_max_alarm | r/w | Input voltage high alarm |
|  | in1_min | r/w | Minimum input voltage |
|  | in1_min_alarm | r/w | Input voltage low alarm |
|  | in2_input | ro | Measured voltage on VAUX pin |
|  |  |  | 0h read_vaux |
|  | in2_label | ro | "vmon" |
|  | in3_average | ro | Average measured output voltage |
|  |  |  | 0h read_avg_vout |
|  | in3_input | ro | Measured output voltage |
|  |  |  | 0h read_vout |
|  | in3_label | ro | "vout1" |
|  |  |  | 10V to 80V |
|  | in3_min | r/w | Minimum output voltage |
|  | in3_min_alarm | r/w | Output voltage low alarm |
|  | power1_alarm | r/w | Input power alarm |
|  |  |  | Default: N/A |
|  | power1_average | ro | Average measured input power |
|  |  |  | 0h read_avg_pin |
|  | power1_input | ro | Measured input power |
|  |  |  | 0h read_pin |
|  | power1_input_highest | ro | Historical maximum power |
|  |  |  | 0h read_pin_peak |
|  | power1_label | ro | "pin" |
|  | power1_max | r/w | Maximum input power limit |
|  | power1_reset_history | wo | Write any value to reset maximum power history |
|  |  |  | N/A |
|  | temp1_crit | r/w | Critical high temperature |
|  | temp1_crit_alarm | r/w | Chip temperature critical high alarm |
|  | temp1_input | ro | Measured temperature |
|  |  |  | -40o C to 125o C, 190h read_temperature_1 |
|  | temp1_max | r/w | Maximum temperature |
|  | temp1_max_alarm | r/w | Chip temperature high alarm |

root@sonic:/sys/bus/i2c/devices/20-0058# ls driver hwmon modalias name power subsystem uevent root@sonic:/sys/bus/i2c/devices/20-0058# cat name lm5066i root@sonic:/sys/bus/i2c/devices/20-0058# cd hwmon root@sonic:/sys/bus/i2c/devices/20-0058/hwmon# ls hwmon8 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon# cd hwmon8 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# ls curr1_average in1_max in3_min power1_max curr1_input in1_max_alarm in3_min_alarm power1_reset_history curr1_label in1_min name subsystem curr1_max in1_min_alarm power temp1_crit curr1_max_alarm in2_input power1_alarm temp1_crit_alarm device in2_label power1_average temp1_input in1_average in3_average power1_input temp1_max in1_input in3_input power1_input_highest temp1_max_alarm in1_label in3_label power1_label uevent root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat curr1_average 464 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat curr1_input 497 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat curr1_label iin root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat curr1_max 1724 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat curr1_max_alarm 0 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat in1_average 4232 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat in1_input 4232 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat in1_label vin root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat in1_max 5553 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat in1_max_alarm 0 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat in1_min 30 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat in1_min_alarm 0 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat name lm5066i root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat power1_alarm 0 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat power1_average 27042915 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat power1_input 27630805 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat power1_input_highest 57613168 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat power1_label pin root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat power1_max 152263374 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat power1_reset_history 2351557 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat temp1_crit 15937 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat temp1_crit_alarm 0 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat temp1_input 9312 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat temp1_max 15937 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cat temp1_max_alarm 0 root@sonic:/sys/bus/i2c/devices/20-0058/hwmon/hwmon8# cd ../../../21-0058 root@sonic:/sys/bus/i2c/devices/21-0058# cat name lm5066i root@sonic:/sys/bus/i2c/devices/21-0058# cd hwmon root@sonic:/sys/bus/i2c/devices/21-0058/hwmon# ls hwmon9 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon# cd hwmon9 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# ls curr1_average in1_max in3_min power1_max curr1_input in1_max_alarm in3_min_alarm power1_reset_history curr1_label in1_min name subsystem curr1_max in1_min_alarm power temp1_crit curr1_max_alarm in2_input power1_alarm temp1_crit_alarm device in2_label power1_average temp1_input in1_average in3_average power1_input temp1_max in1_input in3_input power1_input_highest temp1_max_alarm in1_label in3_label power1_label uevent root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat curr1_average 1306 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat curr1_input 1340 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat curr1_label iin root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat curr1_max 1724 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat curr1_max_alarm 0 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in1_average 4210 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in1_input 4210 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in1_label vin root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in1_max 5553 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in1_max_alarm 0 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in1_min 30 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in1_min_alarm 0 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in2_input 896 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in2_label vmon root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in3_average 4106 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in3_input 4106 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in3_label vout1 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in3_min -108 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat in3_min_alarm 0 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat name lm5066i root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat power1_alarm 0 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat power1_average 71134626 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat power1_input 71134626 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat power1_input_highest 115814226 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat power1_label pin root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat power1_max 152263374 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat power1_reset_history 2351557 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat temp1_crit 15937 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat temp1_crit_alarm 0 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat temp1_input 10625 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat temp1_max 15937 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cat temp1_max_alarm 0 root@sonic:/sys/bus/i2c/devices/21-0058/hwmon/hwmon9# cd /sys/bus/i2c/devices/ root@sonic:/sys/bus/i2c/devices# root@sonic:/sys/bus/i2c/devices# cd 19-0034 root@sonic:/sys/bus/i2c/devices/19-0034# ls driver hwmon modalias name power subsystem uevent root@sonic:/sys/bus/i2c/devices/19-0034# cat name ucd90160a root@sonic:/sys/bus/i2c/devices/19-0034# cd hwmon root@sonic:/sys/bus/i2c/devices/19-0034/hwmon# ls hwmon10 root@sonic:/sys/bus/i2c/devices/19-0034/hwmon# cd hwmon10 root@sonic:/sys/bus/i2c/devices/19-0034/hwmon/hwmon10# ls device in14_crit_alarm in2_label in6_lcrit_alarm in10_crit in14_input in2_lcrit in6_max in10_crit_alarm in14_label in2_lcrit_alarm in6_max_alarm in10_input in14_lcrit in2_max in6_min in10_label in14_lcrit_alarm in2_max_alarm in6_min_alarm in10_lcrit in14_max in2_min in7_crit in10_lcrit_alarm in14_max_alarm in2_min_alarm in7_crit_alarm in10_max in14_min in3_crit in7_input in10_max_alarm in14_min_alarm in3_crit_alarm in7_label in10_min in15_crit in3_input in7_lcrit in10_min_alarm in15_crit_alarm in3_label in7_lcrit_alarm in11_crit in15_input in3_lcrit in7_max in11_crit_alarm in15_label in3_lcrit_alarm in7_max_alarm in11_input in15_lcrit in3_max in7_min in11_label in15_lcrit_alarm in3_max_alarm in7_min_alarm in11_lcrit in15_max in3_min in8_crit in11_lcrit_alarm in15_max_alarm in3_min_alarm in8_crit_alarm in11_max in15_min in4_crit in8_input in11_max_alarm in15_min_alarm in4_crit_alarm in8_label in11_min in16_crit in4_input in8_lcrit in11_min_alarm in16_crit_alarm in4_label in8_lcrit_alarm in12_crit in16_input in4_lcrit in8_max in12_crit_alarm in16_label in4_lcrit_alarm in8_max_alarm in12_input in16_lcrit in4_max in8_min in12_label in16_lcrit_alarm in4_max_alarm in8_min_alarm in12_lcrit in16_max in4_min in9_crit in12_lcrit_alarm in16_max_alarm in4_min_alarm in9_crit_alarm in12_max in16_min in5_crit in9_input in12_max_alarm in16_min_alarm in5_crit_alarm in9_label in12_min in1_crit in5_input in9_lcrit in12_min_alarm in1_crit_alarm in5_label in9_lcrit_alarm in13_crit in1_input in5_lcrit in9_max in13_crit_alarm in1_label in5_lcrit_alarm in9_max_alarm in13_input in1_lcrit in5_max in9_min in13_label in1_lcrit_alarm in5_max_alarm in9_min_alarm in13_lcrit in1_max in5_min name in13_lcrit_alarm in1_max_alarm in5_min_alarm power in13_max in1_min in6_crit subsystem in13_max_alarm in1_min_alarm in6_crit_alarm temp1_alarm in13_min in2_crit in6_input temp1_input in13_min_alarm in2_crit_alarm in6_label uevent in14_crit in2_input in6_lcrit root@sonic:/sys/bus/i2c/devices/19-0034/hwmon/hwmon10# cat in4_max 5500 root@sonic:/sys/bus/i2c/devices/19-0034/hwmon/hwmon10#