After enable the system watchdog, and add the WDT event to PMON_PORST_N, then the WDT time out will triger PMON_PORST_N as what we expected. See the configuration below. The signal PMON_PORST_N is a power on reset for the whole system, the system could not bootup with max reset period 32,256ms that UCD90106A allows. So the system software is keeping under reset. Software like have 300s before issue SYSTEM_WATCHDOG_RESET command.

What’s the possible solution? I’ve tried some experiments to use system reset, but it doesn’t work.

This is the first experiments: Enable Watchdog Timeout and add timeout event to PMON_PORST_N( the board system power on reset)

1. Enable the system watchdog

2. Add WDT time out event to PMON_PORST_N

IT works for the first time bootup because there 300s Start Time, but when timeout happens, software system can’t bootup within 32,256ms, then system will keep reboot.

Second expriment is to try using system reset to have 300s “Start Time” for timeout reset, but it failed.

1. Enable systen reset, assert the reset when watchdog time out

2. Then check “Watch reset pin”, per the description, when system reset is de-asserted, sytem watchdog function will wait the Start Time before monitor the input pin again. I expect it will have give system software 300s bootup time again. But PMON_PORST_N will keep low until SYSTEM_WATCHDOG_RESET command from “TI Fusion Digital Power Designer”.

I’ve captured the System Reset Pin, Pin 20, it asserted 960ms then de-assert as configuration, but PMON_PORST_N be kept low. PMON_PORST_N is only de-asserted after apply SYSTEM_WATCHDOG_RESET command from “TI Fusion Digital Power Designer”