# Texas Instruments

Analog EVM Test Procedure

# SR147 Test Procedure

LMR60460QEVM

# Rev. A

06/03/2024

1.0 GENERAL

  - PURPOSE

    - To provide detailed instructions for testing LMR60460QEVM.

  - SCOPE

1.2.1 	Covers complete instructions for testing LMR60460QEVM.

  - REFERENCE DOCUMENTATION

    - Schematic; SR147_A_sch

    - Assembly Drawing; SR147_A_AssemblyRelease

1.4 MATERIALS

1.4.1 Test Log attached to end of this procedure.

1.5 DEFINITIONS

    - DMM is reference to Digital Multimeters.

    - UUT is reference to Unit Under Test.

    - EVM is reference to Evaluation Module assembly, in this case the UUT.

2.0 SAFETY

2.1 Safety Glasses are to be worn.

2.2 This test must be performed by qualified personnel trained in electronics theory and understand the risks and hazards of the assembly to be tested.

2.3 ESD precautions must be followed while handling electronic assemblies while performing this test.

2.4 Precautions should be observed to avoid touching areas of the assembly that may get hot or present a shock hazard during testing.

3.0 QUALITY

3.1 Test data or reports shall be made available upon request by Texas Instruments.

4.0 APPAREL

4.1 Electrostatic smock

4.2 Electrostatic Gloves or finger cots

4.3 Safety Glasses

4.4 Ground ESD wrist strap.

5.0 EQUIPMENT

  - Specify all equipment needed to perform tests.

5.1.1       Power Supplies:

    - 50V, 4A DC Power Supply

5.1.2      Loads:

    - 12A capable Electronic or Resistive Load with a greater than 5V capability

    - Meters:

    - Two DMM’s for VIN and VOUT

    - Test Leads and other cabling:

Input and output terminal blocks can accept up to 20-AWG wire.

6.0 EQUIPMENT SETUP

6.1 Connection diagram

6.2 Board Photo

- PROCEDURE

  - Connect the input power supply as below:

    - Positive terminal to TP2 labeled VIN_EMI+.

    - Negative terminal to TP1 labeled VIN_EMI–.

  - Connect the load as below:

    - Positive terminal to TP11 labeled VOUT+.

    - Negative terminal to TP10 labeled VOUT–.

  - Connect the DMMs as below:

    - Connect one DMM positive terminal to TP2 labeled VIN_EMI+ and the negative terminal to TP1 labeled VIN_EMI–.

    - Connect one DMM positive terminal to TP11 labeled VOUT+ and the negative terminal to TP10 labeled VOUT–.

  - Set the Input voltage and load current on the equipment.

  - Set the current on the input supply to 6A.

  - Set the Electronic Load to Constant Current (CC) mode.

  - Default switching frequency is 400kHz, no modifications required.

  - Turn on the input voltage.

  - Enable the load current.

  - Measure the output voltage per Table 1 below.

  - Repeat for each test #.

- Table 1 LMR60460QEVM tests

| # | Test Description | VIN | VOUT | FSW | IOUT | VOUT |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Min | Max |
| 1 | VIN = 12V, VOUT = 5V, FSW = 400kHz, IOUT = 6A | 12V | 5V | 400kHz | 6A | 4.9V | 5.1V |
| 2 | VIN = 24V, VOUT = 5V, FSW = 400kHz, IOUT = 6A | 24V | 5V | 400kHz | 6A | 4.9V | 5.1V |
| 3 | VIN = 12V, VOUT = 5V, FSW = 400kHz, IOUT = 3A | 12V | 5V | 400kHz | 3A | 4.9V | 5.1V |
| 4 | VIN = 24V, VOUT = 5V, FSW = 400kHz, IOUT = 3A | 24V | 5V | 400kHz | 3A | 4.9V | 5.1V |

- EQUIPMENT SHUTDOWN

  - Turn off power supply.

  - Unplug all the cables going into device.

9.0 MATERIAL DISPOSITION & TRANSFER

## 9.1 CONFORMING MATERIAL

On the ESD bag, in text clearly visible and readable to the naked eye, add:

For evaluation only; not FCC approved for resale.

This can be included on the label below or added as a separate label.

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled according to the table below, and shipped per the P.O.

| Label 1 Assembly Number + Dash Number if Applicable | Label 2 EVM Orderable Number |
| --- | --- |
| SR147 | LMR60460QEVM |

## 9.2 NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches or exceeds 5%, contact EVM coordinator for assistance.

Change History:

| Date | Changes | Reason | By |
| --- | --- | --- | --- |
| 06/03/2025 | Original |  | Rahil Ajani |