測試日期(Test Date) 2020.01.17  Inductor 22uH 5A

CH1 Panel Power     CH2 Panel Current   CH3 TPS61199 PWM

| 電解(E-Cap) 22uF MLCC 10uFx2 電壓漣波小(small voltage ripple) 電流漣波小(low current ripple) PWM 波形正常(wave form normal (無高頻音 no audio noise) |  |
| --- | --- |
| 電解(E-CAP)10uF MLCC 10uFx2 電壓漣波略大(Voltage ripple slightly larger) 電流漣波略大(Current ripple slightly larger) PWM 波形正常 (無高頻音) PWM waveform is normal |  |
| 電解NA(no E-CAP) MLCC 10uFx2 電壓漣波大 電流漣波大 PWM 波形異常 高頻音嚴重 (Large voltage ripple, Large current ripple, PWM waveform abnormal, High frequency sound) |  |
| 電解NA(No E-CAP) MLCC 10uFx7 電壓漣波大 電流漣波大 PWM 波形異常 有高頻音 (Large voltage ripple, Large current ripple, PWM waveform abnormal, High frequency sound) |  |

測試日期 2020.01.17  Inductor 10uH 3.8A

CH1 Panel Power     CH2 Panel Current   CH3 TPS61199 PWM

| 電解(E-CAP)10uF MLCC N/A 電壓漣波略大 電流漣波略大 PWM 波形正常 (無高頻音) Voltage ripple is slightly larger, Current ripple is slightly larger, PWM waveform is normal (no high frequency sound) |  |
| --- | --- |
| 電解(E-CAP) 10uF MLCC 10uFx1 電壓漣波略大 電流漣波小 PWM 波形正常 (無高頻音) Slightly larger voltage ripple, small current ripple, PWM waveform is normal (no high frequency sound) |  |
| 電解(E-CAP) 10uF MLCC 10uFx2  與22uH 電解10uF MLCC 10uFx2 比對已有明顯不同 (Compared with 22uH and output E-CAP 10uF+ MLCC 10uFx2) 電壓漣波小 電流漣波小 PWM 波形正常 (無高頻音) Low voltage ripple, small current ripple, PWM waveform is normal (no high frequency sound) |  |

測試日期 2020.01.14  Inductor 10uH 3.8A

CH1 Panel Power     CH2 Panel Current   CH3 TPS61199 PWM

| 電解(E-CAP) 22uF MLCC 10uFx2 電壓漣波小 電流漣波小 PWM 波形正常 (Low voltage ripple, small current ripple, PWM waveform is normal) |  |
| --- | --- |
| 電解 (E-CAP)22uF MLCC NA 電壓漣波小 電流漣波略大 PWM 波形正常 (Small voltage ripple, slightly larger current ripple, PWM waveform is normal) |  |
| 電解(E-CAP) 22uFx2 MLCC NA 電壓漣波小 電流漣波略大 PWM 波形正常 (Small voltage ripple, slightly larger current ripple, PWM waveform is normal) |  |
| 電解NA(No E-CAP) MLCC 10uFx2 電壓漣波大 電流漣波大 PWM 波形異常 (arge voltage ripple Large current ripple PWM waveform abnormal) |  |