Texas Instruments USB Serial Adapter API & Tools README

This document describes how to use TI’s Application Programming Interface (API) to programmatically control the TI USB Serial Adapter (USB-TO-GPIO, SLLU093). It also describes some simple tools that are included with the adapter.

Last Updated 15 May 2012

Michael S Muegel

- IMPORTANT NOTICE

Texas Instruments and its subsidiaries (TI) reserve the right to make changes to their products or to discontinue any product or service without notice, and advise customers to obtain the latest version of relevant information to verify, before placing orders, that information being relied on is current and complete. All products are sold subject to the terms and conditions of sale supplied at the time of order acknowledgment, including those pertaining to warranty, patent infringement, and limitation of liability.

TI warrants performance of its products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are utilized to the extent TI deems necessary to support this warranty. Specific testing of all parameters of each device is not necessarily performed, except those mandated by government requirements.

Customers are responsible for their applications using TI components.

In order to minimize risks associated with the customer’s applications, adequate design and operating safeguards must be provided by the customer to minimize inherent or procedural hazards.

TI assumes no liability for applications assistance or customer product design. TI does not warrant or represent that any license, either express or implied, is granted under any patent right, copyright, mask work right, or other intellectual property right of TI covering or relating to any combination, machine, or process in which such products or services might be or are used. TI’s publication of information regarding any third party’s products or services does not constitute TI’s approval, license, warranty or endorsement thereof.

Reproduction of information in TI data books or data sheets is permissible only if reproduction is without alteration and is accompanied by all associated warranties, conditions, limitations and notices.  Representation or reproduction of this information with alteration voids all warranties provided for an associated TI product or service, is an unfair and deceptive business practice, and TI is not responsible nor liable for any such use.

Resale of TI’s products or services with statements different from or beyond the parameters stated by TI for that product or service voids all express and any implied warranties for the associated TI product or service, is an unfair and deceptive business practice, and TI is not responsible nor liable for any such use.

- Also see: Standard Terms and Conditions of Sale for Semiconductor Products.  www.ti.com/sc/docs/stdterms.htm

Mailing Address:

Texas Instruments

Post Office Box 655303

Dallas, Texas 75265

- Copyright © 2007, Texas Instruments Incorporated

- Table of Contents

1	APIs & Code Samples	1

1.1	General API Usage	1

1.2	.NET API	1

1.3	COM API	2

1.4	Distributing Your COM Application	2

1.5	VB6 Sample	2

1.6	Using the API in VB6	3

1.7	Using the API in MATLAB	4

2	Tools	4

3	Release Notes	5

- APIs & Code Samples

There are two Application Programming Interfaces (APIs) available:

- A native Microsoft.NET API for .NET version 2.0. The TIBusAdapters class in the TIDP.SAA namespace is the primary interface to this API.

- A COM API for use by languages and applications that do not support .NET. The TIBusAdapters class is also used in the COM API. The namespace is TIDP.SAA or TIDP_SAA, depending on your development environment.

The TI USB Adapter API Help.chm help file contains documentation for each. It can be launched from the Start Menu:

Find the TIBusAdapters class.

## General API Usage

To document:

- Multiple adapters

- Discovery

- Events

- Return classes and properties within them

- Exception mode

Point to CHM for the most part.

## .NET API

The assembly TIDP.SAA2.dll is automatically get installed to the Global Assembly Cache (GAC) when you run the API installer. In previous distributions of the SAA API there were other utility dlls included, and the main dll was called TIDP.SAA.dll. Starting in release 2.0 of the API installer, only a single dll is needed. Even though the primary namespace is “TIDP.SAA”, the assembly is named TIDP.SAA2.dll to avoid confusion with the old dlls.

To document:

- Create sample

- Redistributing your application (GAC not required)

## COM API

The TIDP.SAA.tlb COM type library gets installed into this folder:

C:\Program Files\Texas Instruments USB Serial Adapter API & Tools\Bin

The required updates to the Windows registry are automatically performed during installation.

## Distributing Your COM Application

When creating an installer or ZIP file for your application, you need to ensure that the following file is copied to the same folder as your executable:

- TIDP.SAA2.dll

You will find this file in the Bin folder. You do not need to copy this file to your application folder on your development PC where this API package was installed. You only need to do this on your “target” end-user PCs.

For example:

## Sample Source Code

C# and Visual Basic 6 (aka VB6) samples bundled with the API. Source code and pre-built binaries are included. The binaries are in the Bin folder:

Both samples do the same thing: provide a test that scans a Texas Instruments UCD device using the SMBus read block function on command code 0xFD. This is how TI UCD91xx, UCD92xx, and UCD90xxx devices identify themselves to our API/GUI. The samples also show how the CONTROL line on the adapter can be get/set.

The sample source code is in the Samples folder. The C# sample project file is in Visual Studio 2010 format, but C# Express 2010 could also be used to open it.

When you run one of the samples, it will check to see if at least one USB adapter is attached to the PC:

Once loaded, the sample GUI has a few test API calls you can make via buttons. For example:

## Using the API in VB6

Add a reference to the API type library:

Scroll the list until you find the USB adapter type library. Then check the “TI USB Serial Adapter API Type Library” and click OK:

This will make the TIDP_SAA2 COM library available through the object browser. Follow the VB6 sample code to create a TIBusAdapters object.

## Using the API in MATLAB

The MATLAB documentation has instructions on using a COM library. The installer handles.NET assembly installation and registry edit to make the USB adapter type library visible to MATLAB. The API does not have any UI components: it is an ActiveX server, not a control. MATLAB’s actxserver() function is used to create a TIBusAdapters object:

Adapters = actxserver('TIDP.SAA.TIBusAdapters') [may be TIDP_SAA]

You can now invoke Adapters.Discover() and adapter read/write functions. The MATLAB methods(), get(), and events() functions can be used to list out available methods and properties:

- Adapters.methods(‘-full’): returns a list of supported methods (aka functions)

- Adapters.get: returns a list of supported properties

- Adapters.events: returns a list of supported events

Newer versions of MATLAB also support using .NET libraries directly. This is the recommended approach because it avoids an unnecessary wrapper, the COM API, and should be easier to use and debug. See the MATLAB documentation on .NET support.

To Do: MATLAB sample, including use of enums and events. Refer to Samples/VB6 Client/Form.frm (text file) for example.

# Tools

To Do: include updated SAA GUI in the installation along with 1.0.10 SAA firmware.

# Release Notes

| Version 1.1 2009-04-26 | - Initial release. |
| --- | --- |
| Version 1.2 2009-05-01 | - Added additional functions to the TIBusAdapters class. This is a wrapper around functionality in the SMBusAdapters class, but may be easier to use because of how multiple adapters are addressed. |
| Version 1.3 2009-05-26 | - Calling .Equals() on a PMBusByte would result in stack overflow. Fixed. - If you had not previously called getControl(), getSignal(), getSMBusAlert() or equivalent, the first call to setControl() would be ignored. Fixed. |
| Version 1.8.6 2010-11-30 | - Switched to using Fusion Digital Power Designer version numbers. The API version number syncs with the API used in that version of the Fusion GUI. - Brand new USB-HID backend used which supports 64 bit Windows. |
| Version 1.8.34 2011-02-18 | - Installer was not installing .NET assemblies in GAC: fixed. - Fixed typos in this document. |
| Version 2.0.0 2011-06-08 | - Merged all assemblies into single assembly, TIDP.SAA2.dll. - Added Set_Pullups() methods to TIBusAdapters class. |
| Version 2.0.1 2012-02-03 | - Improved cross-process and single-process locking of SAA API. API is now fully thread safe (single process) and also can be used from multiple applications (cross-process). |
| Version 2.0.2 2012-02-10 | - Added GPIO_Read/Write() and Set_Pullups() to ITIBusAdapters interface. |
| Version 2.0.3 2012-03-23 | - Various GPIO_Write() functions would give an error in VB6. Fixed. |
| Version 2.0.4 2012-04-10 | - Removed custom System.Linq that provides LINQ for .NET 2.0: this is not needed in SAA API and causes problems when trying to use SAA API under .NET 3.5 and above. |
| Version 2.0.5 2012-04-16 | - If Discover() is called within a worker thread, the API disables adapter detach/re-attach by no longer listens for WM_DEVICECHANGE Win32 message. Why? If Discover() is called in a worker thread, the app will hang if a WM_SETTINGCHANGE message is received, which will be the case if the user changes the desktop image, among other things. The downside of this is that the API will no longer automatically handle the case where the SAA is detached and then re-attached. But you should really be calling Discover() from UI thread anyway, so this change is likely not to be noticed by many and will save a mysterious hang for anyone calling Discover() in a thread. |
| Version 2.0.6 2012-04-25 | - Added Receive_Byte() to TIBusAdapters. |
| Version 2.0.7 2012-05-15 | - TIBusAdapters.Set_Pullups() was not available via COM/ActiveX (VB6). Fixed. Example also added to VB6 sample form. - Added examples for changing I2C bus speed: |