All measurements taken with a 24V input, 360mA load, and 20MHz bandwidth unless otherwise noted.

Efficiency

| Iout | Vout | Iin | Vin | Eff |
| --- | --- | --- | --- | --- |
| 0.000 | 47.88 | 0.039 | 24.00 | 0.0% |
| 0.025 | 47.88 | 0.086 | 24.00 | 58.0% |
| 0.050 | 47.88 | 0.147 | 24.00 | 67.9% |
| 0.075 | 47.88 | 0.194 | 24.00 | 77.1% |
| 0.100 | 47.89 | 0.244 | 24.00 | 81.8% |
| 0.150 | 47.89 | 0.362 | 24.00 | 82.7% |
| 0.200 | 47.89 | 0.470 | 24.00 | 84.9% |
| 0.250 | 47.89 | 0.577 | 24.00 | 86.5% |
| 0.300 | 47.89 | 0.684 | 24.00 | 87.5% |
| 0.350 | 47.89 | 0.793 | 24.00 | 88.1% |
| 0.360 | 47.88 | 0.814 | 24.00 | 88.2% |

Ripple and Noise

Output Ripple					Input Ripple

50mV/div, 2usec/div				50mV/div, 2usec/div

Measured 67mV pk-pk:			Measured 62mV pk-pk:

Dynamic Loading

Load Step, 0A to 360mA			Load Step, 80mA to 360mA

500mV/div, 200mA/div			500mV/div, 200mA/div

5msec/div, 36mA/usec			5msec/div, 36mA/usec

Measured 1.52Vpp:				Measured 719mVpp:

Load Step, 180mA to 360mA

500mV/div, 200mA/div

5msec/div, 36mA/usec

Measured 430mVpp:

Turn On Response

360mA Load					0A Load

1msec/div, 10V/div:				1msec/div, 10V/div:

Stability

BW=5.5Khz	PM=67 degrees	GM=13dB

Thermal:

Top:

Bottom:

Photo:

Top:

Bottom: