tSCLK minimum period = 100 ns. The measured value is 500 ns.  OK

tSCLKH minimum high time = 50 ns. The measured value is 244 ns.  OK

tSCLKL minimum low time = 50 ns. The measured value is 136 ns.  OK

tH_SDI  Minimum, SDI input data hold time = 25 ns. The signal’s logic level is retained for more than 25 ns.  OK

tSU_SDI Minimum SDI input data setup time = 25 ns. The signal’s logic level is retained for more than 25 ns.  OK

tD_SDO Maximum SDO output data delay time = 30 ns. The measured value is 18 ns.  OK

tH_nSCS Minimum nSCS input hold time = 25 ns. The measured value is 1100 ns.  OK

tSU_nSCS Minimum nSCS input setup time = 25 ns. The measured value is 1300 ns.  OK

tHI_nSCS Minimum nSCS high time = 450 ns. The measured nSCS high time is more than 450 ns.  OK

tEN_nSCS Maximum enable delay time = 50 ns. The measured value is 15 ns.  OK

tDIS_nSCS Maximum enable delay time = 50 ns. SDO remains active for more than 50 ns.  TBD if this is an issue

tDIS_nSCS  The measured disable delay is 2000 ns.  TBD if this is an issue