void SPI_Init(void)

{

/* SPI Ports Initialization * Port 3.0, 3.1, 3.2 is used for SIMO SOMI and SCLK respectively , Port 2.0 is Used for nSCS enable*/

P4SEL0 |= (BIT5 + BIT6 + BIT7);

P4DIR |= BIT4;

P4OUT |= BIT4;

// Set P2.3 to output direction for nSCS

UCB1CTLW0 |= UCSWRST;                       // **Put state machine in reset**

UCB1CTLW0 |= UCMST | UCSYNC | UCMSB;           // 3-pin, 8-bit SPI master

// Clock polarity high, MSB

UCB1CTLW0 |= UCSSEL_3;                         // MCLK

UCB1BRW = 10;                 // Master Clock divided by 10 used by USCI clk

//    UCB1BR1 = 0;                                  //

UCB1CTLW0 &= ~UCSWRST;                   // **Initialize USCI state machine**

DRV83xxSPISet(); // make nSCS pin of DRV83xx low to start communication with master SPI;

DRV83xxSPIReset(); // make nSCS pin of DRV83xx High to stop communication with master SPI;

SPIDelay();

}

#define SPI_DELAY 10

#define SPI_BUSY_FLAG 0x01

#define SPI_READ_COMMAND    0x80   // SPI read command

#include "global.h"

void SPIDelay()

{

volatile unsigned int Delay_Count;

for (Delay_Count = SPI_DELAY; Delay_Count > 0; Delay_Count--)

{

;                                        // Wait for slave to initialize

}

}

/*!

\fn static inline void DRV83xxSPIReset()

\brief Holds the  SPI slave in reset

*/

void DRV83xxSPIReset()

{

P4OUT |= BIT4;                                                      // make nSCS pin of DRV83xx High to stop communication with master SPI;

}

/*!

\fn static inline void DRV83xxSPIset()

\brief Releases the AFE SPI slave from reset, so that it can begin accepting characters

*/

void DRV83xxSPISet()

{

P4OUT &= ~BIT4;                             // make nSCS pin of DRV83xx low to start communication with master SPI;

}

void SPI_Write(unsigned short address, unsigned short data)

{

volatile unsigned char dataMSB, dataLSB;

address = ((address << 3) & 0x078);

dataLSB = (data & 0x00FF);

dataMSB = ((data >> 8) & 0x07) | address;

DRV83xxSPISet(); // make nSCS pin of DRV83xx low to start communication with master SPI;

while (!(UCB1IFG & UCTXIFG))

{

;                                     // USCI_A0 TX buffer ready?

}

UCB1TXBUF = dataMSB;                     // Transmit first Byte

while (!(UCB1IFG & UCTXIFG))

{

;                                     // USCI_A0 TX buffer ready?

}

UCB1TXBUF = dataLSB;                     // Transmit Second Byte

while (UCB1STATW & SPI_BUSY_FLAG)

{

;                                  // Wait till Transmission is complete

}

DRV83xxSPIReset(); // make nSCS pin of DRV83xx High to stop communication with master SPI;

SPIDelay();

}

unsigned int SPI_read(unsigned char regAddr)

{

unsigned int data;

// Enable SPI communication by setting appropriate GPIO pins

// ...

DRV83xxSPISet();

UCB1TXBUF = SPI_READ_COMMAND | (regAddr & 0x7F);   // Send SPI read command and register address

while (!(UCB1IFG & UCRXIFG));   // Wait for SPI receive buffer to be ready

data = UCB1RXBUF;   // Read received data

// Disable SPI communication by resetting appropriate GPIO pins

// ...

DRV83xxSPIReset(); // make nSCS pin of DRV83xx High to stop communication with master SPI;

SPIDelay();

return data;

}

unsigned short SPI_Read(unsigned char address)

{

volatile unsigned short dataMSB, dataLSB, data;

address = ((address << 3) & 0x078);

DRV83xxSPISet(); // make nSCS pin of DRV83xx low to start communication with master SPI;

while (!(UCB1IFG & UCTXIFG))

{

;                                    // USCI_A0 TX buffer ready?

}

UCB1TXBUF = address | BIT7; // Transmit the Address of the register to be read , Or with BIT7 to indicate read operation

while (UCB1STATW & SPI_BUSY_FLAG)

{

;                                  // Wait till Transmission is complete

}

dataMSB = UCB1RXBUF & 0x07;              // Recieve the First byte

UCB1TXBUF = address;                     // Transmit Second character

while (UCB1STATW & SPI_BUSY_FLAG)

{

;                                  // Wait till Transmission is complete

}

dataLSB = UCB1RXBUF & 0xFF;              // Recieve the Second byte

DRV83xxSPIReset(); // make nSCS pin of DRV83xx High to stop communication with master SPI;

SPIDelay();

data = ((dataMSB << 8) & 0x0700) | dataLSB;

return (data);

}