/* --COPYRIGHT--,BSD

* Copyright (c) 2015, Texas Instruments Incorporated

* All rights reserved.

*

* Redistribution and use in source and binary forms, with or without

* modification, are permitted provided that the following conditions

* are met:

*

* *    Redistributions of source code must retain the above copyright

*        notice, this list of conditions and the following disclaimer.

*

* *    Redistributions in binary form must reproduce the above copyright

*        notice, this list of conditions and the following disclaimer in the

*        documentation and/or other materials provided with the distribution.

*

* *    Neither the name of Texas Instruments Incorporated nor the names of

*        its contributors may be used to endorse or promote products derived

*        from this software without specific prior written permission.

*

* THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS "AS IS"

* AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO,

* THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

* PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT OWNER OR

* CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL,

* EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO,

* PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS;

* OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY,

* WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR

* OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE,

* EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

* --/COPYRIGHT--*/

//! \file      solutions/instaspin_foc/src/proj_lab011e.c

//! \brief Hall sensor start-up with transition to FAST for InstaSPIN-FOC

//!

//! (C) Copyright 2015, Texas Instruments, Inc.

//! \defgroup PROJ_LAB11e PROJ_LAB11e

//@{

//! \defgroup PROJ_LAB11e_OVERVIEW Project Overview

//!

//! Hall sensor start-up with transition to FAST for InstaSPIN-FOC

//!

//!

// **************************************************************************

// the includes

// system includes

#include <math.h>

#include "main.h"

#include "sw/drivers/drvic/drv8353/src/32b/f28x/f2802x/drv8353.h"

#ifdef FLASH

#pragma CODE_SECTION(mainISR,"ramfuncs");

#pragma CODE_SECTION(runSetTrigger,"ramfuncs");

#pragma CODE_SECTION(runFieldWeakening,"ramfuncs");

#pragma CODE_SECTION(runCurrentReconstruction,"ramfuncs");

#pragma CODE_SECTION(angleDelayComp,"ramfuncs");

#pragma CODE_SECTION(HALLBLDC_State_Check,"ramfuncs");

#pragma CODE_SECTION(HALLBLDC_Ctrl_PwmSet,"ramfuncs");

#pragma CODE_SECTION(HALLBLDC_Ctrl_Run,"ramfuncs");

#endif

// Include header files used in the main function

// **************************************************************************

// the defines

// **************************************************************************

// the globals

uint16_t gDeviceVariant = 0xFF;

CLARKE_Handle      clarkeHandle_I;                              //!< the handle for the current Clarke transform

CLARKE_Obj            clarke_I;                                          //!< the current Clarke transform object

CLARKE_Handle      clarkeHandle_V;                              //!< the handle for the voltage Clarke transform

CLARKE_Obj            clarke_V;                                          //!< the voltage Clarke transform object

EST_Handle            estHandle;                                        //!< the handle for the estimator

FW_Handle              fwHandle;

FW_Obj                    fw;

PID_Obj                  pid[4];                                              //!< three handles for PID controllers 0 - Speed, 1 - Id, 2 - Iq

PID_Handle            pidHandle[4];                                  //!< three objects for PID controllers 0 - Speed, 1 - Id, 2 - Iq

uint16_t                pidCntSpeed;                                    //!< count variable to decimate the execution of the speed PID controller

IPARK_Handle        iparkHandle;                                    //!< the handle for the inverse Park transform

IPARK_Obj              ipark;                                                //!< the inverse Park transform object

FILTER_FO_Handle    filterHandle[6];                        //!< the handles for the 3-current and 3-voltage filters for offset calculation

FILTER_FO_Obj          filter[6];                                    //!< the 3-current and 3-voltage filters for offset calculation

SVGENCURRENT_Obj          svgencurrent;

SVGENCURRENT_Handle    svgencurrentHandle;

SVGEN_Handle        svgenHandle;                                    //!< the handle for the space vector generator

SVGEN_Obj              svgen;                                                //!< the space vector generator object

TRAJ_Handle          trajHandle_Id;                                //!< the handle for the id reference trajectory

TRAJ_Obj                traj_Id;                                            //!< the id reference trajectory object

TRAJ_Handle          trajHandle_spd;                              //!< the handle for the speed reference trajectory

TRAJ_Obj                traj_spd;                                          //!< the speed reference trajectory object

#ifdef CSM_ENABLE

#pragma DATA_SECTION(halHandle,"rom_accessed_data");

#endif

HAL_Handle            halHandle;                                        //!< the handle for the hardware abstraction layer (HAL)

HAL_PwmData_t      gPwmData = {_IQ(0.0), _IQ(0.0), _IQ(0.0)};              //!< contains the three pwm values -1.0 - 0%, 1.0 - 100%

HAL_AdcData_t      gAdcData;                                          //!< contains three current values, three voltage values and one DC buss value

MATH_vec3              gOffsets_I_pu = {_IQ(0.0), _IQ(0.0), _IQ(0.0)};    //!< contains the offsets for the current feedback

MATH_vec3              gOffsets_V_pu = {_IQ(0.0), _IQ(0.0), _IQ(0.0)};    //!< contains the offsets for the voltage feedback

MATH_vec2              gIdq_ref_pu = {_IQ(0.0), _IQ(0.0)};                            //!< contains the Id and Iq references

MATH_vec2              gVdq_out_pu = {_IQ(0.0), _IQ(0.0)};                            //!< contains the output Vd and Vq from the current controllers

MATH_vec2              gIdq_pu = {_IQ(0.0), _IQ(0.0)};                                    //!< contains the Id and Iq measured values

#ifdef CSM_ENABLE

#pragma DATA_SECTION(gUserParams,"rom_accessed_data");

#endif

USER_Params          gUserParams;

volatile MOTOR_Vars_t gMotorVars = MOTOR_Vars_INIT;      //!< the global motor variables that are defined in main.h and used for display in the debugger's watch window

#ifdef FLASH

// Used for running BackGround in flash, and ISR in RAM

extern uint16_t *RamfuncsLoadStart, *RamfuncsLoadEnd, *RamfuncsRunStart;

#ifdef CSM_ENABLE

extern uint16_t *econst_start, *econst_end, *econst_ram_load;

extern uint16_t *switch_start, *switch_end, *switch_ram_load;

#endif

#endif

MATH_vec3 gIavg = {_IQ(0.0), _IQ(0.0), _IQ(0.0)};

uint16_t gIavg_shift = 1;

MATH_vec3 gPwmData_prev = {_IQ(0.0), _IQ(0.0), _IQ(0.0)};

#ifdef DRV8301_SPI

// Watch window interface to the 8301 SPI

DRV_SPI_8301_Vars_t gDrvSpi8301Vars;

#endif

#ifdef DRV8305_SPI

// Watch window interface to the 8305 SPI

DRV_SPI_8305_Vars_t gDrvSpi8305Vars;

#endif

#ifdef DRV8353_SPI

// Watch window interface to the 8305 SPI

DRV_SPI_8353_Vars_t gDrvSpi8353Vars;

#endif

// define CPU time

CPU_TIME_Obj          cpu_time;

CPU_TIME_Handle    cpu_timeHandle;

HAL_DacData_t gDacData;

_iq gFlux_pu_to_Wb_sf;

_iq gFlux_pu_to_VpHz_sf;

_iq gTorque_Ls_Id_Iq_pu_to_Nm_sf;

_iq gTorque_Flux_Iq_pu_to_Nm_sf;

_iq gSpeed_krpm_to_pu_sf = _IQ((float_t)USER_MOTOR_NUM_POLE_PAIRS * 1000.0 / (USER_IQ_FULL_SCALE_FREQ_Hz * 60.0));

_iq gSpeed_hz_to_krpm_sf = _IQ(60.0 / (float_t)USER_MOTOR_NUM_POLE_PAIRS / 1000.0);

_iq gIs_Max_squared_pu = _IQ((USER_MOTOR_MAX_CURRENT * USER_MOTOR_MAX_CURRENT) / (USER_IQ_FULL_SCALE_CURRENT_A * USER_IQ_FULL_SCALE_CURRENT_A));

uint32_t gOffsetCalcCount = 0;

uint16_t gTrjCnt = 0;

volatile bool gFlag_enableRsOnLine = false;

volatile bool gFlag_updateRs = false;

volatile _iq gRsOnLineFreq_Hz = _IQ(0.2);

volatile _iq gRsOnLineId_mag_A = _IQ(0.5);

volatile _iq gRsOnLinePole_Hz = _IQ(0.2);

//variable

_iq speed_pid_out = _IQ(0.0);

_iq angle_est_pu = _IQ(0.0);

_iq speed_est_pu = _IQ(0.0);

_iq angle_pu = _IQ(0.0);

_iq speed_pu = _IQ(0.0);

bool gCadence1 = false;

bool gCadence2 = false;

//BLDC current loop

_iq gHall_BLDC_Is_fdb_pu = _IQ(0.0);

_iq gHall_BLDC_Is_ref_pu = _IQ(0.0);

_iq gHall_PwmDuty = _IQ(0.0);

// Coefficient for pid regulator switch from Hall to Fast

_iq gHall2Fast_Spd_Coef = _IQ(1.0);

_iq gHall2Fast_Iq_coef = _IQ(0.50);

_iq gHall2Fast_Ui_coef = _IQ(0.50);

// Coefficient for pid regulator switch from Fast to Hall

_iq gFast2Hall_Spd_coef = _IQ(1.0);

_iq gFast2Hall_Iq_coef = _IQ(0.0);

_iq gFast2Hall_Ui_coef = _IQ(2.0);

// BLDC speed

_iq gHall_speed_fdb_pu = _IQ(0.0);

_iq gHall_speed_FastToBldc_low_pu    = _IQ(0.400*USER_MOTOR_NUM_POLE_PAIRS*1000.0/(USER_IQ_FULL_SCALE_FREQ_Hz * 60.0));              // 400rpm

_iq gHall_speed_BldcToFast_high_pu = _IQ(0.750*USER_MOTOR_NUM_POLE_PAIRS*1000.0/(USER_IQ_FULL_SCALE_FREQ_Hz * 60.0));              // 750rpm

_iq gPotentiometer = _IQ(0.0);

double gTorqueOnPedalVoltage = 0.0;

double gTorqueOnPedalTorqueUnFiltered = 0.0;

#define FILTER_ARRAY_SIZE 500

double gTorqureUnfilteredArray[FILTER_ARRAY_SIZE] = {0.0};

uint16_t gTorqureUnfilteredArrayIndex = 0;

double gTorqueFiltered = 0.0;

double gTorqueZeroValue = 0.5;

uint16_t gHall_PwmState = 0;

uint16_t gHall_GpioData = 0;

int16_t gHall_State = 0;

int16_t gHall_PrevState = 0;

int16_t gHall_LastState = 0;

int16_t gHall_State_delta = 0;

uint16_t gHall_dir = 0;

uint16_t gHall_dir_prev = 0;

uint16_t gHall_dir_change = 0;

uint16_t gHall_BLDC_Flag_Is_fdb = 0;

uint16_t gHall_PwmIndex[8] = {4, 1, 5, 6, 3, 2, 4, 1};

//uint16_t gHall_PwmIndex[8] = {1, 4, 2, 3, 6, 5, 1, 4};

bool gHall_Flag_EnableBldc = true;

bool gHall_Flag_EnableStartup = true;              // true->enable hall startup

//bool gHall_Flag_EnableStartup = false;        // false->disable hall startup

bool gHall_Flag_CurrentCtrl = true;

bool gHall_Flag_State_Change = false;

bool FS_flag_enableSpeedCtrl = false;

//bool FS_flag_enableSpeedCtrl = true;

_iq FS_Iq_ref_pu = _IQ(0.0);

_iq FS_Iq_ref_pu_fake = _IQ(0.0);

uint32_t gHall_timer_now= 0;

uint32_t gHall_timer_prev = 0;

uint32_t gHall_time_delta_now = 0;

uint32_t gHall_time_delta_prev = 0;

uint32_t gHall_time_delta = 0;

uint32_t gHall_speed_scale = USER_SYSTEM_FREQ_MHz*1000000*100/6;

uint32_t gHall_speed_fdb_0p01Hz = 0;

_iq gHall_Speed_0p01hz_to_pu_sf = _IQ((float_t)0.01/(USER_IQ_FULL_SCALE_FREQ_Hz));

uint16_t gHall_PwmCnt = 0;

uint16_t gHall_PwmCntMax = 5000;

uint32_t gHall_Bldc_Cnt = 0;

uint32_t gHall_Fast_Cnt = 0;

void HALLBLDC_Ctrl_Run(void);

void HALLBLDC_State_Check(void);

void HALLBLDC_Ctrl_Stop(void);

void HALLBLDC_Ctrl_PwmSet(uint16_t PwmState, _iq PwmDuty);

// **************************************************************************

// the functions

void main(void)

{

// IMPORTANT NOTE: If you are not familiar with MotorWare coding guidelines

// please refer to the following document:

// C:/ti/motorware/motorware_1_01_00_1x/docs/motorware_coding_standards.pdf

// Only used if running from FLASH

// Note that the variable FLASH is defined by the project

#ifdef FLASH

// Copy time critical code and Flash setup code to RAM

// The RamfuncsLoadStart, RamfuncsLoadEnd, and RamfuncsRunStart

// symbols are created by the linker. Refer to the linker files.

memCopy((uint16_t *)&RamfuncsLoadStart,(uint16_t *)&RamfuncsLoadEnd,

(uint16_t *)&RamfuncsRunStart);

#ifdef CSM_ENABLE

//copy .econst to unsecure RAM

if(*econst_end - *econst_start)

{

memCopy((uint16_t *)&econst_start,(uint16_t *)&econst_end,(uint16_t *)&econst_ram_load);

}

//copy .switch ot unsecure RAM

if(*switch_end - *switch_start)

{

memCopy((uint16_t *)&switch_start,(uint16_t *)&switch_end,(uint16_t *)&switch_ram_load);

}

#endif

#endif

// initialize the Hardware Abstraction Layer    (HAL)

// halHandle will be used throughout the code to interface with the HAL

// (set parameters, get and set functions, etc) halHandle is required since

// this is how all objects are interfaced, and it allows interface with

// multiple objects by simply passing a different handle. The use of

// handles is explained in this document:

// C:/ti/motorware/motorware_1_01_00_1x/docs/motorware_coding_standards.pdf

halHandle = HAL_init(&hal,sizeof(hal));

// check for errors in user parameters

USER_checkForErrors(&gUserParams);

// store user parameter error in global variable

gMotorVars.UserErrorCode = USER_getErrorCode(&gUserParams);

// do not allow code execution if there is a user parameter error

if(gMotorVars.UserErrorCode != USER_ErrorCode_NoError)

{

for(;;)

{

gMotorVars.Flag_enableSys = false;

}

}

// initialize the Clarke modules

clarkeHandle_I = CLARKE_init(&clarke_I,sizeof(clarke_I));

clarkeHandle_V = CLARKE_init(&clarke_V,sizeof(clarke_V));

// initialize the estimator

estHandle = EST_init((void *)USER_EST_HANDLE_ADDRESS, 0x200);

// initialize the user parameters

USER_setParams(&gUserParams);

// set the hardware abstraction layer parameters

HAL_setParams(halHandle,&gUserParams);

#ifdef FAST_ROM_V1p6

{

CTRL_Handle ctrlHandle = CTRL_init((void *)USER_CTRL_HANDLE_ADDRESS, 0x200);

CTRL_Obj *obj = (CTRL_Obj *)ctrlHandle;

obj->estHandle = estHandle;

// initialize the estimator through the controller

CTRL_setParams(ctrlHandle,&gUserParams);

CTRL_setUserMotorParams(ctrlHandle);

CTRL_setupEstIdleState(ctrlHandle);

}

#else

{

// initialize the estimator

EST_setEstParams(estHandle,&gUserParams);

EST_setupEstIdleState(estHandle);

}

#endif

// disable Rs recalculation by default

gMotorVars.Flag_enableRsRecalc = false;

EST_setFlag_enableRsRecalc(estHandle,false);

// configure RsOnLine

EST_setFlag_enableRsOnLine(estHandle,gFlag_enableRsOnLine);

EST_setFlag_updateRs(estHandle,gFlag_updateRs);

EST_setRsOnLineAngleDelta_pu(estHandle,_IQmpy(gRsOnLineFreq_Hz, _IQ(1.0/USER_ISR_FREQ_Hz)));

EST_setRsOnLineId_mag_pu(estHandle,_IQmpy(gRsOnLineId_mag_A, _IQ(1.0/USER_IQ_FULL_SCALE_CURRENT_A)));

// Calculate coefficients for all filters

{

_iq b0 = _IQmpy(gRsOnLinePole_Hz, _IQ(1.0/USER_ISR_FREQ_Hz));

_iq a1 = b0 - _IQ(1.0);

EST_setRsOnLineFilterParams(estHandle,EST_RsOnLineFilterType_Current,b0,a1,_IQ(0.0),b0,a1,_IQ(0.0));

EST_setRsOnLineFilterParams(estHandle,EST_RsOnLineFilterType_Voltage,b0,a1,_IQ(0.0),b0,a1,_IQ(0.0));

}

// set the number of current sensors

setupClarke_I(clarkeHandle_I,USER_NUM_CURRENT_SENSORS);

// set the number of voltage sensors

setupClarke_V(clarkeHandle_V,USER_NUM_VOLTAGE_SENSORS);

// set the pre-determined current and voltage feeback offset values

gOffsets_I_pu.value[0] = _IQ(I_A_offset);

gOffsets_I_pu.value[1] = _IQ(I_B_offset);

gOffsets_I_pu.value[2] = _IQ(I_C_offset);

gOffsets_V_pu.value[0] = _IQ(V_A_offset);

gOffsets_V_pu.value[1] = _IQ(V_B_offset);

gOffsets_V_pu.value[2] = _IQ(V_C_offset);

// initialize the PID controllers

{

_iq maxCurrent_pu = _IQ(USER_MOTOR_MAX_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A);

_iq maxVoltage_pu = _IQ(USER_MAX_VS_MAG_PU * USER_VD_SF);

float_t fullScaleCurrent = USER_IQ_FULL_SCALE_CURRENT_A;

float_t fullScaleVoltage = USER_IQ_FULL_SCALE_VOLTAGE_V;

float_t IsrPeriod_sec = 1.0 / USER_ISR_FREQ_Hz;

float_t Ls_d = USER_MOTOR_Ls_d;

float_t Ls_q = USER_MOTOR_Ls_q;

float_t Rs = USER_MOTOR_Rs;

float_t RoverLs_d = Rs/Ls_d;

float_t RoverLs_q = Rs/Ls_q;

_iq Kp_Id = _IQ((0.25*Ls_d*fullScaleCurrent)/(IsrPeriod_sec*fullScaleVoltage));

_iq Ki_Id = _IQ(RoverLs_d*IsrPeriod_sec);

_iq Kp_Iq = _IQ((0.25*Ls_q*fullScaleCurrent)/(IsrPeriod_sec*fullScaleVoltage));

_iq Ki_Iq = _IQ(RoverLs_q*IsrPeriod_sec);

pidHandle[0] = PID_init(&pid[0],sizeof(pid[0]));

pidHandle[1] = PID_init(&pid[1],sizeof(pid[1]));

pidHandle[2] = PID_init(&pid[2],sizeof(pid[2]));

pidHandle[3] = PID_init(&pid[3],sizeof(pid[3]));

PID_setGains(pidHandle[0],_IQ(0.2),_IQ(0.005),_IQ(0.0));

PID_setMinMax(pidHandle[0],-maxCurrent_pu,maxCurrent_pu);

PID_setUi(pidHandle[0],_IQ(0.0));

pidCntSpeed = 0;

PID_setGains(pidHandle[1],Kp_Id,Ki_Id,_IQ(0.0));

PID_setMinMax(pidHandle[1],-maxVoltage_pu,maxVoltage_pu);

PID_setUi(pidHandle[1],_IQ(0.0));

PID_setGains(pidHandle[2],Kp_Iq,Ki_Iq,_IQ(0.0));

PID_setMinMax(pidHandle[2],_IQ(0.0),_IQ(0.0));

PID_setUi(pidHandle[2],_IQ(0.0));

PID_setGains(pidHandle[3],Kp_Iq,Ki_Iq,_IQ(0.0));

PID_setMinMax(pidHandle[3],-maxVoltage_pu,maxVoltage_pu);

PID_setUi(pidHandle[3],_IQ(0.0));

}

// initialize the speed reference in kilo RPM where base speed is USER_IQ_FULL_SCALE_FREQ_Hz

gMotorVars.SpeedRef_krpm = _IQmpy(_IQ(30.0), gSpeed_hz_to_krpm_sf);

// initialize the inverse Park module

iparkHandle = IPARK_init(&ipark,sizeof(ipark));

// initialize and configure offsets using filters

{

uint16_t cnt = 0;

_iq b0 = _IQ(gUserParams.offsetPole_rps/(float_t)gUserParams.ctrlFreq_Hz);

_iq a1 = (b0 - _IQ(1.0));

_iq b1 = _IQ(0.0);

for(cnt=0;cnt<6;cnt++)

{

filterHandle[cnt] = FILTER_FO_init(&filter[cnt],sizeof(filter[0]));

FILTER_FO_setDenCoeffs(filterHandle[cnt],a1);

FILTER_FO_setNumCoeffs(filterHandle[cnt],b0,b1);

FILTER_FO_setInitialConditions(filterHandle[cnt],_IQ(0.0),_IQ(0.0));

}

gMotorVars.Flag_enableOffsetcalc = false;

}

// initialize the space vector generator module

svgenHandle = SVGEN_init(&svgen,sizeof(svgen));

// Initialize and setup the 100% SVM generator

svgencurrentHandle = SVGENCURRENT_init(&svgencurrent,sizeof(svgencurrent));

// setup svgen current

{

float_t minWidth_microseconds = 2.0;

uint16_t minWidth_counts = (uint16_t)(minWidth_microseconds * USER_SYSTEM_FREQ_MHz);

float_t fdutyLimit = 0.5-(2.0*minWidth_microseconds*USER_PWM_FREQ_kHz*0.001);

_iq dutyLimit = _IQ(fdutyLimit);

SVGENCURRENT_setMinWidth(svgencurrentHandle, minWidth_counts);

SVGENCURRENT_setIgnoreShunt(svgencurrentHandle, use_all);

SVGENCURRENT_setMode(svgencurrentHandle,all_phase_measurable);

SVGENCURRENT_setVlimit(svgencurrentHandle,dutyLimit);

}

// initialize the speed reference trajectory

trajHandle_spd = TRAJ_init(&traj_spd,sizeof(traj_spd));

// configure the speed reference trajectory

TRAJ_setTargetValue(trajHandle_spd,_IQ(0.0));

TRAJ_setIntValue(trajHandle_spd,_IQ(0.0));

TRAJ_setMinValue(trajHandle_spd,_IQ(-1.0));

TRAJ_setMaxValue(trajHandle_spd,_IQ(1.0));

TRAJ_setMaxDelta(trajHandle_spd,_IQ(USER_MAX_ACCEL_Hzps / USER_IQ_FULL_SCALE_FREQ_Hz / USER_ISR_FREQ_Hz));

// initialize the Id reference trajectory

trajHandle_Id = TRAJ_init(&traj_Id,sizeof(traj_Id));

if(USER_MOTOR_TYPE == MOTOR_Type_Pm)

{

// configure the Id reference trajectory

TRAJ_setTargetValue(trajHandle_Id,_IQ(0.0));

TRAJ_setIntValue(trajHandle_Id,_IQ(0.0));

TRAJ_setMinValue(trajHandle_Id,_IQ(-USER_MOTOR_MAX_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A));

TRAJ_setMaxValue(trajHandle_Id,_IQ(USER_MOTOR_MAX_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A));

TRAJ_setMaxDelta(trajHandle_Id,_IQ(USER_MOTOR_RES_EST_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A / USER_ISR_FREQ_Hz));

// Initialize field weakening

fwHandle = FW_init(&fw,sizeof(fw));

FW_setFlag_enableFw(fwHandle, false); // Disable field weakening

FW_clearCounter(fwHandle); // Clear field weakening counter

FW_setNumIsrTicksPerFwTick(fwHandle, FW_NUM_ISR_TICKS_PER_CTRL_TICK); // Set the number of ISR per field weakening ticks

FW_setDeltas(fwHandle, FW_INC_DELTA, FW_DEC_DELTA); // Set the deltas of field weakening

FW_setOutput(fwHandle, _IQ(0.0)); // Set initial output of field weakening to zero

FW_setMinMax(fwHandle,_IQ(USER_MAX_NEGATIVE_ID_REF_CURRENT_A/USER_IQ_FULL_SCALE_CURRENT_A),_IQ(0.0)); // Set the field weakening controller limits

}

else

{

// configure the Id reference trajectory

TRAJ_setTargetValue(trajHandle_Id,_IQ(0.0));

TRAJ_setIntValue(trajHandle_Id,_IQ(0.0));

TRAJ_setMinValue(trajHandle_Id,_IQ(0.0));

TRAJ_setMaxValue(trajHandle_Id,_IQ(USER_MOTOR_MAGNETIZING_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A));

TRAJ_setMaxDelta(trajHandle_Id,_IQ(USER_MOTOR_MAGNETIZING_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A / USER_ISR_FREQ_Hz));

}

// initialize the CPU usage module

cpu_timeHandle = CPU_TIME_init(&cpu_time,sizeof(cpu_time));

CPU_TIME_setParams(cpu_timeHandle, PWM_getPeriod(halHandle->pwmHandle[0]));

// setup faults

HAL_setupFaults(halHandle);

// initialize the interrupt vector table

HAL_initIntVectorTable(halHandle);

// enable the ADC interrupts

HAL_enableAdcInts(halHandle);

// enable global interrupts

HAL_enableGlobalInts(halHandle);

// enable debug interrupts

HAL_enableDebugInt(halHandle);

// disable the PWM

HAL_disablePwm(halHandle);

// compute scaling factors for flux and torque calculations

gFlux_pu_to_Wb_sf = USER_computeFlux_pu_to_Wb_sf();

gFlux_pu_to_VpHz_sf = USER_computeFlux_pu_to_VpHz_sf();

gTorque_Ls_Id_Iq_pu_to_Nm_sf = USER_computeTorque_Ls_Id_Iq_pu_to_Nm_sf();

gTorque_Flux_Iq_pu_to_Nm_sf = USER_computeTorque_Flux_Iq_pu_to_Nm_sf();

// enable the system by default

gMotorVars.Flag_enableSys = true;

#ifdef DRV8301_SPI

// turn on the DRV8301 if present

HAL_enableDrv(halHandle);

// initialize the DRV8301 interface

HAL_setupDrvSpi(halHandle,&gDrvSpi8301Vars);

#endif

#ifdef DRV8305_SPI

// turn on the DRV8305 if present

HAL_enableDrv(halHandle);

// initialize the DRV8305 interface

HAL_setupDrvSpi(halHandle,&gDrvSpi8305Vars);

#endif

#ifdef DRV8353

HAL_enableDrv(halHandle);

#endif

#ifdef DRV8353_SPI

// initialize the DRV8353 interface

//if(gDeviceVariant)

HAL_setupDrvSpi(halHandle,&gDrvSpi8353Vars);

#endif

gMotorVars.Flag_enableOffsetcalc = true;

while(gMotorVars.Flag_enableOffsetcalc == true) gMotorVars.Flag_Run_Identify = 0;

// Below two lines code for Flash Testing, need to be commented

//    gMotorVars.Flag_enableSys = true;

gMotorVars.Flag_Run_Identify = false;

static uint32_t countertest = 0;

// Begin the background loop

for(;;)

{

// Waiting for enable system flag to be set

while(!(gMotorVars.Flag_enableSys));

// loop while the enable system flag is true

while(gMotorVars.Flag_enableSys)

{

countertest++;

if (countertest < 1000)

{

GPIO_setHigh(halHandle->gpioHandle, GPIO_Number_10);

/*GPIO_setHigh(halHandle->gpioHandle, GPIO_Number_16);

GPIO_setHigh(halHandle->gpioHandle, GPIO_Number_17);

GPIO_setHigh(halHandle->gpioHandle, GPIO_Number_18);

GPIO_setHigh(halHandle->gpioHandle, GPIO_Number_19);*/

}

else if (countertest < 2*1000)

{

GPIO_setLow(halHandle->gpioHandle, GPIO_Number_10);

/*GPIO_setLow(halHandle->gpioHandle, GPIO_Number_16);

GPIO_setLow(halHandle->gpioHandle, GPIO_Number_17);

GPIO_setLow(halHandle->gpioHandle, GPIO_Number_18);

GPIO_setLow(halHandle->gpioHandle, GPIO_Number_19);*/

}

else

{

countertest=0;

//gDrvSpi8353Vars.WriteCmd = true;

//gDrvSpi8353Vars.ReadCmd = true;

}

gPotentiometer = HAL_readPotentiometerData(halHandle);

if(gMotorVars.Flag_Run_Identify)

{

// disable Rs recalculation

EST_setFlag_enableRsRecalc(estHandle,false);

// update estimator state

EST_updateState(estHandle,0);

#ifdef FAST_ROM_V1p6

// call this function to fix 1p6

softwareUpdate1p6(estHandle);

#endif

// enable the PWM

//                      HAL_enablePwm(halHandle);

if(gHall_Flag_EnableStartup == false)

{

// enable the PWM

HAL_enablePwm(halHandle);

}

// set trajectory target for speed reference

TRAJ_setTargetValue(trajHandle_spd,_IQmpy(gMotorVars.SpeedRef_krpm, gSpeed_krpm_to_pu_sf));

if(USER_MOTOR_TYPE == MOTOR_Type_Pm)

{

// set trajectory target for Id reference

TRAJ_setTargetValue(trajHandle_Id,gIdq_ref_pu.value[0]);

}

else

{

if(gMotorVars.Flag_enablePowerWarp)

{

_iq Id_target_pw_pu = EST_runPowerWarp(estHandle,TRAJ_getIntValue(trajHandle_Id),gIdq_pu.value[1]);

TRAJ_setTargetValue(trajHandle_Id,Id_target_pw_pu);

TRAJ_setMinValue(trajHandle_Id,_IQ(USER_MOTOR_MAGNETIZING_CURRENT * 0.3 / USER_IQ_FULL_SCALE_CURRENT_A));

TRAJ_setMaxDelta(trajHandle_Id,_IQ(USER_MOTOR_MAGNETIZING_CURRENT * 0.3 / USER_IQ_FULL_SCALE_CURRENT_A / USER_ISR_FREQ_Hz));

}

else

{

// set trajectory target for Id reference

TRAJ_setTargetValue(trajHandle_Id,_IQ(USER_MOTOR_MAGNETIZING_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A));

TRAJ_setMinValue(trajHandle_Id,_IQ(0.0));

TRAJ_setMaxDelta(trajHandle_Id,_IQ(USER_MOTOR_MAGNETIZING_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A / USER_ISR_FREQ_Hz));

}

}

}

else if(gMotorVars.Flag_enableRsRecalc)

{

// set angle to zero

EST_setAngle_pu(estHandle,_IQ(0.0));

// enable or disable Rs recalculation

EST_setFlag_enableRsRecalc(estHandle,true);

// update estimator state

EST_updateState(estHandle,0);

#ifdef FAST_ROM_V1p6

// call this function to fix 1p6

softwareUpdate1p6(estHandle);

#endif

// enable the PWM

HAL_enablePwm(halHandle);

// set trajectory target for speed reference

TRAJ_setTargetValue(trajHandle_spd,_IQ(0.0));

// set trajectory target for Id reference

TRAJ_setTargetValue(trajHandle_Id,_IQ(USER_MOTOR_RES_EST_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A));

// if done with Rs recalculation, disable flag

if(EST_getState(estHandle) == EST_State_OnLine) gMotorVars.Flag_enableRsRecalc = false;

}

else

{

// set estimator to Idle

EST_setIdle(estHandle);

// disable the PWM

if(!gMotorVars.Flag_enableOffsetcalc) HAL_disablePwm(halHandle);

// clear the speed reference trajectory

TRAJ_setTargetValue(trajHandle_spd,_IQ(0.0));

TRAJ_setIntValue(trajHandle_spd,_IQ(0.0));

// clear the Id reference trajectory

TRAJ_setTargetValue(trajHandle_Id,_IQ(0.0));

TRAJ_setIntValue(trajHandle_Id,_IQ(0.0));

// configure trajectory Id defaults depending on motor type

if(USER_MOTOR_TYPE == MOTOR_Type_Pm)

{

TRAJ_setMinValue(trajHandle_Id,_IQ(-USER_MOTOR_MAX_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A));

TRAJ_setMaxDelta(trajHandle_Id,_IQ(USER_MOTOR_RES_EST_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A / USER_ISR_FREQ_Hz));

}

else

{

TRAJ_setMinValue(trajHandle_Id,_IQ(0.0));

TRAJ_setMaxDelta(trajHandle_Id,_IQ(USER_MOTOR_MAGNETIZING_CURRENT / USER_IQ_FULL_SCALE_CURRENT_A / USER_ISR_FREQ_Hz));

}

// clear integral outputs

PID_setUi(pidHandle[0],_IQ(0.0));

PID_setUi(pidHandle[1],_IQ(0.0));

PID_setUi(pidHandle[2],_IQ(0.0));

// clear Id and Iq references

gIdq_ref_pu.value[0] = _IQ(0.0);

gIdq_ref_pu.value[1] = _IQ(0.0);

// disable RsOnLine flags

gFlag_enableRsOnLine = false;

gFlag_updateRs = false;

// disable PowerWarp flag

gMotorVars.Flag_enablePowerWarp = false;

}

// update the global variables

updateGlobalVariables(estHandle);

// set field weakening enable flag depending on user's input

FW_setFlag_enableFw(fwHandle,gMotorVars.Flag_enableFieldWeakening);

// set the speed acceleration

TRAJ_setMaxDelta(trajHandle_spd,_IQmpy(MAX_ACCEL_KRPMPS_SF,gMotorVars.MaxAccel_krpmps));

// enable/disable the forced angle

EST_setFlag_enableForceAngle(estHandle,gMotorVars.Flag_enableForceAngle);

// enable or disable RsOnLine

EST_setFlag_enableRsOnLine(estHandle,gFlag_enableRsOnLine);

// set slow rotating frequency for RsOnLine

EST_setRsOnLineAngleDelta_pu(estHandle,_IQmpy(gRsOnLineFreq_Hz, _IQ(1.0/USER_ISR_FREQ_Hz)));

// set current amplitude for RsOnLine

EST_setRsOnLineId_mag_pu(estHandle,_IQmpy(gRsOnLineId_mag_A, _IQ(1.0/USER_IQ_FULL_SCALE_CURRENT_A)));

// set flag that updates Rs from RsOnLine value

EST_setFlag_updateRs(estHandle,gFlag_updateRs);

// clear Id for RsOnLine if disabled

if(!gFlag_enableRsOnLine) EST_setRsOnLineId_pu(estHandle,_IQ(0.0));

#ifdef DRV8301_SPI

HAL_writeDrvData(halHandle,&gDrvSpi8301Vars);

HAL_readDrvData(halHandle,&gDrvSpi8301Vars);

#endif

#ifdef DRV8305_SPI

HAL_writeDrvData(halHandle,&gDrvSpi8305Vars);

HAL_readDrvData(halHandle,&gDrvSpi8305Vars);

#endif

#ifdef DRV8353_SPI

HAL_writeDrvData(halHandle,&gDrvSpi8353Vars);

HAL_readDrvData(halHandle,&gDrvSpi8353Vars);

#endif

} // end of while(gFlag_enableSys) loop

// disable the PWM

HAL_disablePwm(halHandle);

gMotorVars.Flag_Run_Identify = false;

} // end of for(;;) loop

} // end of main() function

//! \brief          The main ISR that implements the motor control.

interrupt void mainISR(void)

{

// read the timer 1 value and update the CPU usage module

uint32_t timer1Cnt = HAL_readTimerCnt(halHandle,2);

CPU_TIME_updateCnts(cpu_timeHandle,timer1Cnt);

_iq oneOverDcBus;

MATH_vec2 Iab_pu;

MATH_vec2 Vab_pu;

MATH_vec2 phasor;

// acknowledge the ADC interrupt

HAL_acqAdcInt(halHandle,ADC_IntNumber_1);

// convert the ADC data

HAL_readAdcDataWithOffsets(halHandle,&gAdcData);

//HAL_readAdcData(halHandle,&gAdcData);

// remove offsets

gAdcData.I.value[0] = gAdcData.I.value[0] - gOffsets_I_pu.value[0];

gAdcData.I.value[1] = gAdcData.I.value[1] - gOffsets_I_pu.value[1];

gAdcData.I.value[2] = gAdcData.I.value[2] - gOffsets_I_pu.value[2];

gAdcData.V.value[0] = gAdcData.V.value[0] - gOffsets_V_pu.value[0];

gAdcData.V.value[1] = gAdcData.V.value[1] - gOffsets_V_pu.value[1];

gAdcData.V.value[2] = gAdcData.V.value[2] - gOffsets_V_pu.value[2];

if (gAdcData.I.value[0] > _IQ(2.0))

{

gAdcData.I.value[0] = 0.0;

}

if (gAdcData.I.value[1] > _IQ(2.0))

{

gAdcData.I.value[1] = 0.0;

}

if (gAdcData.I.value[2] > _IQ(2.0))

{

gAdcData.I.value[2] = 0.0;

}

// run the current reconstruction algorithm

runCurrentReconstruction();

// run Hall state check for startup and running

HALLBLDC_State_Check();

// run Clarke transform on current

CLARKE_run(clarkeHandle_I,&gAdcData.I,&Iab_pu);

// run Clarke transform on voltage

CLARKE_run(clarkeHandle_V,&gAdcData.V,&Vab_pu);

// run a trajectory for Id reference, so the reference changes with a ramp instead of a step

TRAJ_run(trajHandle_Id);

// run the estimator

EST_run(estHandle,

&Iab_pu,

&Vab_pu,

gAdcData.dcBus,

TRAJ_getIntValue(trajHandle_spd));

// generate the motor electrical angle

angle_est_pu = EST_getAngle_pu(estHandle);

speed_est_pu = EST_getFm_pu(estHandle);

// get Idq from estimator to avoid sin and cos

EST_getIdq_pu(estHandle,&gIdq_pu);

// run the appropriate controller

if((gMotorVars.Flag_Run_Identify) || (gMotorVars.Flag_enableRsRecalc))

{

_iq refValue;

_iq fbackValue;

_iq outMax_pu;

// when appropriate, run the PID speed controller

if(FS_flag_enableSpeedCtrl == true)

{

if(pidCntSpeed++ >= USER_NUM_CTRL_TICKS_PER_SPEED_TICK)

{

// calculate Id reference squared

_iq Id_ref_squared_pu = _IQmpy(PID_getRefValue(pidHandle[1]),PID_getRefValue(pidHandle[1]));

// Take into consideration that Iq^2+Id^2 = Is^2

_iq Iq_Max_pu = _IQsqrt(gIs_Max_squared_pu - Id_ref_squared_pu);

// clear counter

pidCntSpeed = 0;

// Set new min and max for the speed controller output

PID_setMinMax(pidHandle[0], -Iq_Max_pu, Iq_Max_pu);

// run speed controller

PID_run_spd(pidHandle[0],TRAJ_getIntValue(trajHandle_spd),speed_pu, &speed_pid_out);

gIdq_ref_pu.value[1] = speed_pid_out;

}

}

else

{

gIdq_ref_pu.value[1] = FS_Iq_ref_pu;

}

// get the reference value from the trajectory module

refValue = TRAJ_getIntValue(trajHandle_Id) + EST_getRsOnLineId_pu(estHandle);

// get the feedback value

fbackValue = gIdq_pu.value[0];

// run the Id PID controller

PID_run(pidHandle[1],refValue,fbackValue,&(gVdq_out_pu.value[0]));

// set Iq reference to zero when doing Rs recalculation

if(gMotorVars.Flag_enableRsRecalc) gIdq_ref_pu.value[1] = _IQ(0.0);

// get the Iq reference value

refValue = gIdq_ref_pu.value[1];

// get the feedback value

fbackValue = gIdq_pu.value[1];

// calculate Iq controller limits, and run Iq controller

_iq max_vs = _IQmpy(_IQ(USER_MAX_VS_MAG_PU),EST_getDcBus_pu(estHandle));

outMax_pu = _IQsqrt(_IQmpy(max_vs,max_vs) - _IQmpy(gVdq_out_pu.value[0],gVdq_out_pu.value[0]));

PID_setMinMax(pidHandle[2],-outMax_pu,outMax_pu);

PID_run(pidHandle[2],refValue,fbackValue,&(gVdq_out_pu.value[1]));

// compensate angle for PWM delay

angle_est_pu = angleDelayComp(speed_est_pu, angle_est_pu);

// compute the sin/cos phasor

phasor.value[0] = _IQcosPU(angle_pu);

phasor.value[1] = _IQsinPU(angle_pu);

// set the phasor in the inverse Park transform

IPARK_setPhasor(iparkHandle,&phasor);

// run the inverse Park module

IPARK_run(iparkHandle,&gVdq_out_pu,&Vab_pu);

// run the space Vector Generator (SVGEN) module

oneOverDcBus = EST_getOneOverDcBus_pu(estHandle);

Vab_pu.value[0] = _IQmpy(Vab_pu.value[0],oneOverDcBus);

Vab_pu.value[1] = _IQmpy(Vab_pu.value[1],oneOverDcBus);

SVGEN_run(svgenHandle,&Vab_pu,&(gPwmData.Tabc));

// run the PWM compensation and current ignore algorithm

SVGENCURRENT_compPwmData(svgencurrentHandle,&(gPwmData.Tabc),&gPwmData_prev);

// Run BLDC

HALLBLDC_Ctrl_Run();

// increase traj count

gTrjCnt++;

}

else if(gMotorVars.Flag_enableOffsetcalc == true)

{

runOffsetsCalculation();

}

else

{

// disable the PWM

HAL_disablePwm(halHandle);

// Set the PWMs to 50% duty cycle

gPwmData.Tabc.value[0] = _IQ(0.0);

gPwmData.Tabc.value[1] = _IQ(0.0);

gPwmData.Tabc.value[2] = _IQ(0.0);

HALLBLDC_Ctrl_Stop();

}

// write the PWM compare values

HAL_writePwmData(halHandle,&gPwmData);

if(gTrjCnt >= gUserParams.numCtrlTicksPerTrajTick)

{

// clear counter

gTrjCnt = 0;

// run a trajectory for speed reference, so the reference changes with a ramp instead of a step

TRAJ_run(trajHandle_spd);

}

// run function to set next trigger

if(!gMotorVars.Flag_enableRsRecalc) runSetTrigger();

// run field weakening

if(USER_MOTOR_TYPE == MOTOR_Type_Pm) runFieldWeakening();

gMotorVars.angle_est_pu = angle_pu;

gMotorVars.speed_est_pu = speed_est_pu;

#ifndef F2802xF

// connect inputs of the PWMDAC module.

gDacData.value[0] = gAdcData.I.value[0];

gDacData.value[1] = gIdq_ref_pu.value[0];

gDacData.value[2] = gMotorVars.speed_est_pu;

gDacData.value[3] = gMotorVars.angle_est_pu;

// run PwmDAC

HAL_writeDacData(halHandle,&gDacData);

#endif

// read the timer 2 value and update the CPU usage module

timer1Cnt = HAL_readTimerCnt(halHandle,2);

CPU_TIME_run(cpu_timeHandle,timer1Cnt);

return;

} // end of mainISR() function

_iq angleDelayComp(const _iq fm_pu, const _iq angleUncomp_pu)

{

_iq angleDelta_pu = _IQmpy(fm_pu,_IQ(USER_IQ_FULL_SCALE_FREQ_Hz/(USER_PWM_FREQ_kHz*1000.0)));

_iq angleCompFactor = _IQ(1.0 + (float_t)USER_NUM_PWM_TICKS_PER_ISR_TICK * 0.5);

_iq angleDeltaComp_pu = _IQmpy(angleDelta_pu, angleCompFactor);

uint32_t angleMask = ((uint32_t)0xFFFFFFFF >> (32 - GLOBAL_Q));

_iq angleComp_pu;

_iq angleTmp_pu;

// increment the angle

angleTmp_pu = angleUncomp_pu + angleDeltaComp_pu;

// mask the angle for wrap around

// note: must account for the sign of the angle

angleComp_pu = _IQabs(angleTmp_pu) & angleMask;

// account for sign

if(angleTmp_pu < _IQ(0.0))

{

angleComp_pu = -angleComp_pu;

}

return(angleComp_pu);

} // end of angleDelayComp() function

void runCurrentReconstruction(void)

{

SVGENCURRENT_MeasureShunt_e measurableShuntThisCycle = SVGENCURRENT_getMode(svgencurrentHandle);

SVGENCURRENT_RunRegenCurrent(svgencurrentHandle, (MATH_vec3 *)(gAdcData.I.value));

gIavg.value[0] += (gAdcData.I.value[0] - gIavg.value[0])>>gIavg_shift;

gIavg.value[1] += (gAdcData.I.value[1] - gIavg.value[1])>>gIavg_shift;

gIavg.value[2] += (gAdcData.I.value[2] - gIavg.value[2])>>gIavg_shift;

if(measurableShuntThisCycle > two_phase_measurable)

{

gAdcData.I.value[0] = gIavg.value[0];

gAdcData.I.value[1] = gIavg.value[1];

gAdcData.I.value[2] = gIavg.value[2];

}

return;

} // end of runCurrentReconstruction() function

void runSetTrigger(void)

{

SVGENCURRENT_IgnoreShunt_e ignoreShuntNextCycle = SVGENCURRENT_getIgnoreShunt(svgencurrentHandle);

SVGENCURRENT_VmidShunt_e midVolShunt = SVGENCURRENT_getVmid(svgencurrentHandle);

// Set trigger point in the middle of the low side pulse

HAL_setTrigger(halHandle,ignoreShuntNextCycle,midVolShunt);

return;

} // end of runSetTrigger() function

void runFieldWeakening(void)

{

if(FW_getFlag_enableFw(fwHandle) == true)

{

FW_incCounter(fwHandle);

if(FW_getCounter(fwHandle) > FW_getNumIsrTicksPerFwTick(fwHandle))

{

_iq refValue;

_iq fbackValue;

FW_clearCounter(fwHandle);

refValue = gMotorVars.VsRef;

fbackValue =_IQmpy(gMotorVars.Vs,EST_getOneOverDcBus_pu(estHandle));

FW_run(fwHandle, refValue, fbackValue, &(gIdq_ref_pu.value[0]));

gMotorVars.IdRef_A = _IQmpy(gIdq_ref_pu.value[0], _IQ(USER_IQ_FULL_SCALE_CURRENT_A));

}

}

else

{

gIdq_ref_pu.value[0] = _IQmpy(gMotorVars.IdRef_A, _IQ(1.0/USER_IQ_FULL_SCALE_CURRENT_A));

}

return;

} // end of runFieldWeakening() function

void runOffsetsCalculation(void)

{

uint16_t cnt;

// enable the PWM

HAL_enablePwm(halHandle);

for(cnt=0;cnt<3;cnt++)

{

// Set the PWMs to 50% duty cycle

gPwmData.Tabc.value[cnt] = _IQ(0.0);

// reset offsets used

gOffsets_I_pu.value[cnt] = _IQ(0.0);

gOffsets_V_pu.value[cnt] = _IQ(0.0);

// run offset estimation

FILTER_FO_run(filterHandle[cnt],gAdcData.I.value[cnt]);

FILTER_FO_run(filterHandle[cnt+3],gAdcData.V.value[cnt]);

}

if(gOffsetCalcCount++ >= gUserParams.ctrlWaitTime[CTRL_State_OffLine])

{

gMotorVars.Flag_enableOffsetcalc = false;

gOffsetCalcCount = 0;

for(cnt=0;cnt<3;cnt++)

{

// get calculated offsets from filter

gOffsets_I_pu.value[cnt] = FILTER_FO_get_y1(filterHandle[cnt]);

gOffsets_V_pu.value[cnt] = FILTER_FO_get_y1(filterHandle[cnt+3]);

// clear filters

FILTER_FO_setInitialConditions(filterHandle[cnt],_IQ(0.0),_IQ(0.0));

FILTER_FO_setInitialConditions(filterHandle[cnt+3],_IQ(0.0),_IQ(0.0));

}

}

return;

} // end of runOffsetsCalculation() function

void softwareUpdate1p6(EST_Handle handle)

{

float_t fullScaleInductance = USER_IQ_FULL_SCALE_VOLTAGE_V/(USER_IQ_FULL_SCALE_CURRENT_A*USER_VOLTAGE_FILTER_POLE_rps);

float_t Ls_coarse_max = _IQ30toF(EST_getLs_coarse_max_pu(handle));

int_least8_t lShift = ceil(log(USER_MOTOR_Ls_d/(Ls_coarse_max*fullScaleInductance))/log(2.0));

uint_least8_t Ls_qFmt = 30 - lShift;

float_t L_max = fullScaleInductance * pow(2.0,lShift);

_iq Ls_d_pu = _IQ30(USER_MOTOR_Ls_d / L_max);

_iq Ls_q_pu = _IQ30(USER_MOTOR_Ls_q / L_max);

// store the results

EST_setLs_d_pu(handle,Ls_d_pu);

EST_setLs_q_pu(handle,Ls_q_pu);

EST_setLs_qFmt(handle,Ls_qFmt);

return;

} // end of softwareUpdate1p6() function

//! \brief          Setup the Clarke transform for either 2 or 3 sensors.

//! \param[in] handle                          The clarke (CLARKE) handle

//! \param[in] numCurrentSensors    The number of current sensors

void setupClarke_I(CLARKE_Handle handle,const uint_least8_t numCurrentSensors)

{

_iq alpha_sf,beta_sf;

// initialize the Clarke transform module for current

if(numCurrentSensors == 3)

{

alpha_sf = _IQ(MATH_ONE_OVER_THREE);

beta_sf = _IQ(MATH_ONE_OVER_SQRT_THREE);

}

else if(numCurrentSensors == 2)

{

alpha_sf = _IQ(1.0);

beta_sf = _IQ(MATH_ONE_OVER_SQRT_THREE);

}

else

{

alpha_sf = _IQ(0.0);

beta_sf = _IQ(0.0);

}

// set the parameters

CLARKE_setScaleFactors(handle,alpha_sf,beta_sf);

CLARKE_setNumSensors(handle,numCurrentSensors);

return;

} // end of setupClarke_I() function

//! \brief          Setup the Clarke transform for either 2 or 3 sensors.

//! \param[in] handle                          The clarke (CLARKE) handle

//! \param[in] numVoltageSensors    The number of voltage sensors

void setupClarke_V(CLARKE_Handle handle,const uint_least8_t numVoltageSensors)

{

_iq alpha_sf,beta_sf;

// initialize the Clarke transform module for voltage

if(numVoltageSensors == 3)

{

alpha_sf = _IQ(MATH_ONE_OVER_THREE);

beta_sf = _IQ(MATH_ONE_OVER_SQRT_THREE);

}

else

{

alpha_sf = _IQ(0.0);

beta_sf = _IQ(0.0);

}

// set the parameters

CLARKE_setScaleFactors(handle,alpha_sf,beta_sf);

CLARKE_setNumSensors(handle,numVoltageSensors);

return;

} // end of setupClarke_V() function

//! \brief          Update the global variables (gMotorVars).

//! \param[in] handle    The estimator (EST) handle

void updateGlobalVariables(EST_Handle handle)

{

// get the speed estimate

gMotorVars.Speed_krpm = EST_getSpeed_krpm(handle);

// get the torque estimate

{

_iq Flux_pu = EST_getFlux_pu(handle);

_iq Id_pu = PID_getFbackValue(pidHandle[1]);

_iq Iq_pu = PID_getFbackValue(pidHandle[2]);

_iq Ld_minus_Lq_pu = _IQ30toIQ(EST_getLs_d_pu(handle)-EST_getLs_q_pu(handle));

_iq Torque_Flux_Iq_Nm = _IQmpy(_IQmpy(Flux_pu,Iq_pu),gTorque_Flux_Iq_pu_to_Nm_sf);

_iq Torque_Ls_Id_Iq_Nm = _IQmpy(_IQmpy(_IQmpy(Ld_minus_Lq_pu,Id_pu),Iq_pu),gTorque_Ls_Id_Iq_pu_to_Nm_sf);

_iq Torque_Nm = Torque_Flux_Iq_Nm + Torque_Ls_Id_Iq_Nm;

gMotorVars.Torque_Nm = Torque_Nm;

}

// get the magnetizing current

gMotorVars.MagnCurr_A = EST_getIdRated(handle);

// get the rotor resistance

gMotorVars.Rr_Ohm = EST_getRr_Ohm(handle);

// get the stator resistance

gMotorVars.Rs_Ohm = EST_getRs_Ohm(handle);

// get the online stator resistance

gMotorVars.RsOnLine_Ohm = EST_getRsOnLine_Ohm(handle);

// get the stator inductance in the direct coordinate direction

gMotorVars.Lsd_H = EST_getLs_d_H(handle);

// get the stator inductance in the quadrature coordinate direction

gMotorVars.Lsq_H = EST_getLs_q_H(handle);

// get the flux in V/Hz in floating point

gMotorVars.Flux_VpHz = EST_getFlux_VpHz(handle);

// get the flux in Wb in fixed point

gMotorVars.Flux_Wb = _IQmpy(EST_getFlux_pu(handle),gFlux_pu_to_Wb_sf);

// get the estimator state

gMotorVars.EstState = EST_getState(handle);

// Get the DC buss voltage

gMotorVars.VdcBus_kV = _IQmpy(gAdcData.dcBus,_IQ(USER_IQ_FULL_SCALE_VOLTAGE_V/1000.0));

//Update torque

gTorqueOnPedalVoltage = ADC_readResult(halHandle->adcHandle,ADC_ResultNumber_8) * 3.3;

gTorqueOnPedalVoltage = gTorqueOnPedalVoltage / 4095.0;

if ((gTorqueOnPedalVoltage - gTorqueZeroValue > 0.0))

{

gTorqueOnPedalTorqueUnFiltered = (gTorqueOnPedalVoltage - gTorqueZeroValue) / 0.04;

}

else {gTorqueOnPedalTorqueUnFiltered = 0.0;}

if (gTorqureUnfilteredArrayIndex < FILTER_ARRAY_SIZE)

{

gTorqureUnfilteredArrayIndex++;

}

else { gTorqureUnfilteredArrayIndex = 0;}

gTorqureUnfilteredArray[gTorqureUnfilteredArrayIndex] = gTorqueOnPedalTorqueUnFiltered;

uint16_t idxFilterLoop = 0;

gTorqueFiltered = 0.0;

double TorqueFilteredTemp = 0.0;

for (idxFilterLoop=0; idxFilterLoop<FILTER_ARRAY_SIZE; idxFilterLoop++)

{

TorqueFilteredTemp = TorqueFilteredTemp + gTorqureUnfilteredArray[idxFilterLoop];

//gTorqueFiltered = gTorqueFiltered + gTorqureUnfilteredArray[idxFilterLoop];

}

gTorqueFiltered = TorqueFilteredTemp/500.0;

FS_Iq_ref_pu_fake = _IQmpyI32(_IQ(0.1), gTorqueFiltered);

if (gTorqueFiltered < 3.5)

{

FS_Iq_ref_pu_fake = _IQ(0.0);

}

if (FS_Iq_ref_pu_fake > _IQ(0.5))

{

FS_Iq_ref_pu_fake = _IQ(0.5);

}

//FS_Iq_ref_pu = FS_Iq_ref_pu_fake;

//gCadence1 = HAL_readGpio(halHandle, GPIO_Number_28);

//gCadence2 = HAL_readGpio(halHandle, GPIO_Number_29);

// read Vd and Vq vectors per units

gMotorVars.Vd = gVdq_out_pu.value[0];

gMotorVars.Vq = gVdq_out_pu.value[1];

// calculate vector Vs in per units

gMotorVars.Vs = _IQsqrt(_IQmpy(gMotorVars.Vd, gMotorVars.Vd) + _IQmpy(gMotorVars.Vq, gMotorVars.Vq));

// read Id and Iq vectors in amps

gMotorVars.Id_A = _IQmpy(gIdq_pu.value[0], _IQ(USER_IQ_FULL_SCALE_CURRENT_A));

gMotorVars.Iq_A = _IQmpy(gIdq_pu.value[1], _IQ(USER_IQ_FULL_SCALE_CURRENT_A));

// calculate vector Is in amps

gMotorVars.Is_A = _IQsqrt(_IQmpy(gMotorVars.Id_A, gMotorVars.Id_A) + _IQmpy(gMotorVars.Iq_A, gMotorVars.Iq_A));

return;

} // end of updateGlobalVariables() function

//! \brief

void HALLBLDC_Ctrl_Run(void)

{

if(gHall_Flag_EnableStartup == true)

{

gHall_PwmState = gHall_PwmIndex[gHall_State];

if(_IQabs(speed_est_pu)    < gHall_speed_FastToBldc_low_pu)      // FAST to Hall

{

if(gHall_Flag_EnableBldc == false)

{

if(gHall_Bldc_Cnt > 20)

{

gHall_Flag_EnableBldc = true;

gHall_Bldc_Cnt = 0;

if(gHall_Flag_CurrentCtrl == true)            // Torque Control Mode

{

// The following instructions load the parameters for the speed PI

// controller.

PID_setGains(pidHandle[0],_IQ(0.1),_IQ(0.005),_IQ(0.0));

// Set the initial condition value for the integrator output to 0

PID_setUi(pidHandle[3], _IQmpy(pid[2].Ui, gFast2Hall_Ui_coef));

}

else                // Speed Control Mode

{

// The following instructions load the parameters for the speed PI

// controller.

PID_setGains(pidHandle[0],_IQ(0.1),_IQ(0.005),_IQ(0.0));

gHall_PwmDuty = pid[2].Ui;

// Set the initial condition value for the integrator output to 0

PID_setUi(pidHandle[0], _IQmpy(pid[2].Ui, gFast2Hall_Spd_coef));

}

}

else

gHall_Bldc_Cnt++;

}

}

else if(_IQabs(gHall_speed_fdb_pu) > gHall_speed_BldcToFast_high_pu)                        // Hall to FAST

{

if(gHall_Flag_EnableBldc == true)

{

if(gHall_Fast_Cnt > 20)

{

gHall_Flag_EnableBldc = false;

gHall_Fast_Cnt = 0;

if(gHall_Flag_CurrentCtrl == true)                // Torque Control

{

// The following instructions load the parameters for the speed PI

// controller.

PID_setGains(pidHandle[0],_IQ(2.0),_IQ(0.02),_IQ(0.0));

// Set the initial condition value for the integrator output to 0, Id

PID_setUi(pidHandle[1],_IQ(0.0));

// Set the initial condition value for the integrator output to 0, Iq

PID_setUi(pidHandle[2], _IQmpy(pid[3].Ui, gHall2Fast_Ui_coef));

}

else                            // speed control

{

// The following instructions load the parameters for the speed PI

// controller.

PID_setGains(pidHandle[0],_IQ(2.0),_IQ(0.02),_IQ(0.0));

// Set the initial condition value for the integrator output to 0, speed

PID_setUi(pidHandle[0], _IQmpy(gHall_PwmDuty, gHall2Fast_Spd_Coef));

// Set the initial condition value for the integrator output to 0, Iq

PID_setUi(pidHandle[2], _IQmpy(gHall_PwmDuty, gHall2Fast_Iq_coef));

// Set the initial condition value for the integrator output to 0, Id

PID_setUi(pidHandle[1],_IQmpy(gHall_PwmDuty, _IQ(0.0)));

}

PWM_setSocAPulseSrc(hal.pwmHandle[PWM_Number_1],PWM_SocPulseSrc_CounterEqualZero);

HAL_enablePwm(halHandle);

}

else

gHall_Fast_Cnt++;

}

}

if(gHall_Flag_EnableBldc)

{

angle_pu = angle_est_pu;

speed_pu = gHall_speed_fdb_pu;

if(gHall_Flag_CurrentCtrl == true)            // Torque Control Mode

{

gHall_BLDC_Is_fdb_pu = gAdcData.I.value[gHall_BLDC_Flag_Is_fdb];

if(FS_flag_enableSpeedCtrl == true)

{

gHall_BLDC_Is_ref_pu = speed_pid_out;

}

else

{

gHall_BLDC_Is_ref_pu = FS_Iq_ref_pu;

}

// BLDC current loop

PID_run(pidHandle[3],gHall_BLDC_Is_ref_pu,gHall_BLDC_Is_fdb_pu,&gHall_PwmDuty);

HALLBLDC_Ctrl_PwmSet(gHall_PwmState, gHall_PwmDuty);

}

else                                                                        // Speed Control Mode

{

gHall_PwmDuty = speed_pid_out;

HALLBLDC_Ctrl_PwmSet(gHall_PwmState, gHall_PwmDuty);

}

}

else

{

angle_pu = angle_est_pu;

speed_pu = speed_est_pu;

}

}

else        //(gHall_Flag_EnableStartup == false)

{

angle_pu = angle_est_pu;

speed_pu = speed_est_pu;

}

return;

}

//! \brief

void HALLBLDC_State_Check(void)

{

// Hall_A, Hall_B, Hall_C

gHall_GpioData    = !(~HAL_readGpio(halHandle, (GPIO_Number_e)HAL_HallGpio_C) & 0x1) << 2;      //CAP1->J10/J4_1->green

gHall_GpioData += !(~HAL_readGpio(halHandle, (GPIO_Number_e)HAL_HallGpio_B) & 0x1) << 1;      //CAP2->J10/J4_2->green&white

gHall_GpioData += !(~HAL_readGpio(halHandle, (GPIO_Number_e)HAL_HallGpio_A) & 0x1);                //CAP3->J10/J4_3->gray&white

gHall_State = gHall_GpioData;

if(gHall_State != gHall_PrevState)

{

gHall_timer_now = HAL_readTimerCnt(halHandle,2);

gHall_time_delta_now = gHall_timer_prev - gHall_timer_now;

gHall_timer_prev = gHall_timer_now;

gHall_time_delta = (gHall_time_delta_now + gHall_time_delta_prev)>>1;

gHall_time_delta_prev = gHall_time_delta_now;

gHall_speed_fdb_0p01Hz = gHall_speed_scale/gHall_time_delta;

gHall_speed_fdb_pu = gHall_speed_fdb_0p01Hz*gHall_Speed_0p01hz_to_pu_sf;

//            if(TRAJ_getIntValue(trajHandle_spd) < _IQ(0.0))

//            {

//                    gHall_speed_fdb_pu = -gHall_speed_fdb_pu;

//            }

//direction check

gHall_State_delta = gHall_PwmIndex[gHall_State] - gHall_PwmIndex[gHall_PrevState];

if((gHall_State_delta == -1) || (gHall_State_delta == 5))

{

gHall_dir = 1;            // positive direction

}

else if((gHall_State_delta == 1) || (gHall_State_delta == -5))

{

gHall_dir = 2;            // negative direction

gHall_speed_fdb_pu = -gHall_speed_fdb_pu;

}

else

{

gHall_dir = 0;            // direction change

}

//check if dirction is chagned.

//if direction is changed, speed feedback is reset.

if(gHall_dir != gHall_dir_prev)

{

gHall_speed_fdb_pu = _IQ(0.0);

gHall_dir_change = 1;

}

gHall_LastState = gHall_PrevState;

gHall_PrevState = gHall_State;

gHall_dir_prev = gHall_dir;

gHall_Flag_State_Change = true;

gHall_PwmCnt = 0;

}

else

{

gHall_PwmCnt++;

if(gHall_PwmCnt > gHall_PwmCntMax)

{

gHall_speed_fdb_pu = _IQ(0.0);

gHall_PwmCnt = 0;

}

}

return;

}

//! \brief

void HALLBLDC_Ctrl_Stop(void)

{

gHall_Flag_EnableBldc = true;

gHall_Flag_State_Change = false;

gHall_speed_fdb_pu = _IQ(0.0);

// Set the initial condition value for the integrator output to 0

PID_setUi(pidHandle[0],_IQ(0.0));

PID_setUi(pidHandle[1],_IQ(0.0));

PID_setUi(pidHandle[2],_IQ(0.0));

PID_setUi(pidHandle[3],_IQ(0.0));

}

//! \brief

void HALLBLDC_Ctrl_PwmSet(uint16_t PwmState, _iq PwmDuty)

{

switch(PwmState)

{

case 0:          // V+/W-

{

PWM_setOneShotTrip(hal.pwmHandle[PWM_Number_1]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_2]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_3]);

gPwmData.Tabc.value[0] = _IQ(0.0);

gPwmData.Tabc.value[1] = PwmDuty;

gPwmData.Tabc.value[2] = -PwmDuty;

gHall_BLDC_Flag_Is_fdb = 1;

break;

}

case 1:          // U+/W-

{

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_1]);

PWM_setOneShotTrip(hal.pwmHandle[PWM_Number_2]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_3]);

gPwmData.Tabc.value[0] = PwmDuty;

gPwmData.Tabc.value[1] = _IQ(0.0);

gPwmData.Tabc.value[2] = -PwmDuty;

gHall_BLDC_Flag_Is_fdb = 0;

break;

}

case 2:          //U+/V-

{

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_1]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_2]);

PWM_setOneShotTrip(hal.pwmHandle[PWM_Number_3]);

gPwmData.Tabc.value[0] = PwmDuty;

gPwmData.Tabc.value[1] = -PwmDuty;

gPwmData.Tabc.value[2] = _IQ(0.0);

gHall_BLDC_Flag_Is_fdb = 0;

break;

}

case 3:          //W+/V-

{

PWM_setOneShotTrip(hal.pwmHandle[PWM_Number_1]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_2]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_3]);

gPwmData.Tabc.value[0] = _IQ(0.0);

gPwmData.Tabc.value[1] = -PwmDuty;

gPwmData.Tabc.value[2] = PwmDuty;

gHall_BLDC_Flag_Is_fdb = 2;

break;

}

case 4:          //W+/U-

{

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_1]);

PWM_setOneShotTrip(hal.pwmHandle[PWM_Number_2]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_3]);

gPwmData.Tabc.value[0] = -PwmDuty;

gPwmData.Tabc.value[1] = _IQ(0.0);

gPwmData.Tabc.value[2] = PwmDuty;

gHall_BLDC_Flag_Is_fdb = 2;

break;

}

case 5:          // V+/U-

{

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_1]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_2]);

PWM_setOneShotTrip(hal.pwmHandle[PWM_Number_3]);

gPwmData.Tabc.value[0] = -PwmDuty;

gPwmData.Tabc.value[1] = PwmDuty;

gPwmData.Tabc.value[2] = _IQ(0.0);

gHall_BLDC_Flag_Is_fdb = 1;

break;

}

case 6:          // V+/W-

{

PWM_setOneShotTrip(hal.pwmHandle[PWM_Number_1]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_2]);

PWM_clearOneShotTrip(hal.pwmHandle[PWM_Number_3]);

gPwmData.Tabc.value[0] = _IQ(0.0);

gPwmData.Tabc.value[1] = PwmDuty;

gPwmData.Tabc.value[2] = -PwmDuty;

gHall_BLDC_Flag_Is_fdb = 1;

break;

}

default:        // N/A

break;

}

}

//@} //defgroup

// end of file