-The main frequency of failure device is 24Mhz

-The SDK in failure device is SDK-2-20-00-49

-Base on the BLE_Stack example

The config of advertisement:

#define DEFAULT_ADVERTISING_INTERVAL          800 //500ms

#define DEFAULT_DISCOVERABLE_MODE             GAP_ADTYPE_FLAGS_GENERAL

The following is the initialization process of WDT. And the process is in SimplePeripheral_taskFxn Funcion.

#define OVERFLOW_TIME 65000000*3/2

static void SimplePeripheral_taskFxn(UArg a0, UArg a1)

{

uint32_t reloadValue;

uint16_t ret = 0;

// Initialize application

SimplePeripheral_init();

Util_startClock(&periodicClock);

Task_sleep(100000/10);

Watchdog_init();

Watchdog_Params_init(&params);

params.resetMode = Watchdog_RESET_ON;

params.callbackFxn = NULL;//(Watchdog_Callback) UserCallbackFxn;

watchdogHandle = Watchdog_open(Board_WATCHDOG0, &params);

Watchdog_setReload(watchdogHandle,OVERFLOW_TIME);

Watchdog_clear(watchdogHandle);

if (watchdogHandle == NULL){

// Error opening Watchdog

//while (1);

SysCtrlSystemReset();

}

//其余省略

///Clear dog

if (events & SBP_PERIODIC_EVT)

{

Util_startClock(&periodicClock);

// Perform periodic application task

//SimplePeripheral_performPeriodicTask();

//省略部分

Watchdog_setReload(watchdogHandle,OVERFLOW_TIME);

//Watchdog_clear(watchdogHandle);

}

///Create a timer in SimplePeripheral_init() funciton

// Create one-shot clocks for internal periodic events.

Util_constructClock(&periodicClock, SimplePeripheral_clockHandler,

SBP_PERIODIC_EVT_PERIOD, 0, false, SBP_PERIODIC_EVT);

宏

#define SBP_PERIODIC_EVT                      Event_Id_00

// How often to perform periodic event (in msec)

#define SBP_PERIODIC_EVT_PERIOD               60000

//The timer start in SimplePeripheral_taskFxn funciton

Util_startClock(&periodicClock);