void Pcm1863_RegsterInit (unsigned char idx)

{

pError = 0;

unsigned char ret=0;

sWrite_i2c (0, 0);                // switch to page 0;

if (idx == PCM1863_I2S_MASTER_IDX)

{

sWrite_i2c (REG_DEV_CLOCK_SET, 0x11);//#define REG_DEV_CLOCK_SET        0x20

sWrite_i2c (REG_DEV_SCK_DIVIDER, 0x03);//#define REG_DEV_SCK_DIVIDER      0x26

sWrite_i2c (REG_DEV_BCK_DIVIDER, 0x3F);//#define REG_DEV_BCK_DIVIDER      0x27

}

else

{

sWrite_i2c (REG_DEV_CLOCK_SET, 0x01);

}

sWrite_i2c (REG_ADC1L_INPUT, RBIT_REG_6_8_DIFF_1);//#define REG_ADC1L_INPUT            0x06

sWrite_i2c (REG_ADC1R_INPUT, RBIT_REG_7_9_DIFF_2);//#define REG_ADC1R_INPUT            0x07

sWrite_i2c (REG_PGA_VAL_CH1_L, 0);   //#define REG_PGA_VAL_CH1_L       0x01

sWrite_i2c (REG_PGA_VAL_CH1_R, 0);  //#define REG_PGA_VAL_CH1_R       0x02

sWrite_i2c (REG_PGA_VAL_CH2_L, 0);   //#define REG_PGA_VAL_CH2_L       0x03

sWrite_i2c (REG_PGA_VAL_CH2_R, 0);  //#define REG_PGA_VAL_CH2_R       0x04

sWrite_i2c (0x00, 0x00);   // page 0

sWrite_i2c (0x70, 0x70);   // is power on mode（0x70）, on sleep mode

sWrite_i2c (0x00, 0xfd);    // page 253

sWrite_i2c (0x14, 0x10);   // Global bias current trim 75%

//====end======//

#if 1

//set threshold of singal (SLEEP TO ACTIVE) -30dB

sWrite_i2c (0x00, 0x01);   // page 1

sWrite_i2c (0x02, 0x2d);

sWrite_i2c (0x04, 0x00);//-62dB

sWrite_i2c (0x05, 0x1a);

sWrite_i2c (0x06, 0x07);

sWrite_i2c (0x01, 0x01);   // excute to write

vTaskDelay (100 / portTICK_RATE_MS);

//DelayMs(100);

sWrite_i2c (0x00, 0x01);   // page 1

sWrite_i2c (0x02, 0x2c);

sWrite_i2c (0x04, 0x00);//-62dB

sWrite_i2c (0x05, 0x1a);

sWrite_i2c (0x06, 0x07);

sWrite_i2c (0x01, 0x01);   // excute to write

vTaskDelay (100 / portTICK_RATE_MS);

//DelayMs(100);

#endif

sWrite_i2c (0x00, 0x00);   // page 0

sWrite_i2c (0x60, 0x01);   // enable ENG Interrup.

sWrite_i2c (0x34, 0x01);   // set time from 1 ~ 30 mins

sWrite_i2c (0x31, 0xFF);

sWrite_i2c (0x36, 0x00);   // NO repeat

sWrite_i2c (0x00, 0x00);   // page 0

sWrite_i2c (0x70, 0x72);// is power on mode, on sleep mode(0X72)

sWrite_i2c (PGA_CONTROL_MAPPING, 0xF0);//#define PGA_CONTROL_MAPPING  0x19

SwI2Cx_BusSel (I2C_BUS_NUM);

if (pError)

{

UART_DEBUGF (PCM1863_DBG, ("PCM1863[%d] init failed\r\n", idx));

}

else

{

//UART_DEBUGF (PCM1863_DBG, ("PCM1863 init finished\r\n"));

}

pcmSleepMode[idx] = true;

return;

}

void ADCInputGainSet (unsigned char inputIndx, unsigned char gainIndx)

{

unsigned char reg;

if(gainIndx<PCM1863_GAININDEX_MIN||gainIndx>PCM1863_GAININDEX_MAX)

return;

if (inputIndx < IDX_PCM1863_NUM)

{

pcmChip_sel (inputIndx);

SwI2Cx_BusSel (PCM1863_I2C_BUS_BASE + (inputIndx / 2));

//range of gain 0 ~ 24; step 0.5dB  -> 0dB ~ +12dB

reg = gainIndx;

//sWrite_i2c (REG_PGA_VAL_CH1_L, reg);

//sWrite_i2c (REG_PGA_VAL_CH1_R, reg);

sWrite_i2c (DPGA_VAL_CH1_L, 0x40|reg); //#define DPGA_VAL_CH1_L   0x0F

sWrite_i2c (DPGA_VAL_CH1_R,  0x40|reg);// #define DPGA_VAL_CH1_R     0x16

sWrite_i2c (DPGA_VAL_CH2_L, 0x40|reg); //#define DPGA_VAL_CH2_L   0x17

sWrite_i2c (DPGA_VAL_CH2_R,  0x40|reg);// #define DPGA_VAL_CH2_R     0x18

SwI2Cx_BusSel (I2C_BUS_NUM);

}