TI TPA3106D1 Audio Power Amplifier EVM   output power test

TPA3106D1EVM is powered with DC 24V 4.5A power supply. The output voltage on 8 Ohm 50 W dummy resistor load is observed with oscilloscope. The power supply is checked, output shown NO notable pulsation under TPA3106D1EVM operation in loaded state.

TEST 1. Input signal (form waveform generator) is 3 kHz. Amplifier gain is set to 36dB.

Corresponding output voltage (oscilloscope probe X10) becomes clamped starting from input voltage 0.8V p-p. Measured output voltage is ~ 44V p-p (~ 29 W).

TEST 2. Input signal (form waveform generator) is 3 kHz. Amplifier gain is set to 20dB.

Corresponding output voltage (oscilloscope probe X10) becomes clamped starting from input voltage 0.8V p-p. Measured output voltage is again ~ 44V p-p (~ 29 W).

The DC current consumption is ~ 1.5 A. The temperature of chip is low enough.