| 2.1 Mode Memory Map — Book 0x8C |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SUB ADDRESS | PAGE | REGISTER NAME | NUMBER OF BYTES/ FORMAT | DEFAULT VALUE | DESCRIPTION |
| CROSS OVER BQS |  |  |  |  |  |
| 0x58 | 0x11 | BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | Cross Over BQ coefficient |
| 0x5C | 0x11 | BQ 1 B1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x60 | 0x11 | BQ 1 B2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| 0x64 | 0x11 | BQ 1 A1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x68 | 0x11 | BQ 1 A2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| 0x6C | 0x11 | BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | Cross Over BQ coefficient |
| 0x70 | 0x11 | BQ 2 B1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x74 | 0x11 | BQ 2 B2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| 0x78 | 0x11 | BQ 2 A1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x7C | 0x11 | BQ 2 A2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| 0x08 | 0x12 | BQ 3 B0 | 4 / 1.31 | 0x7FFFFFFF | Cross Over BQ coefficient |
| 0x0C | 0x12 | BQ 3 B1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x10 | 0x12 | BQ 3 B2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| 0x14 | 0x12 | BQ 3 A1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x18 | 0x12 | BQ 3 A2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| 0x1C | 0x12 | BQ 4 B0 | 4 / 1.31 | 0x7FFFFFFF | Cross Over BQ coefficient |
| 0x20 | 0x12 | BQ 4 B1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x24 | 0x12 | BQ 4 B2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| 0x28 | 0x12 | BQ 4 A1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x2C | 0x12 | BQ 4 A2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| 0x30 | 0x12 | BQ 5 B0 | 4 / 1.31 | 0x7FFFFFFF | Cross Over BQ coefficient |
| 0x34 | 0x12 | BQ 5 B1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x38 | 0x12 | BQ 5 B2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| 0x3C | 0x12 | BQ 5 A1 | 4 / 2.30 | 0x00000000 | Cross Over BQ coefficient |
| 0x40 | 0x12 | BQ 5 A2 | 4 / 1.31 | 0x00000000 | Cross Over BQ coefficient |
| SUB CROSS OVER BQS |  |  |  |  |  |
| 0x44 | 0x12 | CH-Sub BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | Sub Cross Over BQ coefficient |
| 0x48 | 0x12 | CH-Sub BQ 1 B1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x4C | 0x12 | CH-Sub BQ 1 B2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x50 | 0x12 | CH-Sub BQ 1 A1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x54 | 0x12 | CH-Sub BQ 1 A2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x58 | 0x12 | CH-Sub BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | Sub Cross Over BQ coefficient |
| 0x5C | 0x12 | CH-Sub BQ 2 B1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x60 | 0x12 | CH-Sub BQ 2 B2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x64 | 0x12 | CH-Sub BQ 2 A1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x68 | 0x12 | CH-Sub BQ 2 A2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x6C | 0x12 | CH-Sub BQ 3 B0 | 4 / 1.31 | 0x7FFFFFFF | Sub Cross Over BQ coefficient |
| 0x70 | 0x12 | CH-Sub BQ 3 B1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x74 | 0x12 | CH-Sub BQ 3 B2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x78 | 0x12 | CH-Sub BQ 3 A1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x7C | 0x12 | CH-Sub BQ 3 A2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x08 | 0x13 | CH-Sub BQ 4 B0 | 4 / 1.31 | 0x7FFFFFFF | Sub Cross Over BQ coefficient |
| 0x0C | 0x13 | CH-Sub BQ 4 B1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x10 | 0x13 | CH-Sub BQ 4 B2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x14 | 0x13 | CH-Sub BQ 4 A1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x18 | 0x13 | CH-Sub BQ 4 A2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x1C | 0x13 | CH-Sub BQ 5 B0 | 4 / 1.31 | 0x7FFFFFFF | Sub Cross Over BQ coefficient |
| 0x20 | 0x13 | CH-Sub BQ 5 B1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x24 | 0x13 | CH-Sub BQ 5 B2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x28 | 0x13 | CH-Sub BQ 5 A1 | 4 / 2.30 | 0x00000000 | Sub Cross Over BQ coefficient |
| 0x2C | 0x13 | CH-Sub BQ 5 A2 | 4 / 1.31 | 0x00000000 | Sub Cross Over BQ coefficient |
| DRC 1 BQ |  |  |  |  |  |
| 0x30 | 0x13 | BQ B0 | 4 / 1.31 | 0x7FFFFFFF | DRC 1 BQ coefficient |
| 0x34 | 0x13 | BQ B1 | 4 / 2.30 | 0x00000000 | DRC 1 BQ coefficient |
| 0x38 | 0x13 | BQ B2 | 4 / 1.31 | 0x00000000 | DRC 1 BQ coefficient |
| 0x3C | 0x13 | BQ A1 | 4 / 2.30 | 0x00000000 | DRC 1 BQ coefficient |
| 0x40 | 0x13 | BQ A2 | 4 / 1.31 | 0x00000000 | DRC 1 BQ coefficient |
| 0x44 | 0x13 | CH-Sub BQ B0 | 4 / 1.31 | 0x7FFFFFFF | Sub DRC 1 BQ coefficient |
| 0x48 | 0x13 | CH-Sub BQ B1 | 4 / 2.30 | 0x00000000 | Sub DRC 1 BQ coefficient |
| 0x4C | 0x13 | CH-Sub BQ B2 | 4 / 1.31 | 0x00000000 | Sub DRC 1 BQ coefficient |
| 0x50 | 0x13 | CH-Sub BQ A1 | 4 / 2.30 | 0x00000000 | Sub DRC 1 BQ coefficient |
| 0x54 | 0x13 | CH-Sub BQ A2 | 4 / 1.31 | 0x00000000 | Sub DRC 1 BQ coefficient |
| DRC 3 BQ |  |  |  |  |  |
| 0x58 | 0x13 | BQ B0 | 4 / 1.31 | 0x7FFFFFFF | DRC 3 BQ coefficient |
| 0x5C | 0x13 | BQ B1 | 4 / 2.30 | 0x00000000 | DRC 3 BQ coefficient |
| 0x60 | 0x13 | BQ B2 | 4 / 1.31 | 0x00000000 | DRC 3 BQ coefficient |
| 0x64 | 0x13 | BQ A1 | 4 / 2.30 | 0x00000000 | DRC 3 BQ coefficient |
| 0x68 | 0x13 | BQ A2 | 4 / 1.31 | 0x00000000 | DRC 3 BQ coefficient |
| 0x6C | 0x13 | CH-Sub BQ B0 | 4 / 1.31 | 0x7FFFFFFF | Sub DRC 3 BQ coefficient |
| 0x70 | 0x13 | CH-Sub BQ B1 | 4 / 2.30 | 0x00000000 | Sub DRC 3 BQ coefficient |
| 0x74 | 0x13 | CH-Sub BQ B2 | 4 / 1.31 | 0x00000000 | Sub DRC 3 BQ coefficient |
| 0x78 | 0x13 | CH-Sub BQ A1 | 4 / 2.30 | 0x00000000 | Sub DRC 3 BQ coefficient |
| 0x7C | 0x13 | CH-Sub BQ A2 | 4 / 1.31 | 0x00000000 | Sub DRC 3 BQ coefficient |
| DRC 2 BQ |  |  |  |  |  |
| 0x08 | 0x14 | BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | DRC 2 BQ coefficient |
| 0x0C | 0x14 | BQ 1 B1 | 4 / 2.30 | 0x00000000 | DRC 2 BQ coefficient |
| 0x10 | 0x14 | BQ 1 B2 | 4 / 1.31 | 0x00000000 | DRC 2 BQ coefficient |
| 0x14 | 0x14 | BQ 1 A1 | 4 / 2.30 | 0x00000000 | DRC 2 BQ coefficient |
| 0x18 | 0x14 | BQ 1 A2 | 4 / 1.31 | 0x00000000 | DRC 2 BQ coefficient |
| 0x1C | 0x14 | BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | DRC 2 BQ coefficient |
| 0x20 | 0x14 | BQ 2 B1 | 4 / 2.30 | 0x00000000 | DRC 2 BQ coefficient |
| 0x24 | 0x14 | BQ 2 B2 | 4 / 1.31 | 0x00000000 | DRC 2 BQ coefficient |
| 0x28 | 0x14 | BQ 2 A1 | 4 / 2.30 | 0x00000000 | DRC 2 BQ coefficient |
| 0x2C | 0x14 | BQ 2 A2 | 4 / 1.31 | 0x00000000 | DRC 2 BQ coefficient |
| 0x30 | 0x14 | CH-Sub BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | Sub DRC 2 BQ coefficient |
| 0x34 | 0x14 | CH-Sub BQ 1 B1 | 4 / 2.30 | 0x00000000 | Sub DRC 2 BQ coefficient |
| 0x38 | 0x14 | CH-Sub BQ 1 B2 | 4 / 1.31 | 0x00000000 | Sub DRC 2 BQ coefficient |
| 0x3C | 0x14 | CH-Sub BQ 1 A1 | 4 / 2.30 | 0x00000000 | Sub DRC 2 BQ coefficient |
| 0x40 | 0x14 | CH-Sub BQ 1 A2 | 4 / 1.31 | 0x00000000 | Sub DRC 2 BQ coefficient |
| 0x44 | 0x14 | CH-Sub BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | Sub DRC 2 BQ coefficient |
| 0x48 | 0x14 | CH-Sub BQ 2 B1 | 4 / 2.30 | 0x00000000 | Sub DRC 2 BQ coefficient |
| 0x4C | 0x14 | CH-Sub BQ 2 B2 | 4 / 1.31 | 0x00000000 | Sub DRC 2 BQ coefficient |
| 0x50 | 0x14 | CH-Sub BQ 2 A1 | 4 / 2.30 | 0x00000000 | Sub DRC 2 BQ coefficient |
| 0x54 | 0x14 | CH-Sub BQ 2 A2 | 4 / 1.31 | 0x00000000 | Sub DRC 2 BQ coefficient |
| DPEQ SENSE BQ |  |  |  |  |  |
| 0x58 | 0x14 | BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | DPEQ sense BQ coefficient |
| 0x5C | 0x14 | BQ 1 B1 | 4 / 2.30 | 0x00000000 | DPEQ sense BQ coefficient |
| 0x60 | 0x14 | BQ 1 B2 | 4 / 1.31 | 0x00000000 | DPEQ sense BQ coefficient |
| 0x64 | 0x14 | BQ 1 A1 | 4 / 2.30 | 0x00000000 | DPEQ sense BQ coefficient |
| 0x68 | 0x14 | BQ 1 A2 | 4 / 1.31 | 0x00000000 | DPEQ sense BQ coefficient |
| 0x6C | 0x14 | BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | DPEQ sense BQ coefficient |
| 0x70 | 0x14 | BQ 2 B1 | 4 / 2.30 | 0x00000000 | DPEQ sense BQ coefficient |
| 0x74 | 0x14 | BQ 2 B2 | 4 / 1.31 | 0x00000000 | DPEQ sense BQ coefficient |
| 0x78 | 0x14 | BQ 2 A1 | 4 / 2.30 | 0x00000000 | DPEQ sense BQ coefficient |
| 0x7C | 0x14 | BQ 2 A2 | 4 / 1.31 | 0x00000000 | DPEQ sense BQ coefficient |
| DPEQ LOW LEVEL PATH BQ |  |  |  |  |  |
| 0x08 | 0x15 | BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | DPEQ low BQ coefficient |
| 0x0C | 0x15 | BQ 1 B1 | 4 / 2.30 | 0x00000000 | DPEQ low BQ coefficient |
| 0x10 | 0x15 | BQ 1 B2 | 4 / 1.31 | 0x00000000 | DPEQ low BQ coefficient |
| 0x14 | 0x15 | BQ 1 A1 | 4 / 2.30 | 0x00000000 | DPEQ low BQ coefficient |
| 0x18 | 0x15 | BQ 1 A2 | 4 / 1.31 | 0x00000000 | DPEQ low BQ coefficient |
| 0x1C | 0x15 | BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | DPEQ low BQ coefficient |
| 0x20 | 0x15 | BQ 2 B1 | 4 / 2.30 | 0x00000000 | DPEQ low BQ coefficient |
| 0x24 | 0x15 | BQ 2 B2 | 4 / 1.31 | 0x00000000 | DPEQ low BQ coefficient |
| 0x28 | 0x15 | BQ 2 A1 | 4 / 2.30 | 0x00000000 | DPEQ low BQ coefficient |
| 0x2C | 0x15 | BQ 2 A2 | 4 / 1.31 | 0x00000000 | DPEQ low BQ coefficient |
| DPEQ HIGH LEVEL PATH BQ |  |  |  |  |  |
| 0x30 | 0x15 | BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | DPEQ high BQ coefficient |
| 0x34 | 0x15 | BQ 1 B1 | 4 / 2.30 | 0x00000000 | DPEQ high BQ coefficient |
| 0x38 | 0x15 | BQ 1 B2 | 4 / 1.31 | 0x00000000 | DPEQ high BQ coefficient |
| 0x3C | 0x15 | BQ 1 A1 | 4 / 2.30 | 0x00000000 | DPEQ high BQ coefficient |
| 0x40 | 0x15 | BQ 1 A2 | 4 / 1.31 | 0x00000000 | DPEQ high BQ coefficient |
| 0x44 | 0x15 | BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | DPEQ high BQ coefficient |
| 0x48 | 0x15 | BQ 2 B1 | 4 / 2.30 | 0x00000000 | DPEQ high BQ coefficient |
| 0x4C | 0x15 | BQ 2 B2 | 4 / 1.31 | 0x00000000 | DPEQ high BQ coefficient |
| 0x50 | 0x15 | BQ 2 A1 | 4 / 2.30 | 0x00000000 | DPEQ high BQ coefficient |
| 0x54 | 0x15 | BQ 2 A2 | 4 / 1.31 | 0x00000000 | DPEQ high BQ coefficient |
| EQ LEFT 14 BQS |  |  |  |  |  |
| 0x58 | 0x15 | CH -L BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x5C | 0x15 | CH -L BQ 1 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x60 | 0x15 | CH -L BQ 1 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x64 | 0x15 | CH -L BQ 1 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x68 | 0x15 | CH -L BQ 1 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x6C | 0x15 | CH -L BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x70 | 0x15 | CH -L BQ 2 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x74 | 0x15 | CH -L BQ 2 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x78 | 0x15 | CH -L BQ 2 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x7C | 0x15 | CH -L BQ 2 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x08 | 0x16 | CH -L BQ 3 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x0C | 0x16 | CH -L BQ 3 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x10 | 0x16 | CH -L BQ 3 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x14 | 0x16 | CH -L BQ 3 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x18 | 0x16 | CH -L BQ 3 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x1C | 0x16 | CH -L BQ 4 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x20 | 0x16 | CH -L BQ 4 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x24 | 0x16 | CH -L BQ 4 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x28 | 0x16 | CH -L BQ 4 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x2C | 0x16 | CH -L BQ 4 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x30 | 0x16 | CH -L BQ 5 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x34 | 0x16 | CH -L BQ 5 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x38 | 0x16 | CH -L BQ 5 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x3C | 0x16 | CH -L BQ 5 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x40 | 0x16 | CH -L BQ 5 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x44 | 0x16 | CH -L BQ 6 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x48 | 0x16 | CH -L BQ 6 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x4C | 0x16 | CH -L BQ 6 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x50 | 0x16 | CH -L BQ 6 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x54 | 0x16 | CH -LBQ 6 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x58 | 0x16 | CH -L BQ 7 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x5C | 0x16 | CH -L BQ 7 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x60 | 0x16 | CH -L BQ 7 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x64 | 0x16 | CH -L BQ 7 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x68 | 0x16 | CH -L BQ 7 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x6C | 0x16 | CH -L BQ 8 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x70 | 0x16 | CH -L BQ 8 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x74 | 0x16 | CH -L BQ 8 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x78 | 0x16 | CH -L BQ 8 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x7C | 0x16 | CH -L BQ 8 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x08 | 0x17 | CH -L BQ 9 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x0C | 0x17 | CH -L BQ 9 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x10 | 0x17 | CH -L BQ 9 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x14 | 0x17 | CH -L BQ 9 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x18 | 0x17 | CH -L BQ 9 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x1C | 0x17 | CH -L BQ 10 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x20 | 0x17 | CH -L BQ 10 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x24 | 0x17 | CH -L BQ 10 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x28 | 0x17 | CH -L BQ 10 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x2C | 0x17 | CH -L BQ 10 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x30 | 0x17 | CH -L BQ 11 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x34 | 0x17 | CH -L BQ 11 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x38 | 0x17 | CH -L BQ 11 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x3C | 0x17 | CH -L BQ 11 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x40 | 0x17 | CH -L BQ 11 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x44 | 0x17 | CH -L BQ 12 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x48 | 0x17 | CH -L BQ 12 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x4C | 0x17 | CH -L BQ 12 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x50 | 0x17 | CH -L BQ 12 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x54 | 0x17 | CH -L BQ 12 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x58 | 0x17 | CH -L BQ 13 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x5C | 0x17 | CH -L BQ 13 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x60 | 0x17 | CH -L BQ 13 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x64 | 0x17 | CH -L BQ 13 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x68 | 0x17 | CH -L BQ 13 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x6C | 0x17 | CH -L BQ 14 B0 | 4 / 1.31 | 0x7FFFFFFF | Left BQ coefficient |
| 0x70 | 0x17 | CH -L BQ 14 B1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x74 | 0x17 | CH -L BQ 14 B2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| 0x78 | 0x17 | CH -L BQ 14 A1 | 4 / 2.30 | 0x00000000 | Left BQ coefficient |
| 0x7C | 0x17 | CH -L BQ 14 A2 | 4 / 1.31 | 0x00000000 | Left BQ coefficient |
| LEFT INTGAIN BQ |  |  |  |  |  |
| 0x08 | 0x18 | CH -L BQ 15 B0 | 4 / 5.27 | 0x08000000 | Left gain scale BQ coefficient |
| 0x0C | 0x18 | CH -L BQ 15 B1 | 4 / 6.26 | 0x00000000 | Left gain scale BQ coefficient |
| 0x10 | 0x18 | CH -L BQ 15 B2 | 4 / 5.27 | 0x00000000 | Left gain scale BQ coefficient |
| 0x14 | 0x18 | CH -L BQ 15 A1 | 4 / 2.30 | 0x00000000 | Left gain scale BQ coefficient |
| 0x18 | 0x18 | CH -L BQ 15 A2 | 4 / 1.31 | 0x00000000 | Left gain scale BQ coefficient |
| EQ RIGHT 14 BQS |  |  |  |  |  |
| 0x1C | 0x18 | CH -R BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x20 | 0x18 | CH -R BQ 1 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x24 | 0x18 | CH -R BQ 1 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x28 | 0x18 | CH -R BQ 1 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x2C | 0x18 | CH -R BQ 1 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x30 | 0x18 | CH -R BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x34 | 0x18 | CH -R BQ 2 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x38 | 0x18 | CH -R BQ 2 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x3C | 0x18 | CH -R BQ 2 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x40 | 0x18 | CH -R BQ 2 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x44 | 0x18 | CH -R BQ 3 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x48 | 0x18 | CH -R BQ 3 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x4C | 0x18 | CH -R BQ 3 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x50 | 0x18 | CH -R BQ 3 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x54 | 0x18 | CH -R BQ 3 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x58 | 0x18 | CH -R BQ 4 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x5C | 0x18 | CH -R BQ 4 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x60 | 0x18 | CH -R BQ 4 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x64 | 0x18 | CH -R BQ 4 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x68 | 0x18 | CH -R BQ 4 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x6C | 0x18 | CH -R BQ 5 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x70 | 0x18 | CH -R BQ 5 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x74 | 0x18 | CH -R BQ 5 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x78 | 0x18 | CH -R BQ 5 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x7C | 0x18 | CH -R BQ 5 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x08 | 0x19 | CH -R BQ 6 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x0C | 0x19 | CH -R BQ 6 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x10 | 0x19 | CH -R BQ 6 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x14 | 0x19 | CH -R BQ 6 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x18 | 0x19 | CH -R BQ 6 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x1C | 0x19 | CH -R BQ 7 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x20 | 0x19 | CH -R BQ 7 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x24 | 0x19 | CH -R BQ 7 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x28 | 0x19 | CH -R BQ 7 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x2C | 0x19 | CH -R BQ 7 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x30 | 0x19 | CH -R BQ 8 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x34 | 0x19 | CH -R BQ 8 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x38 | 0x19 | CH -R BQ 8 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x3C | 0x19 | CH -R BQ 8 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x40 | 0x19 | CH -R BQ 8 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x44 | 0x19 | CH -R BQ 9 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x48 | 0x19 | CH -R BQ 9 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x4C | 0x19 | CH -R BQ 9 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x50 | 0x19 | CH -R BQ 9 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x54 | 0x19 | CH -R BQ 9 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x58 | 0x19 | CH -R BQ 10 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x5C | 0x19 | CH -R BQ 10 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x60 | 0x19 | CH -R BQ 10 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x64 | 0x19 | CH -R BQ 10 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x68 | 0x19 | CH -R BQ 10 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x6C | 0x19 | CH -R BQ 11 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x70 | 0x19 | CH -R BQ 11 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x74 | 0x19 | CH -R BQ 11 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x78 | 0x19 | CH -R BQ 11 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x7C | 0x19 | CH -R BQ 11 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x08 | 0x1A | CH -R BQ 12 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x0C | 0x1A | CH -R BQ 12 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x10 | 0x1A | CH -R BQ 12 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x14 | 0x1A | CH -R BQ 12 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x18 | 0x1A | CH -R BQ 12 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x1C | 0x1A | CH -R BQ 13 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x20 | 0x1A | CH -R BQ 13 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x24 | 0x1A | CH -R BQ 13 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x28 | 0x1A | CH -R BQ 13 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x2C | 0x1A | CH -R BQ 13 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x30 | 0x1A | CH -R BQ 14 B0 | 4 / 1.31 | 0x7FFFFFFF | Right BQ coefficient |
| 0x34 | 0x1A | CH -R BQ 14 B1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x38 | 0x1A | CH -R BQ 14 B2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| 0x3C | 0x1A | CH -R BQ 14 A1 | 4 / 2.30 | 0x00000000 | Right BQ coefficient |
| 0x40 | 0x1A | CH -R BQ 14 A2 | 4 / 1.31 | 0x00000000 | Right BQ coefficient |
| RIGHT INTGAIN BQ |  |  |  |  |  |
| 0x44 | 0x1A | CH -R BQ 15 B0 | 4 / 5.27 | 0x08000000 | Right gain scale BQ coefficient |
| 0x48 | 0x1A | CH -R BQ 15 B1 | 4 / 6.26 | 0x00000000 | Right gain scale BQ coefficient |
| 0x4C | 0x1A | CH -R BQ 15 B2 | 4 / 5.27 | 0x00000000 | Right gain scale BQ coefficient |
| 0x50 | 0x1A | CH -R BQ 15 A1 | 4 / 2.30 | 0x00000000 | Right gain scale BQ coefficient |
| 0x54 | 0x1A | CH -R BQ 15 A2 | 4 / 1.31 | 0x00000000 | Right gain scale BQ coefficient |
| VOLUME ALPHA FILTER |  |  |  |  |  |
| 0x58 | 0x1A | Softening Filter Alpha | 4 / 1.31 | 0x00A7264A | Volume time constant |
| PHASE OPTIMIZER |  |  |  |  |  |
| 0x60 | 0x1A | Delay Left | 4 / 32.0 | 0x00000000 | Left Channel Phase Optimizer |
| 0x64 | 0x1A | Delay Right | 4 / 32.0 | 0x00000000 | Right Channel Phase Optimizer |
| 0x68 | 0x1A | Delay Sub | 4 / 32.0 | 0x00000000 | Sub Channel Phase Optimizer |
| MAIN DRC 1 |  |  |  |  |  |
| 0x6C | 0x1A | DRC1 Energy | 4 / 1.31 | 0x7FFFFFFF | DRC1 Energy Time constant |
| 0x70 | 0x1A | DRC1 Attack | 4 / 1.31 | 0x7FFFFFFF | DRC1 Attack Time constant |
| 0x74 | 0x1A | DRC1 Decay | 4 / 1.31 | 0x7FFFFFFF | DRC1 Decay Time constant |
| 0x78 | 0x1A | K0_1 | 4 / 9.23 | 0x00000000 | DRC1 Region 1 Slope (comp/Exp) |
| 0x7C | 0x1A | K1_1 | 4 / 9.23 | 0x00000000 | DRC1 Region 2 Slope (comp/Exp) |
| 0x08 | 0x1B | K2_1 | 4 / 9.23 | 0x00000000 | DRC1 Region 3 Slope (comp/Exp) |
| 0x0C | 0x1B | T1_1 | 4 / 9.23 | 0xE7000000 | DRC1 Threshold 1 |
| 0x10 | 0x1B | T2_1 | 4 / 9.23 | 0xFE800000 | DRC1 Threshold 2 |
| 0x14 | 0x1B | off1_1 | 4 / 9.23 | 0x00000000 | DRC1 Offset 1 |
| 0x18 | 0x1B | off2_1 | 4 / 9.23 | 0x00000000 | DRC1 Offset 2 |
| MAIN DRC 2 |  |  |  |  |  |
| 0x1C | 0x1B | DRC2 Energy | 4 / 1.31 | 0x7FFFFFFF | DRC2 Energy Time constant |
| 0x20 | 0x1B | DRC2 Attack | 4 / 1.31 | 0x7FFFFFFF | DRC2 Attack Time constant |
| 0x24 | 0x1B | DRC2 Decay | 4 / 1.31 | 0x7FFFFFFF | DRC2 Decay Time constant |
| 0x28 | 0x1B | k0_2 | 4 / 9.23 | 0x00000000 | DRC2 Region 1 Slope (comp/Exp) |
| 0x2C | 0x1B | k1_2 | 4 / 9.23 | 0x00000000 | DRC2 Region 2 Slope (comp/Exp) |
| 0x30 | 0x1B | k2_2 | 4 / 9.23 | 0x00000000 | DRC2 Region 3 Slope (comp/Exp) |
| 0x34 | 0x1B | t1_2 | 4 / 9.23 | 0xE7000000 | DRC2 Threshold 1 |
| 0x38 | 0x1B | t2_2 | 4 / 9.23 | 0xFE800000 | DRC2 Threshold 2 |
| 0x3C | 0x1B | off1_2 | 4 / 9.23 | 0x00000000 | DRC2 Offset 1 |
| 0x40 | 0x1B | off2_2 | 4 / 9.23 | 0x00000000 | DRC2 Offset 2 |
| MAIN DRC 3 |  |  |  |  |  |
| 0x44 | 0x1B | DRC3 Energy | 4 / 1.31 | 0x7FFFFFFF | DRC3 Energy Time constant |
| 0x48 | 0x1B | DRC3 Attack | 4 / 1.31 | 0x7FFFFFFF | DRC3 Attack Time constant |
| 0x4C | 0x1B | DRC3 Decay | 4 / 1.31 | 0x7FFFFFFF | DRC3 Decay Time constant |
| 0x50 | 0x1B | k0_3 | 4 / 9.23 | 0x00000000 | DRC3 Region 1 Slope (comp/Exp) |
| 0x54 | 0x1B | k1_3 | 4 / 9.23 | 0x00000000 | DRC3 Region 2 Slope (comp/Exp) |
| 0x58 | 0x1B | k1_3 | 4 / 9.23 | 0x00000000 | DRC3 Region 3 Slope (comp/Exp) |
| 0x5C | 0x1B | t1_3 | 4 / 9.23 | 0xE7000000 | DRC3 Threshold 1 |
| 0x60 | 0x1B | t2_3 | 4 / 9.23 | 0xFE800000 | DRC3 Threshold 2 |
| 0x64 | 0x1B | off1_3 | 4 / 9.23 | 0x00000000 | DRC3 Offset 1 |
| 0x68 | 0x1B | off2_3 | 4 / 9.23 | 0x00000000 | DRC3 Offset 2 |
| SUB DRC 1 |  |  |  |  |  |
| 0x6C | 0x1B | CH-Sub DRC1 Energy | 4 / 1.31 | 0x7FFFFFFF | Sub Channel DRC1 Energy Time constant |
| 0x70 | 0x1B | CH-Sub DRC1 Attack | 4 / 1.31 | 0x7FFFFFFF | Sub Channel DRC1 Attack Time constant |
| 0x74 | 0x1B | CH-Sub DRC1 Decay | 4 / 1.31 | 0x7FFFFFFF | Sub Channel DRC1 Decay Time constant |
| 0x78 | 0x1B | CH-Sub K0_1 | 4 / 9.23 | 0x00000000 | Sub Channel DRC1 Region 1 Slope (comp/Exp) |
| 0x7C | 0x1B | CH-Sub K1_1 | 4 / 9.23 | 0x00000000 | Sub Channel DRC1 Region 2 Slope (comp/Exp) |
| 0x08 | 0x1C | CH-Sub K2_1 | 4 / 9.23 | 0x00000000 | Sub Channel DRC1 Region 3 Slope (comp/Exp) |
| 0x0C | 0x1C | CH-Sub T1_1 | 4 / 9.23 | 0xE7000000 | Sub Channel DRC1 Threshold 1 |
| 0x10 | 0x1C | CH-Sub T2_1 | 4 / 9.23 | 0xFE800000 | Sub Channel DRC1 Threshold 2 |
| 0x14 | 0x1C | CH-Sub off1_1 | 4 / 9.23 | 0x00000000 | Sub Channel DRC1 Offset 1 |
| 0x18 | 0x1C | CH-Sub off2_1 | 4 / 9.23 | 0x00000000 | Sub Channel DRC1 Offset 2 |
| SUB DRC 2 |  |  |  |  |  |
| 0x1C | 0x1C | CH-Sub DRC2 Energy | 4 / 1.31 | 0x7FFFFFFF | Sub Channel DRC2 Energy Time constant |
| 0x20 | 0x1C | CH-Sub DRC2 Attack | 4 / 1.31 | 0x7FFFFFFF | Sub Channel DRC2 Attack Time constant |
| 0x24 | 0x1C | CH-Sub DRC2 Decay | 4 / 1.31 | 0x7FFFFFFF | Sub Channel DRC2 Decay Time constant |
| 0x28 | 0x1C | CH-Sub k0_2 | 4 / 9.23 | 0x00000000 | Sub Channel DRC2 Region 1 Slope (comp/Exp) |
| 0x2C | 0x1C | CH-Sub k1_2 | 4 / 9.23 | 0x00000000 | Sub Channel DRC2 Region 2 Slope (comp/Exp) |
| 0x30 | 0x1C | CH-Sub k2_2 | 4 / 9.23 | 0x00000000 | Sub Channel DRC2 Region 3 Slope (comp/Exp) |
| 0x34 | 0x1C | CH-Sub t1_2 | 4 / 9.23 | 0xE7000000 | Sub Channel DRC2 Threshold 1 |
| 0x38 | 0x1C | CH-Sub t2_2 | 4 / 9.23 | 0xFE800000 | Sub Channel DRC2 Threshold 2 |
| 0x3C | 0x1C | CH-Sub off1_2 | 4 / 9.23 | 0x00000000 | Sub Channel DRC2 Offset 1 |
| 0x40 | 0x1C | CH-Sub off2_2 | 4 / 9.23 | 0x00000000 | Sub Channel DRC2 Offset 2 |
| SUB DRC 3 |  |  |  |  |  |
| 0x44 | 0x1C | CH-Sub DRC3 Energy | 4 / 1.31 | 0x7FFFFFFF | Sub Channel DRC3 Energy Time constant |
| 0x48 | 0x1C | CH-Sub DRC3 Attack | 4 / 1.31 | 0x7FFFFFFF | Sub Channel DRC3 Attack Time constant |
| 0x4C | 0x1C | CH-Sub DRC3 Decay | 4 / 1.31 | 0x7FFFFFFF | Sub Channel DRC3 Decay Time constant |
| 0x50 | 0x1C | CH-Sub k0_3 | 4 / 9.23 | 0x00000000 | Sub Channel DRC3 Region 1 Slope (comp/Exp) |
| 0x54 | 0x1C | CH-Sub k1_3 | 4 / 9.23 | 0x00000000 | Sub Channel DRC1 Region 2 Slope (comp/Exp) |
| 0x58 | 0x1C | CH-Sub k1_3 | 4 / 9.23 | 0x00000000 | Sub Channel DRC3 Region 3 Slope (comp/Exp) |
| 0x5C | 0x1C | CH-Sub t1_3 | 4 / 9.23 | 0xE7000000 | Sub Channel DRC3 Threshold 1 |
| 0x60 | 0x1C | CH-Sub t2_3 | 4 / 9.23 | 0xFE800000 | Sub Channel DRC3 Threshold 2 |
| 0x64 | 0x1C | CH-Sub off1_3 | 4 / 9.23 | 0x00000000 | Sub Channel DRC3 Offset 1 |
| 0x68 | 0x1C | CH-Sub off2_3 | 4 / 9.23 | 0x00000000 | Sub Channel DRC3 Offset 2 |
| SPATIALIZER BQS |  |  |  |  |  |
| 0x6C | 0x1C | BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | Spatializer BQ coefficient |
| 0x70 | 0x1C | BQ 1 B1 | 4 / 2.30 | 0x00000000 | Spatializer BQ coefficient |
| 0x74 | 0x1C | BQ 1 B2 | 4 / 1.31 | 0x00000000 | Spatializer BQ coefficient |
| 0x78 | 0x1C | BQ 1 A1 | 4 / 2.30 | 0x00000000 | Spatializer BQ coefficient |
| 0x7C | 0x1C | BQ 1 A2 | 4 / 1.31 | 0x00000000 | Spatializer BQ coefficient |
| 0x08 | 0x1D | BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | Spatializer BQ coefficient |
| 0x0C | 0x1D | BQ 2 B1 | 4 / 2.30 | 0x00000000 | Spatializer BQ coefficient |
| 0x10 | 0x1D | BQ 2 B2 | 4 / 1.31 | 0x00000000 | Spatializer BQ coefficient |
| 0x14 | 0x1D | BQ 2 A1 | 4 / 2.30 | 0x00000000 | Spatializer BQ coefficient |
| 0x18 | 0x1D | BQ 2 A2 | 4 / 1.31 | 0x00000000 | Spatializer BQ coefficient |
| DPEQ CONTROL |  |  |  |  |  |
| 0x1C | 0x1D | Alpha | 4 / 1.31 | 0x02DEAD00 | DPEQ Sense Energy Time constant |
| 0x20 | 0x1D | Gain | 4 / 1.31 | 0x74013901 | DPEQ Threshold Gain |
| 0x24 | 0x1D | Offset | 4 / 1.31 | 0x0020C49B | DPEQ Threshold Offset |
| SUB CHANNEL CONTROL |  |  |  |  |  |
| 0x28 | 0x1D | No Sub | 4 / 32.0 | 0x00000000 | Sub Channel Flag |
| THD CLIPPER |  |  |  |  |  |
| 0x2C | 0x1D | CH-LR THD Boost | 4 / 9.23 | 0x00800000 | THD LR Channel prescale coefficient |
| 0x30 | 0x1D | CH-Sub THD Boost | 4 / 9.23 | 0x00800000 | THD Sub Channel prescale coefficient |
| 0x34 | 0x1D | CH-L Fine Volume | 4 / 2.30 | 0x3FFFFFFF | THD L Channel postscale coefficient |
| 0x38 | 0x1D | CH-R Fine Volume | 4 / 2.30 | 0x3FFFFFFF | THD R Channel postscale coefficient |
| 0x3C | 0x1D | CH-Sub Fine Volume | 4 / 2.30 | 0x3FFFFFFF | THD Sub Channel postscale coefficient |
| DRC MIXER GAIN |  |  |  |  |  |
| 0x40 | 0x1D | DRC 1 Mixer Gain | 4 / 9.23 | 0x00800000 | DRC 1 Mixer Gain coefficient |
| 0x44 | 0x1D | DRC 2 Mixer Gain | 4 / 9.23 | 0x00000000 | DRC 2 Mixer Gain coefficient |
| 0x48 | 0x1D | DRC 3 Mixer Gain | 4 / 9.23 | 0x00000000 | DRC 3 Mixer Gain coefficient |
| 0x4C | 0x1D | CH-Sub DRC 1 Mixer Gain | 4 / 9.23 | 0x00800000 | CH-Sub DRC 1 Mixer Gain coefficient |
| 0x50 | 0x1D | CH-Sub DRC 2 Mixer Gain | 4 / 9.23 | 0x00000000 | CH-Sub DRC 2 Mixer Gain coefficient |
| 0x54 | 0x1D | CH-Sub DRC 3 Mixer Gain | 4 / 9.23 | 0x00000000 | CH-Sub DRC 3 Mixer Gain coefficient |
| GANG EQ |  |  |  |  |  |
| 0x58 | 0x1D | Gang LR EQ | 4 / 32.0 | 0x00000000 | Gang Left and Right EQ flag |
| INPUT MIXER |  |  |  |  |  |
| 0x5C | 0x1D | Left to Left | 4 / 9.23 | 0x00800000 | Left Channel Mixer Left Input Gain |
| 0x60 | 0x1D | Right to Left | 4 / 9.23 | 0x00000000 | Left Channel Mixer Right Input Gain |
| 0x64 | 0x1D | Left to Right | 4 / 9.23 | 0x00000000 | Right Channel Mixer Left Input Gain |
| 0x68 | 0x1D | Right to Right | 4 / 9.23 | 0x00800000 | Right Channel Mixer Right Input Gain |
| MIX/GAIN ADJUST |  |  |  |  |  |
| 0x6C | 0x1D | Left to Sub | 4 / 9.23 | 0x00000000 | Sub Channel Mixer Left Input Gain |
| 0x70 | 0x1D | Right to Sub | 4 / 9.23 | 0x00000000 | Sub Channel Mixer Right Input Gain |
| 0x74 | 0x1D | Sub Mix ScratchL | 4 / 9.23 | 0x00800000 | Sub Channel Mixer ScratchL Input Gain |
| 0x78 | 0x1D | Sub Mix ScratchR | 4 / 9.23 | 0x00000000 | Sub Channel Mixer ScratchR Input Gain |
| 0x7C | 0x1D | Bass Mono Left | 4 / 9.23 | 0x00800000 | Mono Left Input Gain |
| 0x08 | 0x1E | Bass Mono Right | 4 / 9.23 | 0x00800000 | Mono Right Input Gain |
| 0x0C | 0x1E | Bass Mono Sub | 4 / 9.23 | 0x00000000 | Mono Sub Input Gain |
| SPATIALIZER LEVEL |  |  |  |  |  |
| 0x10 | 0x1E | Spatializer Level | 4 / 9.23 | 0x00000000 | Spatializer Level coefficient |
| OUTPUT CROSS BAR |  |  |  |  |  |
| 0x14 | 0x1E | Digital Left from Left | 4 / 9.23 | 0x00800000 | I2S Left output gain from left |
| 0x18 | 0x1E | Digital Left from Right | 4 / 9.23 | 0x00000000 | I2S Left output gain from Right |
| 0x1C | 0x1E | Digital Left from Sub | 4 / 9.23 | 0x00000000 | I2S Left output gain from Sub |
| 0x20 | 0x1E | Digital Right from Left | 4 / 9.23 | 0x00000000 | I2S Right output gain from left |
| 0x24 | 0x1E | Digital Right from Right | 4 / 9.23 | 0x00800000 | I2S Right output gain from Right |
| 0x28 | 0x1E | Digital Right from Sub | 4 / 9.23 | 0x00000000 | I2S Right output gain from Sub |
| 0x2C | 0x1E | Analog Left from Left | 4 / 9.23 | 0x00800000 | Analog Left output gain from left |
| 0x30 | 0x1E | Analog Left from Right | 4 / 9.23 | 0x00000000 | Analog Left output gain from Right |
| 0x34 | 0x1E | Analog Left from Sub | 4 / 9.23 | 0x00000000 | Analog Left output gain from Sub |
| 0x38 | 0x1E | Analog Right from Left | 4 / 9.23 | 0x00000000 | Analog Right output gain from left |
| 0x3C | 0x1E | Analog Right from Right | 4 / 9.23 | 0x00800000 | Analog Right output gain from Right |
| 0x40 | 0x1E | Analog Right from Sub | 4 / 9.23 | 0x00000000 | Analog Right output gain from Sub |
| VOLUME CONTROL |  |  |  |  |  |
| 0x44 | 0x1E | CH-L Volume | 4 / 9.23 | 0x00800000 | Left Channel volume coefficient |
| 0x48 | 0x1E | CH-R Volume | 4 / 9.23 | 0x00800000 | Right Channel volume coefficient |
| 0x4C | 0x1E | CH-Sub Volume | 4 / 9.23 | 0x00800000 | Sub Channel volume coefficient |
| DPEQ GAIN SCALE |  |  |  |  |  |
| 0x50 | 0x1E | DPEQ Sense Scale | 4 / 2.30 | 0x40000000 | DPEQ Sense Input Gain |
| 0x54 | 0x1E | DPEQ Low Scale | 4 / 6.26 | 0x04000000 | DPEQ Low Input Gain |
| 0x58 | 0x1E | DPEQ High Scale | 4 / 6.26 | 0x04000000 | DPEQ High Input Gain |
| THERMAL PROTECT |  |  |  |  |  |
| 0x5C | 0x1E | Thermal | 4 / - | 0x0020C49C | Generated by GUI |
| 0x60 | 0x1E | Thermal | 4 / - | 0x0020C49C | Generated by GUI |
| 0x64 | 0x1E | Thermal | 4 / - | 0x0020C49C | Generated by GUI |
| 0x68 | 0x1E | Thermal | 4 / - | 0x0020C49C | Generated by GUI |
| AGL |  |  |  |  |  |
| 0x6C | 0x1E | CH-Sub Attack Threshold | 4 / 1.31 | 0x40000000 | Sub Channel Threshold linear |
| 0x70 | 0x1E | CH-Sub Softening Filter Alpha | 4 / 1.31 | 0x06153BD1 | Sub Channel AGL Alpha Time constant |
| 0x74 | 0x1E | CH-Sub Attack Rate | 4 / 1.31 | 0x0001B4E8 | Sub Channel AGL Attack Time constant |
| 0x78 | 0x1E | CH-Sub AGL Enable | 4 / 1.31 | 0x40000000 | Sub Channel AGL Enable flag |
| 0x08 | 0x1F | CH-Sub Softening Filter Omega | 4 / 1.31 | 0x79EAC42F | Sub Channel AGL Omega Time constant |
| 0x0C | 0x1F | CH-Sub Release Rate | 4 / 1.31 | 0x00002BB1 | Sub Channel AGL Release Time constant |
| 0x14 | 0x1F | Attack Threshold | 4 / 1.31 | 0x40000000 | Threshold linear |
| 0x18 | 0x1F | Softening Filter Alpha | 4 / 1.31 | 0x06153BD1 | AGL Alpha Time constant |
| 0x1C | 0x1F | Attack Rate | 4 / 1.31 | 0x0001B4E8 | AGL Attack Time constant |
| 0x20 | 0x1F | AGL Enable | 4 / 1.31 | 0x40000000 | AGL Enable flag |
| 0x28 | 0x1F | Softening Filter Omega | 4 / 1.31 | 0x79EAC42F | AGL Omega Time constant |
| 0x2C | 0x1F | Release Rate | 4 / 1.31 | 0x00002BB1 | AGL Release Time constant |
| SWAP FLAG |  |  |  |  |  |
| 0x64 | 0x33 | DSP Memory Update | 4 / 32.0 | 0x00000000 | Swap flag |
| RAM |  |  |  |  |  |
| THERMAL PROTECT |  |  |  |  |  |
| 0x34 | 0x1F | Thermal | 4 / - | 0x00A7264A | Generated by GUI |
| 0x38 | 0x1F | Thermal | 4 / - | 0x00A7264A | Generated by GUI |
| MORPHING CONTROL |  |  |  |  |  |
| 0x3C | 0x1F | Morphing Energy | 4 / 1.31 | 0x7FFFFFFF | Morphing Energy Time constant |
| 0x40 | 0x1F | Morphing Attack | 4 / 1.31 | 0x7FFFFFFF | Morphing Attack Time constant |
| 0x44 | 0x1F | Morphing Decay | 4 / 1.31 | 0x7FFFFFFF | Morphing Decay Time constant |
| 0x48 | 0x1F | Morphing K0_1 | 4 / 9.23 | 0x00000000 | Morphing Region 1 Slope (comp/Exp) |
| 0x4C | 0x1F | Morphing K1_1 | 4 / 9.23 | 0x00000000 | Morphing Region 2 Slope (comp/Exp) |
| 0x50 | 0x1F | Morphing K2_1 | 4 / 9.23 | 0x00000000 | Morphing Region 3 Slope (comp/Exp) |
| 0x54 | 0x1F | Morphing T1_1 | 4 / 9.23 | 0xE7000000 | Morphing Threshold 1 |
| 0x58 | 0x1F | Morphing T2_1 | 4 / 9.23 | 0xFE800000 | Morphing Threshold 2 |
| 0x5C | 0x1F | Morphing off1_1 | 4 / 9.23 | 0x00000000 | Morphing Offset 1 |
| 0x60 | 0x1F | Morphing off2_1 | 4 / 9.23 | 0x00000000 | Morphing Offset 2 |
| EXCURSION |  |  |  |  |  |
| 0x64 | 0x1F | Excursion | 4 / - | 0x024D9999 | Generated by GUI |
| 0x68 | 0x1F | Excursion | 4 / - | 0xFDB26667 | Generated by GUI |
| 0x6C | 0x1F | Excursion | 4 / - | 0x00800000 | Generated by GUI |
| 0x70 | 0x1F | Excursion | 4 / - | 0x062488E5 | Generated by GUI |
| 0x74 | 0x1F | Excursion | 4 / - | 0xFC1F497A | Generated by GUI |
| 0x78 | 0x1F | Excursion | 4 / - | 0x02E18722 | Generated by GUI |
| 0x7C | 0x1F | Excursion | 4 / - | 0x7A0570FF | Generated by GUI |
| 0x08 | 0x20 | Excursion | 4 / - | 0x89A41D50 | Generated by GUI |
| 0x0C | 0x20 | Excursion | 4 / - | 0x7FFFFFFF | Generated by GUI |
| 0x10 | 0x20 | Excursion | 4 / - | 0x00000000 | Generated by GUI |
| 0x14 | 0x20 | Excursion | 4 / - | 0x00000000 | Generated by GUI |
| 0x18 | 0x20 | Excursion | 4 / - | 0x00000000 | Generated by GUI |
| 0x1C | 0x20 | Excursion | 4 / - | 0x00000000 | Generated by GUI |
| 0x20 | 0x20 | Excursion | 4 / - | 0x7FFFFFFF | Generated by GUI |
| 0x24 | 0x20 | Excursion | 4 / - | 0x00000000 | Generated by GUI |
| 0x28 | 0x20 | Excursion | 4 / - | 0x765BE2B0 | Generated by GUI |
| 0x2C | 0x20 | Excursion | 4 / - | 0x50CDD9C9 | Generated by GUI |
| 0x30 | 0x20 | Excursion | 4 / - | 0xC3F79E7C | Generated by GUI |
| 0x34 | 0x20 | Excursion | 4 / - | 0x7FFFFFFF | Generated by GUI |
| 0x38 | 0x20 | Excursion | 4 / - | 0x00000000 | Generated by GUI |
| 0x3C | 0x20 | Excursion | 4 / - | 0x00000000 | Generated by GUI |
| 0x40 | 0x20 | Excursion | 4 / - | 0x00000000 | Generated by GUI |
| 0x44 | 0x20 | Excursion | 4 / - | 0x00000000 | Generated by GUI |
| 0x48 | 0x20 | Excursion | 4 / - | 0x05C7955C | Generated by GUI |
| 0x4C | 0x20 | Excursion | 4 / - | 0x05C7955C | Generated by GUI |
| 0x50 | 0x20 | Excursion | 4 / - | 0x05C7955C | Generated by GUI |
| 0x54 | 0x20 | Excursion | 4 / - | 0x6CC6628E | Generated by GUI |
| 0x58 | 0x20 | Excursion | 4 / - | 0xA162B94B | Generated by GUI |
| 0x5C | 0x20 | Excursion | 4 / - | 0x05C7955C | Generated by GUI |
| 0x60 | 0x20 | Excursion | 4 / - | 0x05C7955C | Generated by GUI |
| 0x64 | 0x20 | Excursion | 4 / - | 0x05C7955C | Generated by GUI |
| 0x68 | 0x20 | Excursion | 4 / - | 0x6CC6628E | Generated by GUI |
| 0x6C | 0x20 | Excursion | 4 / - | 0xA162B94B | Generated by GUI |
| 0x70 | 0x20 | Excursion | 4 / - | 0x6E0A82F4 | Generated by GUI |
| 0x74 | 0x20 | Excursion | 4 / - | 0x91F57D0C | Generated by GUI |
| 0x78 | 0x20 | Excursion | 4 / - | 0x6E0A82F4 | Generated by GUI |
| 0x7C | 0x20 | Excursion | 4 / - | 0x6CC6628E | Generated by GUI |
| 0x08 | 0x21 | Excursion | 4 / - | 0xA162B94B | Generated by GUI |
| 0x0C | 0x21 | Excursion | 4 / - | 0x6E0A82F4 | Generated by GUI |
| 0x10 | 0x21 | Excursion | 4 / - | 0x91F57D0C | Generated by GUI |
| 0x14 | 0x21 | Excursion | 4 / - | 0x6E0A82F4 | Generated by GUI |
| 0x18 | 0x21 | Excursion | 4 / - | 0x6CC6628E | Generated by GUI |
| 0x1C | 0x21 | Excursion | 4 / - | 0xA162B94B | Generated by GUI |
| MORPHING BQS |  |  |  |  |  |
| 0x20 | 0x21 | CH-L BQ 1 B0 | 4 / 1.31 | 0x7FFFFFFF | Left Channel Morphing BQ coefficient |
| 0x24 | 0x21 | CH-L BQ 1 B1 | 4 / 2.30 | 0x00000000 | Left Channel Morphing BQ coefficient |
| 0x28 | 0x21 | CH-L BQ 1 B2 | 4 / 1.31 | 0x00000000 | Left Channel Morphing BQ coefficient |
| 0x2C | 0x21 | CH-L BQ 1 A1 | 4 / 2.30 | 0x00000000 | Left Channel Morphing BQ coefficient |
| 0x30 | 0x21 | CH-L BQ 1 A2 | 4 / 1.31 | 0x00000000 | Left Channel Morphing BQ coefficient |
| 0x34 | 0x21 | CH-L BQ 2 B0 | 4 / 1.31 | 0x7FFFFFFF | Left Channel Morphing BQ coefficient |
| 0x38 | 0x21 | CH-L BQ 2 B1 | 4 / 2.30 | 0x00000000 | Left Channel Morphing BQ coefficient |
| 0x3C | 0x21 | CH-L BQ 2 B2 | 4 / 1.31 | 0x00000000 | Left Channel Morphing BQ coefficient |
| 0x40 | 0x21 | CH-L BQ 2 A1 | 4 / 2.30 | 0x00000000 | Left Channel Morphing BQ coefficient |
| 0x44 | 0x21 | CH-L BQ 2 A2 | 4 / 1.31 | 0x00000000 | Left Channel Morphing BQ coefficient |
| 0x48 | 0x21 | CH-Sub Gain scale | 4 / 9.23 | 0x00800000 | Sub Channel Morphing Gain |
| THERMAL PROTECT TRESHOLD |  |  |  |  |  |
| 0x4C | 0x21 | Thermal | 4 / - | 0x7FFFFFFF | Generated by GUI |
| 0x50 | 0x21 | Thermal | 4 / - | 0x7FFFFFFF | Generated by GUI |