##### wpa_supplicant configuration file template #####

#

# This option can be used to allow wpa_supplicant to overwrite configuration

# file whenever configuration is changed (e.g., new network block is added with

# wpa_cli or wpa_gui, or a password is changed). This is required for

# wpa_cli/wpa_gui to be able to store the configuration changes permanently.

# Please note that overwriting configuration file will remove the comments from

# it.

update_config=1

ctrl_interface=/var/run/wpa_supplicant

# IEEE 802.1X/EAPOL version

# wpa_supplicant is implemented based on IEEE Std 802.1X-2004 which defines

# EAPOL version 2. However, there are many APs that do not handle the new

# version number correctly (they seem to drop the frames completely). In order

# to make wpa_supplicant interoperate with these APs, the version number is set

# to 1 by default. This configuration value can be used to set it to the new

# version (2).

eapol_version=1

# AP scanning/selection

# By default, wpa_supplicant requests driver to perform AP scanning and then

# uses the scan results to select a suitable AP. Another alternative is to

# allow the driver to take care of AP scanning and selection and use

# wpa_supplicant just to process EAPOL frames based on IEEE 802.11 association

# information from the driver.

# 1: wpa_supplicant initiates scanning and AP selection; if no APs matching to

#        the currently enabled networks are found, a new network (IBSS or AP mode

#        operation) may be initialized (if configured) (default)

# 0: driver takes care of scanning, AP selection, and IEEE 802.11 association

#        parameters (e.g., WPA IE generation); this mode can also be used with

#        non-WPA drivers when using IEEE 802.1X mode; do not try to associate with

#        APs (i.e., external program needs to control association). This mode must

#        also be used when using wired Ethernet drivers.

# 2: like 0, but associate with APs using security policy and SSID (but not

#        BSSID); this can be used, e.g., with ndiswrapper and NDIS drivers to

#        enable operation with hidden SSIDs and optimized roaming; in this mode,

#        the network blocks in the configuration file are tried one by one until

#        the driver reports successful association; each network block should have

#        explicit security policy (i.e., only one option in the lists) for

#        key_mgmt, pairwise, group, proto variables

# When using IBSS or AP mode, ap_scan=2 mode can force the new network to be

# created immediately regardless of scan results. ap_scan=1 mode will first try

# to scan for existing networks and only if no matches with the enabled

# networks are found, a new IBSS or AP mode network is created.

ap_scan=1

# EAP fast re-authentication

# By default, fast re-authentication is enabled for all EAP methods that

# support it. This variable can be used to disable fast re-authentication.

# Normally, there is no need to disable this.

fast_reauth=1

device_name=Blaze

manufacturer=TI

model_name=Wilink

model_number=wl18xx

serial_number=12345

driver_param=use_multi_chan_concurrent=1 use_p2p_group_interface=1

# Timeout in seconds to detect STA inactivity (default: 300 seconds)

#

# This timeout value is used in P2P GO mode to clean up

# inactive stations.

p2p_go_max_inactivity=60

p2p_go_ht40=1

disassoc_low_ack=1

p2p_disabled=1