7.3.1.2.5 JESD204B Pattern Test

The DAC3xJ8x family of devices supports the following test patterns for JESD204B:

• Link layer test pattern

– Verify repeating /D.21.5/ high frequency pattern for random jitter (RJ)

– Verify repeating /K.28.5/ mixed frequency pattern for deterministic jitter (DJ)

– Verify repeating initial lane alignment (ILA) sequence

– RPAT, JSPAT or JTSPAT pattern can be verified using errors counter of 8b/10b errors produced over an amount of time to get an estimate of BER.

The test procedures for link layer test pattern are as follow:

- Enable the link layer test pattern through register jesd_test_seq in config74 register (0x4A). The respective setting for each test is as follow:

  - 2b’00 : test sequence disabled

  - 2b’01 : verify repeating D.21.5 high frequency pattern for random jitter

  - 2b’10 : verify repeating K.28.5 mixed frequency pattern for deterministic jitter

  - 2b’11 : verify repeating ILA sequence

- Clear bits 8 to15 of the config4 register (address 0x04) to disable the “lane error” alarm mask. Clear the bits according to the respective active lanes (for example, bit 8 is for lane0, bit 15 is for lane 7). If any error occurs on the respective lanes, the error result will show up at bit 8 of the config100 to config107 register, and the CMOS ALARM pin will trigger to signal the error.

- Set the JESD204B TX logic device to output the corresponding test pattern.

- Once the associated pattern is transmitted from the JESD204B TX logic device for the link layer pattern test, the error will be reflected on bit 8 of config100 to config107 for the respective JESD204B RX lane and also the CMOS ALARM pin. The polarity of CMOS ALARM pin depends alarm_out_pol bit in bit 3 of config0 register.

• Transport layer test pattern: implements a short transport layer pattern check based on F = 1,2,4 or 8. The

short test pattern has a duration of one frame period and is repeated continuously for the duration of the test.

Refer to JESD204B standard section 5.1.6 for more details.

– F = 1 : Looks for a constant 0xF1.

– F = 2 : Each frame should consist of 0xF1, 0xE2

– F = 4 : Looks for a constant 0xF1, 0xE2, 0xD3, 0xC4

– F = 8 : Each frame should consist of 0xF1, 0xE2, 0xD3, 0xC4, 0xB5, 0xA6, 0x97, 0x81

The test procedures for transport layer test pattern are as follow:

- Set shortest_ena to 1b’1 in bit 12 of the config2 register (address 0x02). This enables short transport layer checker.

- Clear bits 8 to15 of the config6 register (address 0x06) to disable the “Short Test Error” alarm mask. Clear the bits according to the respective active lanes (for example, bit 8 is for lane0, bit 15 is for lane 7). If any error occurs on the respective lanes, the error result will show up at bit 8 to 15 of the config109 register, and the CMOS ALARM pin will trigger to signal the error.

- Set the JESD204B TX logic device to output the corresponding test pattern.

- Check the result at bits 8 to 15 of the config109 register and also the CMOS ALARM pin. The polarity of CMOS ALARM pin depends alarm_out_pol bit in bit 3 of config0 register.