//--------------------------------------------------

#define   STATUS  0x00 //Status Control Register 0

#define   MUX     0x01 //Multiplexer Control Register 0

#define   ADCON   0x02 //A/D Control Register 0

#define   DRATE   0x03 //A/D Data Rate Control Register 0

#define   IO      0X04 //GPIO Control Register 0

#define   OFC0    0x05 //Offset Calibration Coefficient Register 1

#define   OFC1    0x06 //Offset Calibration Coefficient Register 2

#define   OFC2    0x07 //Offset Calibration Coefficient Register 2

#define   FSC0    0x08 //Full scale Callibration Coefficient Register 0

#define   FSC1    0x09 //Full scale Callibration Coefficient Register 1

#define   FSC2    0x0A //Full scale Callibration Coefficient Register 2

/*STATUS - Status Control Register 0 ( see p30)*/

/* BIT7 - BIT6 -  BIT5   -  BIT4   -  BIT3   -  BIT2   -  BIT1   -  BIT0 */

/* ID   - ID   -  ID     -  ID     -  ORDER  -  ACAL   -  BUFEN  -  DRDY */

#define STATUS_RESET 0x01 // Reset STATUS Register

/*Bits 7 - 4 ID3, ID2, ID1, ID0 Factory Programmed Identification Bits(Read Only)*/

/*ORDER1:0  Data Output Bit Order*/

#define ORDER_MSB B00000000 // Most significant Bit first (default)

#define ORDER_LSB B00001000//Least significant Bit first

/*Input data is always shifted in most significant byte and bit first. Output data is always shifted out most significant

byte first. The ORDER bit only controls the bit order of the output data within the byte.*/

/*ACAL1:0 Auto Calibration*/

#define ACAL_OFF B00000000 // Auto Calibration Disabled (default)

#define ACAL_ON  B00000100 // Auto Calibration Enabled

/*When Auto-Calibration is enabled, self-calibration begins at the completion of the WREG command that changes

the PGA (bits 0-2 of ADCON register), DR (bits 7-0 in the DRATE register) or BUFEN (bit 1 in the STATUS register)

values.*/

/*BUFEN1:0 Analog Input Buffer Enable*/

#define BUFEN_OFF B00000000 //Buffer Disabled (default)

#define BUFEN_ON  B00000010 //BUffer Enabled

/*DRDY1:0 Data Ready (Read Only) Duplicates the state of the DRDY pin*/

/* MUX - Multiplexer Control Register 0 (see p31 - bring together with bitwise OR | */

/* BIT7  - BIT6  -  BIT5   -  BIT4   -  BIT3   -  BIT2   -  BIT1   -  BIT0 */

/* PSEL3 - PSEL2 -  PSEL1  -  PSEL0  -  NSEL3  -  NSEL2   - NSEL1   - NSEL0 */

#define MUX_RESET 0x01      // Reset MUX0 Register

/* PSEL3:0 Positive input channel selection bits */

#define P_AIN0 B00000000 //(default)

#define P_AIN1 B00010000

#define P_AIN2 B00100000

#define P_AIN3 B00110000

#define P_AIN4 B01000000

#define P_AIN5 B01010000

#define P_AIN6 B01100000

#define P_AIN7 B01110000

#define P_AINCOM B10000000

/* NSEL3:0 Negativ input channel selection bits */

#define N_AIN0 B00000000

#define N_AIN1 B00000001 //(default)

#define N_AIN2 B00000010

#define N_AIN3 B00000011

#define N_AIN4 B00000100

#define N_AIN5 B00000101

#define N_AIN6 B00000110

#define N_AIN7 B00000111

#define N_AINCOM B00001000

// Differential settings

#define DIFF0_1 B00000001

#define DIFF2_3 B00100011

#define DIFF4_5 B01000101

#define DIFF6_7 B01100111

/*ADCON - A/D Control Register 0 ( see p31)*/

/* BIT7 - BIT6   -  BIT5   -  BIT4   -  BIT3   -  BIT2   -  BIT1   -  BIT0 */

/* 0    - CLK1   -  CLK0   -  SDCS1  -  SDCS0  -  PGA2   -  PGA1   -  PAG0 */

#define ADCON_RESET 0x20 // Reset ADCON Register

/*CLK2:0 D0/CLKOUT Clock Out Rate Setting*/

#define CLK_OFF B00000000 //Clock Out off

#define CLK_1   B00100000 //Clock Out Frequency = fCLKIN (default)

#define CLK_2   B01000000 //Clock Out Frequency = fCLKIN/2

#define CLK_4   B01100000 //Clock Out Frequency = fCLKIN/4

/*When not using CLKOUT, it is recommended that it be turned off. These bits can only be reset using the RESET pin.*/

/*SDCS2:0 Sensor Detection Current Sources*/

#define SDCS_OFF B00000000//Sensor Detect Off (default)

#define SDCS_05  B00001000//Sensor Detect Current 0.5?A

#define SDCS_2   B00010000//Sensor Detect Current 2?A

#define SDCS_10  B00011000//Sensor Detect Current 10?A

/*The Sensor Detect Current Sources can be activated to verify the integrity of an external sensor supplying a signal to the

ADS1255/6. A shorted sensor produces a very small signal while an open-circuit sensor produces a very large signal.*/

/*PGA3:0 Programmable Gain Amplifier Setting*/

#define PGA_1   B00000000 //(default)

#define PGA_2   B00000001

#define PGA_4   B00000010

#define PGA_8   B00000011

#define PGA_16  B00000100

#define PGA_32  B00000101

#define PGA_64  B00000110

#define PGAmulti_1    0.0000005960464832810452   //PGA-1  ą5V        /16777215

#define PGAmulti_2    0.0000002980232416405226   //PGA-2  ą2.5V      /16777215

#define PGAmulti_4    0.0000001490116208202613   //PGA-4  ą1.25V     /16777215

#define PGAmulti_8    0.00000007450581041013064  //PGA-8  ą0.625V    /16777215

#define PGAmulti_16   0.00000003725290520506532  //PGA-16 ą312.5mV   /16777215

#define PGAmulti_32   0.00000001862645260253266  //PGA-32 ą156.25mV  /16777215

#define PGAmulti_64   0.000000009313226301266331 //PGA-64 ą78.125mV  /16777215

/*DRATE - A/D Data Rate Register 0 ( see p32)*/

/* BIT7 - BIT6   -  BIT5   -  BIT4   -  BIT3   -  BIT2   -  BIT1   -  BIT0 */

/* DR7  - DR6    -  DR5    -  DR4    -  DR3    -  DR2    -  DR1    -  DR0 */

#define DRATE_RESET 0xF0 // Reset DRATE Register

/*DR7:0 Data Rate Setting*/

#define DR_30000 B11110000 //30.000 SPS (default)

#define DR_15000 B11100000 //15.000 SPS

#define DR_7500  B11010000 //7.500 SPS

#define DR_3750  B11000000 //3.750 SPS

#define DR_2000  B10110000 //2.000 SPS

#define DR_1000  B10100001 //1.000 SPS

#define DR_500   B10010010 //500 SPS

#define DR_100   B10000010 //100 SPS

#define DR_60    B01110010 //60 SPS

#define DR_50    B01100011 //50 SPS

#define DR_30    B01010011 //30 SPS

#define DR_25    B01000011 //25 SPS

#define DR_15    B00110011 //15 SPS

#define DR_10    B00100011 //10 SPS

#define DR_5     B00010011 //5 SPS

#define DR2_5    B00000011 //2,5 SPS

/*IO - GPIO Control Register 0 ( see p32)*/

/* BIT7 - BIT6   -  BIT5   -  BIT4   -  BIT3   -  BIT2   -  BIT1   -  BIT0 */

/* DIR3 - DIR2   -  DIR1   -  DIR0   -  DIO3   -  DIO2   -  DIO1   -  DIO0 */

#define IO_RESET 0xE0 // Reset IO Register

/*DIR3 - Digital I/O Direction for Pin D3*/

#define DIR3_OUT B00000000 //D3 is an output

#define DIR_IN   B10000000 //D3 is an input (default)

/*DIR2 - Digital I/O Direction for Pin D3*/

#define DIR2_OUT B00000000 //D2 is an output

#define DIR2_IN  B01000000 //D2 is an input (default)

/*DIR1 - Digital I/O Direction for Pin D3*/

#define DIR1_OUT B00000000 //D1 is an output

#define DIR1_IN  B001ßß000 //D1 is an input (default)

/*DIR0 - Digital I/O Direction for Pin D3*/

#define DIR0_OUT B00000000 //D0/CLKOUT is an output

#define DIR0_IN  B00010000 //D0/CLKOUT is an input (default)

/*DIO3:0 Status of Digital I/O, Read Only*/

/* SPI COMMAND DEFINITIONS (p34) */

/*SYSTEM CONTROL */

#define   WAKEUP  0x00  //Exit Sleep Mode

#define   STANDBY 0xFD  //Enter Sleep Mode

#define   SYNC    0xFC  //Synchornize the A/D Conversion

#define   RESET   0xFE  //Reset To Power UP values

#define   NOP     0xFF  //No operation

/*DATA READ*/

#define   RDATA   0x01  //Read data once

#define   RDATAC  0x03  //Read data continously

#define   SDATAC  0x0F  //Stop reading data continously

/*READ REGISTER */

#define   RREG    0x10  //Read From Register

#define   WREG    0x50  //Write To Register

/*Calibration */

#define   SYSOCAL 0xF3  //System Offset Calibration

#define   SYSGCAL 0xF2  //System Gain Calibration

#define   SELFCAL 0xF0  //Self offset then gain calibration

#define   SELFOCAL 0xF1 //Self offset calibration

#define   SELFGCAL 0xF2 //Self gain calibration

//---------------------------------------------------

#define cs 10 // chip select

#define rdy 9 // data ready, input

#define rst 8

#define MISOPIN 12

#define SCLKPIN 13

#define rdy 5

#define DIN 11

#define cs 10

#define SPISPEED 1250000

// 1700000

#include <SPI.h>

void setup()

{

Serial.begin(115200);

pinMode(SCLKPIN, OUTPUT); pinMode(MISOPIN, INPUT);pinMode(cs,OUTPUT); pinMode(rdy, INPUT);pinMode(rst,OUTPUT);pinMode(DIN,OUTPUT);

digitalWrite(cs, LOW);

reset_adc();

delay(500);

SPI.begin(); //start the spi-bus

delay(500);

while (digitalRead(rdy)) {}  // wait for ready_line to go low

SPI.beginTransaction(SPISettings(SPISPEED, MSBFIRST, SPI_MODE1)); // start SPI

delayMicroseconds(10);

//Reset to Power-Up Values (FEh)

//SPI.transfer(0xFE);

//delayMicroseconds(100);

byte status_reg = 0 ;  // address (datasheet p. 30)

//PGA SETTING

//1 ±5V

//2 ±2.5V

//4 ±1.25V

//8 ±0.625V

//16 ±312.5mV

//32 ±156.25mV

//64 ±78.125mV

// ----------------------------------------------

// Status setup

// PGA SETTING

Serial.println("Status Data setup started");

byte status_data = 0x01; //status: Most Significant Bit First, Auto-Calibration Disabled, Analog Input Buffer Disabled

SPI.transfer(0x50 | status_reg);

SPI.transfer(0x00);   // 2nd command byte, write one register only

SPI.transfer(status_data);   // write the databyte to the register

delayMicroseconds(10);

//-----------------------------------------------

//-------------------------------

// ADCON setup

//byte adcon_data = 0x00; // 0 00 00 000

Serial.println("ADCON Data setup started");

SPI.transfer(0x50 | ADCON);

SPI.transfer(0x00);   // 2nd command byte, write one register only

SPI.transfer(PGA_1);   // write the databyte to the register

Serial.println("ADCON Data setup ended");

delayMicroseconds(10);

//--------------------------------

//--------------------------------

// Data Rate setup

Serial.println("DRATE Data setup started");

SPI.transfer(0x50 | DRATE);

SPI.transfer(0x00);

SPI.transfer(DR_100);

Serial.println("DRATE Data setup complete");

delayMicroseconds(10);

//--------------------------------

//--------------------------------

//Self-calibration full

Serial.println("Full Self-cal setup started");

SPI.transfer(SELFCAL);

delayMicroseconds(1300);

Serial.println("Full Self-cal setup complete");

//--------------------------------

digitalWrite(cs, HIGH);

Serial.println("configured, starting");

}

void reset_adc()

{

digitalWrite(rst, LOW);

delay(1);

digitalWrite(rst,HIGH);

//Another method to make reset

//digitalWrite(SCLKPIN,LOW);

//delayMicroseconds(326);

//digitalWrite(SCLKPIN,HIGH);

//delayMicroseconds((500+750+1250)/CLKIN);

}

void Self_cal_full()

{

Serial.println("Full Self-cal setup started");

SPI.transfer(SELFCAL);

delayMicroseconds(1300);

Serial.println("Full Self-cal setup complete");

}

void Self_cal_offset()

{

Serial.println("Self offset calibration setup started");

SPI.transfer(SELFOCAL);

delayMicroseconds(1300);

Serial.println("Self offset calibration setup complete");

}

void Self_cal_gain()

{

Serial.println("Self gain calibration setup started");

SPI.transfer(SELFGCAL);

delayMicroseconds(1300);

Serial.println("Self gain calibration setup complete");

}

void ReadReg_STATUS()

{

byte reg_val;

digitalWrite(cs, LOW);

SPI.beginTransaction(SPISettings(SPISPEED, MSBFIRST, SPI_MODE1)); // start SPI

delayMicroseconds(10);

while (digitalRead(rdy)) {} ;

SPI.transfer(0x10);

delayMicroseconds(10);

//SYNC command 1111 1100

SPI.transfer(SYNC);

delayMicroseconds(10);

//WAKEUP 0000 0000

SPI.transfer(WAKEUP);

delayMicroseconds(10);

reg_val = SPI.transfer(0);

reg_val <<= 8; //shift to left

reg_val |= SPI.transfer(0);

reg_val <<= 8;

reg_val |= SPI.transfer(0);

Serial.print("HEX: "); Serial.println(reg_val,HEX);

Serial.print("BIN: "); Serial.println(reg_val,BIN);

delayMicroseconds(10);

digitalWrite(cs, HIGH);

SPI.endTransaction();

}

void AIN_0_COM()

{

unsigned long adc_val =0; // store reading

digitalWrite(cs, LOW);

SPI.beginTransaction(SPISettings(SPISPEED, MSBFIRST, SPI_MODE1)); // start SPI

delayMicroseconds(10);

//The most efficient way to cycle through the inputs is to

//change the multiplexer setting (using a WREG command

//to the multiplexer register MUX) immediately after DRDY

//goes low. Then, after changing the multiplexer, restart the

//conversion process by issuing the SYNC and WAKEUP

//commands, and retrieve the data with the RDATA

//command.

while (digitalRead(rdy)) {} ;

//byte channel = 0;

//byte data = (channel << 4) | (1 << 3); //AIN-channel and AINCOM

//byte data = 1;

SPI.transfer(0x50 | 1); // MUX register (0101 rrrr)

SPI.transfer(0x00);   // 2nd command byte, write one register only

//SPI.transfer(AIN0_COM);   // write the databyte to the register

SPI.transfer(P_AIN0 + N_AINCOM);

delayMicroseconds(10);

//SYNC command 1111 1100

SPI.transfer(SYNC);

//delayMicroseconds(10);

//WAKEUP 0000 0000

SPI.transfer(WAKEUP);

//delayMicroseconds(10);

SPI.transfer(0x01); // Read Data 0000  0001 (01h)

//delayMicroseconds(10);

adc_val = SPI.transfer(0);

adc_val <<= 8; //shift to left

adc_val |= SPI.transfer(0);

adc_val <<= 8;

adc_val |= SPI.transfer(0);

delayMicroseconds(10);

digitalWrite(cs, HIGH);

SPI.endTransaction();

//The ADS1255/6 output 24 bits of data in Binary Two's

//Complement format. The LSB has a weight of

//2VREF/(PGA(223 - 1)). A positive full-scale input produces

//an output code of 7FFFFFh and the negative full-scale

//input produces an output code of 800000h.

if(adc_val > 0x7fffff){ //if MSB == 1

adc_val = (16777215ul - adc_val) + 1; //do 2's complement

}

//Serial.print("AIN0_COM: ");

Serial.println(adc_val*PGAmulti_1,10);

//Serial.println(" Volt");

//Serial.println(adc_val*PGAmulti_1*1000,2);

}

void AIN_1_COM()

{

unsigned long adc_val =0; // store reading

digitalWrite(cs, LOW);

SPI.beginTransaction(SPISettings(SPISPEED, MSBFIRST, SPI_MODE1)); // start SPI

delayMicroseconds(10);

//The most efficient way to cycle through the inputs is to

//change the multiplexer setting (using a WREG command

//to the multiplexer register MUX) immediately after DRDY

//goes low. Then, after changing the multiplexer, restart the

//conversion process by issuing the SYNC and WAKEUP

//commands, and retrieve the data with the RDATA

//command.

while (digitalRead(rdy)) {} ;

//byte channel = 0;

//byte data = (channel << 4) | (1 << 3); //AIN-channel and AINCOM

//byte data = 1;

SPI.transfer(0x50 | 1); // MUX register

SPI.transfer(0x00);   // 2nd command byte, write one register only

//SPI.transfer(AIN0_COM);   // write the databyte to the register

SPI.transfer(P_AIN1 + N_AINCOM);

delayMicroseconds(10);

//SYNC command 1111 1100

SPI.transfer(SYNC);

delayMicroseconds(10);

//WAKEUP 0000 0000

SPI.transfer(WAKEUP);

delayMicroseconds(10);

SPI.transfer(0x01); // Read Data 0000  0001 (01h)

delayMicroseconds(10);

adc_val = SPI.transfer(0);

adc_val <<= 8; //shift to left

adc_val |= SPI.transfer(0);

adc_val <<= 8;

adc_val |= SPI.transfer(0);

delayMicroseconds(10);

digitalWrite(cs, HIGH);

SPI.endTransaction();

//The ADS1255/6 output 24 bits of data in Binary Two's

//Complement format. The LSB has a weight of

//2VREF/(PGA(223 - 1)). A positive full-scale input produces

//an output code of 7FFFFFh and the negative full-scale

//input produces an output code of 800000h.

if(adc_val > 0x7fffff){ //if MSB == 1

adc_val = (16777215ul - adc_val) + 1; //do 2's complement

}

Serial.print("AIN1_COM: "); Serial.print(adc_val*PGAmulti_1,5); Serial.println(" Volt");

}

void DIFFdata0_1()

{

unsigned long adc_val =0; // store reading

digitalWrite(cs, LOW);

SPI.beginTransaction(SPISettings(SPISPEED, MSBFIRST, SPI_MODE1)); // start SPI

delayMicroseconds(10);

//The most efficient way to cycle through the inputs is to

//change the multiplexer setting (using a WREG command

//to the multiplexer register MUX) immediately after DRDY

//goes low. Then, after changing the multiplexer, restart the

//conversion process by issuing the SYNC and WAKEUP

//commands, and retrieve the data with the RDATA

//command.

while (digitalRead(rdy)) {} ;

//byte channel = 0;

//byte data = (channel << 4) | (1 << 3); //AIN-channel and AINCOM

//byte data = 1;

SPI.transfer(0x50 | 1); // MUX register

SPI.transfer(0x00);   // 2nd command byte, write one register only

SPI.transfer(DIFF0_1);   // write the databyte to the register

delayMicroseconds(10);

//SYNC command 1111 1100

SPI.transfer(SYNC);

delayMicroseconds(10);

//WAKEUP 0000 0000

SPI.transfer(WAKEUP);

delayMicroseconds(10);

SPI.transfer(RDATA); // Read Data 0000  0001 (01h)

delayMicroseconds(10);

adc_val = SPI.transfer(0);

adc_val <<= 8; //shift to left

adc_val |= SPI.transfer(0);

adc_val <<= 8;

adc_val |= SPI.transfer(0);

Serial.println(adc_val, HEX);

delayMicroseconds(10);

digitalWrite(cs, HIGH);

SPI.endTransaction();

//The ADS1255/6 output 24 bits of data in Binary Two's

//Complement format. The LSB has a weight of

//2VREF/(PGA(223 - 1)). A positive full-scale input produces

//an output code of 7FFFFFh and the negative full-scale

//input produces an output code of 800000h.

if(adc_val > 0x7fffff){ //if MSB == 1

adc_val = (16777215ul - adc_val) + 1; //do 2's complement

}

Serial.print("DIFF0_1: "); Serial.print(adc_val*PGAmulti_1,6); Serial.println(" Volt");

}

void DIFFdata2_3()

{

unsigned long adc_val =0; // store reading

digitalWrite(cs, LOW);

SPI.beginTransaction(SPISettings(SPISPEED, MSBFIRST, SPI_MODE1)); // start SPI

delayMicroseconds(10);

//The most efficient way to cycle through the inputs is to

//change the multiplexer setting (using a WREG command

//to the multiplexer register MUX) immediately after DRDY

//goes low. Then, after changing the multiplexer, restart the

//conversion process by issuing the SYNC and WAKEUP

//commands, and retrieve the data with the RDATA

//command.

while (digitalRead(rdy)) {} ;

SPI.transfer(0x50 | 1); // MUX register

SPI.transfer(0x00);   // 2nd command byte, write one register only

SPI.transfer(DIFF2_3);   // write the databyte to the register

delayMicroseconds(10);

//SYNC command 1111 1100

SPI.transfer(SYNC);

delayMicroseconds(10);

//WAKEUP 0000 0000

SPI.transfer(WAKEUP);

delayMicroseconds(10);

SPI.transfer(RDATA); // Read Data 0000  0001 (01h)

delayMicroseconds(10);

adc_val = SPI.transfer(0);

adc_val <<= 8; //shift to left

adc_val |= SPI.transfer(0);

adc_val <<= 8;

adc_val |= SPI.transfer(0);

delayMicroseconds(10);

digitalWrite(cs, HIGH);

SPI.endTransaction();

Serial.println(adc_val);

//The ADS1255/6 output 24 bits of data in Binary Two's

//Complement format. The LSB has a weight of

//2VREF/(PGA(223 - 1)). A positive full-scale input produces

//an output code of 7FFFFFh and the negative full-scale

//input produces an output code of 800000h.

if(adc_val > 0x7fffff){ //if MSB == 1

adc_val = (16777215ul - adc_val) + 1; //do 2's complement

}

Serial.print("DIFF2_3: ");

Serial.print(adc_val*PGAmulti_1,5);

Serial.println(" Volt");

}

void loop()

{

//  while (Serial.available() > 0)

//  {

AIN_0_COM();

//DIFFdata0_1();

//delay(100);

//ReadReg_STATUS();

delay(100);

digitalWrite(cs, HIGH);

//  }

}