Download and Extract the Design files

- Download and extract the design files from the following link:

ftp://ftp.actel.com/outgoing/RTG4_ADC12DJ3200.rar

- Extract this file to a local drive.

Programing using Flash pro Software

- Launch the FlashPro software.

Go to Start > Programs > Microsemi > Libero SoC v11.8 > Program Debug >FlashPro

- Click New Project, In the New Project window, enter the project name.

- Click Browse and navigate to the location where the project is required to be saved.

- Select Single device as the Programming mode.

- Click OK to save the project.

- Click Configure Device. (? Don’t see this step in video ?)

- Click Browse,  navigate to RTG4_ADC12DJ3200\Programing_file\JESD204B_RTG4.stp

- Click run program action button to program the RTG4 FPGA.

Configuring ADC12DJ3200

- Open ADC12DJ3200 GUI

- Configure this GUI as shown in the following Figures.

- Configuring Clock and Data mode. (click Program Clocks and ADC after selecting 1000M and JMODE11)

- Configuring JESD204B. (Matt’s comments – turn off JESD block enable, turn off scrambler, than turn back on JESD block enable) (this step is not shown in the video)

- Configuring NCO

- Configuring LMK0428

- Reset the board once programing and ADC configuration is completed by pressing   SW7 DEVRST button on board.

Debugging using Identify

- Open Identify pro software.

Go to Start > Programs > Microsemi >Synopsys>Identify RTL debugger

- Open the identify project, navigate to

RTG4_ADC12DJ3200\Libero_synthesis\synthesis\JESD204B_RTG4_TOP_syn.prj  as shown in Figure.

- Click Debug button and select Run as shown in figure to start capturing samples. (Matt’s comments – you have to hit Stop before you can look at data)

- Click Open waveform Display button to see the captured data as shown in figure.

- Following figure shows the final captured samples. (Matt’s comments – select the DATA_OUT, right click and ???......  see video)

Useful links:

- RTG4: https://www.microsemi.com/products/fpga-soc/radtolerant-fpgas/rtg4#documents

- Identify: https://www.microsemi.com/products/fpga-soc/design-resources/programming/identify-me-downloads#documents

- Libero: https://www.microsemi.com/products/fpga-soc/design-resources/design-software/libero-soc