- Turn on Device clock and SYSREF to the ADC

- Issue a hard reset

- Write the following registers

General Register:

0x00:	0x81

Master Page Regs: 
0x59: 0x20

Main Digital Page Regs: 
0xf7: 0x01    	
0x00: 0x01

0x00: 0x00

JESD Digital Page Regs:  
0x00: 0x80       
0x01: 0x04            
0x06: 0x1f       
0x07: 0x08       
0x16: 0x80       

JESD Analog Page Regs: 
0x12: 0x02              
0x16: 0x02       
0x17: 0x40       
0x17: 0x00      

Offset Read Page Regs:  
0x68: 0x02

- Turn off SYSREF to ADC only

- Setup FPGA for JESD link initialization.

- Capture data from ADC.