Using the TI evaluation modules and software:

- Connect 5V power and USB to both boards

- Turn on power (using SW6) to data capture board

- Turn on power (using S1) to ADC board

- Start the ADC12J4000EVM GUI program

- Select “EVM” tab on ADC12J4000EVM GUI

  - Select clock source (On-board)

  - Select Fs (FS=4000 Msps)

  - Select decimation mode (Bypass Mode; DDR)

  - Click “Program Clocks and ADC” button once

- Select “Control” tab on ADC12J4000EVM GUI

  - Click “ Excute Foreground CAL” button once

- Start the HSDC Pro software

- Select the ADC12J4000_BYPASS device to upload FPGA firmware

- Enter the “ADC Output Data Rate” (should match the Fs from above)

- Select the number of samples to collect

- Click the “Capture” button to acquire a set of samples

To use external trigger:

- “Data Capture Options” => “Trigger Option”

- Select “Trigger mode enable” then OK

- Click the “Read DDR Memory” button. Samples will be acquired on external trigger event. This must occur within 12 seconds of clicking button. NOTE: if continuous triggers is applied to the trigger input, this seems to hang the board after the first successful trigger event.

Screen captures of setup: