/*

- * ADS1243.h

- *

- *  Created on: May 31, 2017

- *      Author: COMP-2

- */

- #ifndef ADS1243_H_

- #define ADS1243_H_

- #if defined (__MSP432P401R__)

- /* Definition of GPIO Port Bits Used for Communication */

- // P10.0

- #define ADS1243_CS          0x01

- // P10.2

- #define ADS1243_DIN         0x04

- // P10.3

- #define ADS1243_DOUT        0x08

- // P2.5

- #define ADS1243_DRDY        0x20

- // P10.1

- #define ADS1243_SCLK        0x02

- // P2.4

- #define ADS1243_PDWN        0x10

- #endif

- /* Error Return Values */

- #define ADS1243_NO_ERROR        0

- #define ADS1243_ERROR           -1

- /* ADS1243 Register Definitions */

- #define ADS1243_0_SETUP          0x00

- #define ADS1243_1_MUX            0x01

- #define ADS1243_2_ACR            0x02

- #define ADS1243_3_ODAC           0x03

- #define ADS1243_4_DIO            0x04

- #define ADS1243_5_DIR            0x05

- #define ADS1243_6_IOCON          0x06

- #define ADS1243_7_OCR0           0x07

- #define ADS1243_8_OCR1           0x08

- #define ADS1243_9_OCR2           0x09

- #define ADS1243_10_FSR0          0x0A

- #define ADS1243_11_FSR1          0x0B

- #define ADS1243_12_FSR2          0x0C

- #define ADS1243_13_DOR2          0x0D

- #define ADS1243_14_DOR1          0x0E

- #define ADS1243_15_DOR0          0x0F

- /* ADS1243 Register 0 (SETUP) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //        Factory programmed                     | BOSC      |      PGA[2:0]                  |

- //

- // Define  Burn out current source

- #define ADS1243_BOSC_EN         0x40

- #define ADS1243_BOSC_DS         0x00

- // Define Gain set

- #define ADS1243_PGA_1           0x00

- #define ADS1243_PGA_2           0x01

- #define ADS1243_PGA_4           0x02

- #define ADS1243_PGA_8           0x03

- #define ADS1243_PGA_16          0x04

- #define ADS1243_PGA_32          0x05

- #define ADS1243_PGA_64          0x06

- #define ADS1243_PGA_128         0x07

- /* ADS1243 Register 1 (MUX) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                PSEL[7:4]      |         NSEL[7:4]                          |

- // Positive channel selection

- #define ADS1243_PAIN0          0x00

- #define ADS1243_PAIN1          0x10

- #define ADS1243_PAIN2          0x20

- #define ADS1243_PAIN3          0x30

- #define ADS1243_PAIN4          0x40

- #define ADS1243_PAIN5          0x50

- #define ADS1243_PAIN6          0x60

- #define ADS1243_PAIN7          0x70

- // Negative channel selection

- #define ADS1243_NAIN0          0x00

- #define ADS1243_NAIN1          0x01

- #define ADS1243_NAIN2          0x02

- #define ADS1243_NAIN3          0x03

- #define ADS1243_NAIN4          0x04

- #define ADS1243_NAIN5          0x05

- #define ADS1243_NAIN6          0x06

- #define ADS1243_NAIN7          0x07

- /* ADS1243 Register 2 (ACR) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //  ~DRDY    |DATA FORMATE|    SPEED |BUFFER ENABLE|BIT ORDER| RANGE SELECT|  DATA RATE[1:0]

- //

- #define ADS1243_DRDY_READ       0x80

- #define ADS1243_BIPLAR          0x00

- #define ADS1243_UNIPOLAR        0x40

- #define ADS1243_SPEED_0         0x00

- #define ADS1243_SPEED_1         0x20

- #define ADS1243_BUFEN           0x00

- #define ADS1243_BUFDS           0x10

- #define ADS1243_MSB             0x00

- #define ADS1243_LSB             0x08

- #define ADS1243_SCALE_0         0x00

- #define ADS1243_SCALE_1         0x04

- #define ADS1243_DR_0            0x00

- #define ADS1243_DR_1            0x01

- #define ADS1243_DR_2            0x02

- /* ADS1243 Register 3 (ODAC) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- // SIGN      |              OFFSET VALUE

- #define ADS1243_POSITIVE         0x00

- #define ADS1243_NEGATIVE         0x80

- // OFFSET VALUE SHOULD BE GIVEN BY MANUALLY

- /* ADS1243 Register 4 (DIO) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         DIO[7:0]

- //

- // Define GPIO Direction (0-Output; 1-Input)

- #define ADS1243_DIO_0            0x01

- #define ADS1243_DIO_1            0x02

- #define ADS1243_DIO_2            0x04

- #define ADS1243_DIO_3            0x08

- #define ADS1243_DIO_4            0x10

- #define ADS1243_DIO_5            0x20

- #define ADS1243_DIO_6            0x40

- #define ADS1243_DIO_7            0x80

- /* ADS1243 Register 5 (DIR) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         DIR[7:0]

- //

- // Define GPIO Direction (0-Output; 1-Input)

- #define ADS1243_IO_0            0x01

- #define ADS1243_IO_1            0x02

- #define ADS1243_IO_2            0x04

- #define ADS1243_IO_3            0x08

- #define ADS1243_IO_4            0x10

- #define ADS1243_IO_5            0x20

- #define ADS1243_IO_6            0x40

- #define ADS1243_IO_7            0x80

- /* ADS1243 Register 6  I/O CONFIGRATION (IOCON) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                        IO[7:0]

- //

- // Define GPIO Direction (0-Analog; 1-Data)

- #define ADS1243_IOCON_0            0x01

- #define ADS1243_IOCON_1            0x02

- #define ADS1243_IOCON_2            0x04

- #define ADS1243_IOCON_3            0x08

- #define ADS1243_IOCON_4            0x10

- #define ADS1243_IOCON_5            0x20

- #define ADS1243_IOCON_6            0x40

- #define ADS1243_IOCON_7            0x80

- /* Command Definitions */

- // System Control

- #define ADS1243_CMD_WAKEUP      0xFB

- #define ADS1243_CMD_SLEEP       0xFD

- #define ADS1243_CMD_RESET       0xFE

- #define ADS1243_CMD_DSYNC       0xFC

- // Data Read

- #define ADS1243_CMD_RDATA       0x01

- #define ADS1243_CMD_RDATAC      0x03

- #define ADS1243_CMD_STOPC       0x0F

- // Read Register

- #define ADS1243_CMD_RREG        0x10

- // Write Register

- #define ADS1243_CMD_WREG        0x50

- // Calibration

- #define ADS1243_CMD_SYSOCAL     0xF3

- #define ADS1243_CMD_SYSGCAL     0xF4

- #define ADS1243_CMD_SELFOCAL    0xF1

- #define ADS1243_CMD_SELFGCAL    0xF2

- #define ADS1243_CMD_SELFCAL     0xF0

- /* ADS1243 Register 7 (OCR0) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         OCR0[7:0]

- //

- /* ADS1243 Register 8 (OCR1) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         OCR1[7:0]

- //

- /* ADS1243 Register 9 (OCR2) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         OCR2[7:0]

- //

- /* ADS1243 Register 10 (FSR0) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                        FSR0[7:0]

- //

- /* ADS1243 Register 11 (FSR1) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         FSR1[7:0]

- //

- /* ADS1243 Register 12 (FSR2) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         FSR2[7:0]

- //

- /* ADS1243 Register 15 (DOR0) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                       DOR0[7:0]

- //

- /* ADS1243 Register 14 (DOR1) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         DOR1[7:0]

- //

- /* ADS1243 Register 13 (DOR2) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         DOR2[7:0]

- /* Low Level ADS1243 Device Functions */

- void InitConfig(void);                          // Device initialization, setting a default starting condition

- int ADS1243WaitForDataReady(int Timeout);       // DRDY polling, with a maximum polling wait time if needed for break from loop; Timeout=0 is no timeout

- void ADS1243AssertCS(int fAssert);              // Assert/deassert CS (0=low, 1=high)

- void ADS1243SendByte(unsigned char cData );     // Send byte to the ADS1243

- unsigned char ADS1243ReceiveByte(void);         // Receive byte from the ADS1243

- /* ADS1243 Higher Level Functions */

- // Commands as given in datasheet

- void ADS1243SendWakeup(void);                   // Send device Wakeup Command

- void ADS1243SendSleep(void);                    // Set device to Sleep

- void ADS1243SendSync(void);                     // Sync device

- void ADS1243SendResetCommand(void);             // Send a device Reset Command

- long ADS1243ReadData(void);                     // Read the data results by command

- void ADS1243ReadRegister(int StartAddress, int NumReg,unsigned * pData);    // Read the register(s) (NumReg is the total number of registers read)

- void ADS1243WriteRegister(int StartAddress, int NumReg,unsigned * pData);   // Write the register(s) (NumReg is the total number of registers written)

- void ADS1243WriteSequence(int StartAddress, int NumReg,unsigned * pData);   // Write the register(s) (NumReg is the total number of registers written)

- void ADS1243SendRDATAC(void);                   // Read data continuous mode

- void ADS1243SendSTOPC(void);                   // Stop read data continuous mode

- void ADS1243SendSYSOCAL(void);                  // System offset calibration

- void ADS1243SendSYSGCAL(void);                  // System gain calibration

- void ADS1243SendSELFOCAL(void);                 // Self offset calibration

- /* Register Set Value Commands */

- // Relate to MUX0

- int ADS1243SetBurnOutSource(int BurnOut);

- int ADS1243SetChannel(void);      // pMux (0=AINP. 1=AINN); vMux (value of mux channel selection)

- // Relate to VBIAS

- int ADS1243SetBias(unsigned char vBias);

- // Relate to MUX1

- int ADS1243SetIntRef(int sRef);

- int ADS1243SetVoltageReference(int VoltageRef);

- int ADS1243SetSystemMonitor(int Monitor);

- // Relate to SYS0

- int ADS1243SetGain(void);

- int ADS1243SetDataRate(int DataRate);

- // Relate to OFC (3 registers)

- int ADS1243SetOFC(long RegOffset);

- // Relate to FSC (3 registers)

- int ADS1243SetFSC(long RegGain);

- // Relate to IDAC0

- int ADS1243SetDRDYMode(int DRDYMode);

- int ADS1243SetCurrentDACOutput(int CurrentOutput);

- // Relate to IDAC1

- int ADS1243SetIDACRouting(int IDACroute, int IDACdir);      // IDACRoute (0 = I1DIR, 1 = I2DIR)

- // Relate to GPIOCFG

- int ADS1243SetGPIOConfig(unsigned char cdata);

- // Relate to GPIODIR

- int ADS1243SetGPIODir(unsigned char cdata);

- // Relate to GPIODAT

- int ADS1243SetGPIO(unsigned char cdata);

- /* Register Get Value Commands */

- // Relate to MUX0

- int ADS1243GetBurnOutSource(void);

- int ADS1243GetChannel(int cMux);

- int ADS1243GetSpeed (void);

- // Relate to VBIAS

- unsigned char ADS1243GetBias(void);

- //Relate to MUX1

- int ADS1243GetCLKSTAT(void);

- int ADS1243GetIntRef(void);

- int ADS1243GetVoltageReference(void);

- int ADS1243GetSystemMonitor(void);

- // Relate to SYS0

- int ADS1243GetGain(void);

- int ADS1243GetDataRate(void);

- // Relate to OFC (3 registers)

- long ADS1243GetOFC(void);

- // Relate to FSC (3 registers)

- long ADS1243GetFSC(void);

- // Relate to GPIOCFG

- unsigned char ADS1243GetGPIOConfig(void);/*

- * ADS1243.h

- *

- *  Created on: May 31, 2017

- *      Author: COMP-2

- */

- #ifndef ADS1243_H_

- #define ADS1243_H_

- #if defined (__MSP432P401R__)

- /* Definition of GPIO Port Bits Used for Communication */

- // P10.0

- #define ADS1243_CS          0x01

- // P10.2

- #define ADS1243_DIN         0x04

- // P10.3

- #define ADS1243_DOUT        0x08

- // P2.5

- #define ADS1243_DRDY        0x20

- // P10.1

- #define ADS1243_SCLK        0x02

- // P2.4

- #define ADS1243_PDWN        0x10

- #endif

- /* Error Return Values */

- #define ADS1243_NO_ERROR        0

- #define ADS1243_ERROR           -1

- /* ADS1243 Register Definitions */

- #define ADS1243_0_SETUP          0x00

- #define ADS1243_1_MUX            0x01

- #define ADS1243_2_ACR            0x02

- #define ADS1243_3_ODAC           0x03

- #define ADS1243_4_DIO            0x04

- #define ADS1243_5_DIR            0x05

- #define ADS1243_6_IOCON          0x06

- #define ADS1243_7_OCR0           0x07

- #define ADS1243_8_OCR1           0x08

- #define ADS1243_9_OCR2           0x09

- #define ADS1243_10_FSR0          0x0A

- #define ADS1243_11_FSR1          0x0B

- #define ADS1243_12_FSR2          0x0C

- #define ADS1243_13_DOR2          0x0D

- #define ADS1243_14_DOR1          0x0E

- #define ADS1243_15_DOR0          0x0F

- /* ADS1243 Register 0 (SETUP) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //        Factory programmed                     | BOSC      |      PGA[2:0]                  |

- //

- // Define  Burn out current source

- #define ADS1243_BOSC_EN         0x08

- #define ADS1243_BOSC_DS         0x00

- // Define Gain set

- #define ADS1243_PGA_1           0x00

- #define ADS1243_PGA_2           0x01

- #define ADS1243_PGA_4           0x02

- #define ADS1243_PGA_8           0x03

- #define ADS1243_PGA_16          0x04

- #define ADS1243_PGA_32          0x05

- #define ADS1243_PGA_64          0x06

- #define ADS1243_PGA_128         0x07

- /* ADS1243 Register 1 (MUX) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                PSEL[7:4]      |         NSEL[7:4]                          |

- // Positive channel selection

- #define ADS1243_PAIN0          0x00

- #define ADS1243_PAIN1          0x10

- #define ADS1243_PAIN2          0x20

- #define ADS1243_PAIN3          0x30

- #define ADS1243_PAIN4          0x40

- #define ADS1243_PAIN5          0x50

- #define ADS1243_PAIN6          0x60

- #define ADS1243_PAIN7          0x70

- // Negative channel selection

- #define ADS1243_NAIN0          0x00

- #define ADS1243_NAIN1          0x01

- #define ADS1243_NAIN2          0x02

- #define ADS1243_NAIN3          0x03

- #define ADS1243_NAIN4          0x04

- #define ADS1243_NAIN5          0x05

- #define ADS1243_NAIN6          0x06

- #define ADS1243_NAIN7          0x07

- /* ADS1243 Register 2 (ACR) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //  ~DRDY    |DATA FORMATE|    SPEED |BUFFER ENABLE|BIT ORDER| RANGE SELECT|  DATA RATE[1:0]

- //

- #define ADS1243_DRDY_READ       0x80

- #define ADS1243_BIPLAR          0x00

- #define ADS1243_UNIPOLAR        0x40

- #define ADS1243_SPEED_0         0x00

- #define ADS1243_SPEED_1         0x20

- #define ADS1243_BUFEN           0x00

- #define ADS1243_BUFDS           0x10

- #define ADS1243_MSB             0x00

- #define ADS1243_LSB             0x08

- #define ADS1243_SCALE_0         0x00

- #define ADS1243_SCALE_1         0x04

- #define ADS1243_DR_0            0x00

- #define ADS1243_DR_1            0x01

- #define ADS1243_DR_2            0x02

- /* ADS1243 Register 3 (ODAC) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- // SIGN      |              OFFSET VALUE

- #define ADS1243_POSITIVE         0x00

- #define ADS1243_NEGATIVE         0x80

- // OFFSET VALUE SHOULD BE GIVEN BY MANUALLY

- /* ADS1243 Register 4 (DIO) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         DIO[7:0]

- //

- // Define GPIO Direction (0-Output; 1-Input)

- #define ADS1243_DIO_0            0x01

- #define ADS1243_DIO_1            0x02

- #define ADS1243_DIO_2            0x04

- #define ADS1243_DIO_3            0x08

- #define ADS1243_DIO_4            0x10

- #define ADS1243_DIO_5            0x20

- #define ADS1243_DIO_6            0x40

- #define ADS1243_DIO_7            0x80

- /* ADS1243 Register 5 (DIR) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         DIR[7:0]

- //

- // Define GPIO Direction (0-Output; 1-Input)

- #define ADS1243_IO_0            0x01

- #define ADS1243_IO_1            0x02

- #define ADS1243_IO_2            0x04

- #define ADS1243_IO_3            0x08

- #define ADS1243_IO_4            0x10

- #define ADS1243_IO_5            0x20

- #define ADS1243_IO_6            0x40

- #define ADS1243_IO_7            0x80

- /* ADS1243 Register 6  I/O CONFIGRATION (IOCON) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                        IO[7:0]

- //

- // Define GPIO Direction (0-Analog; 1-Data)

- #define ADS1243_IOCON_0            0x01

- #define ADS1243_IOCON_1            0x02

- #define ADS1243_IOCON_2            0x04

- #define ADS1243_IOCON_3            0x08

- #define ADS1243_IOCON_4            0x10

- #define ADS1243_IOCON_5            0x20

- #define ADS1243_IOCON_6            0x40

- #define ADS1243_IOCON_7            0x80

- /* Command Definitions */

- // System Control

- #define ADS1243_CMD_WAKEUP      0xFB

- #define ADS1243_CMD_SLEEP       0xFD

- #define ADS1243_CMD_RESET       0xFE

- #define ADS1243_CMD_DSYNC       0xFC

- // Data Read

- #define ADS1243_CMD_RDATA       0x01

- #define ADS1243_CMD_RDATAC      0x03

- #define ADS1243_CMD_STOPC       0x0F

- // Read Register

- #define ADS1243_CMD_RREG        0x10

- // Write Register

- #define ADS1243_CMD_WREG        0x50

- // Calibration

- #define ADS1243_CMD_SYSOCAL     0xF3

- #define ADS1243_CMD_SYSGCAL     0xF4

- #define ADS1243_CMD_SELFOCAL    0xF1

- #define ADS1243_CMD_SELFGCAL    0xF2

- #define ADS1243_CMD_SELFCAL     0xF0

- /* ADS1243 Register 7 (OCR0) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         OCR0[7:0]

- //

- /* ADS1243 Register 8 (OCR1) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         OCR1[7:0]

- //

- /* ADS1243 Register 9 (OCR2) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         OCR2[7:0]

- //

- /* ADS1243 Register 10 (FSR0) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                        FSR0[7:0]

- //

- /* ADS1243 Register 11 (FSR1) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         FSR1[7:0]

- //

- /* ADS1243 Register 12 (FSR2) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         FSR2[7:0]

- //

- /* ADS1243 Register 15 (DOR0) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                       DOR0[7:0]

- //

- /* ADS1243 Register 14 (DOR1) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         DOR1[7:0]

- //

- /* ADS1243 Register 13 (DOR2) Definition */

- //   Bit 7   |   Bit 6   |   Bit 5   |   Bit 4   |   Bit 3   |   Bit 2   |   Bit 1   |   Bit 0

- //--------------------------------------------------------------------------------------------

- //                                         DOR2[7:0]

- /* Low Level ADS1243 Device Functions */

- void InitConfig(void);                          // Device initialization, setting a default starting condition

- int ADS1243WaitForDataReady(int Timeout);       // DRDY polling, with a maximum polling wait time if needed for break from loop; Timeout=0 is no timeout

- void ADS1243AssertCS(int fAssert);              // Assert/deassert CS (0=low, 1=high)

- void ADS1243SendByte(unsigned char cData );     // Send byte to the ADS1243

- unsigned char ADS1243ReceiveByte(void);         // Receive byte from the ADS1243

- /* ADS1243 Higher Level Functions */

- // Commands as given in datasheet

- void ADS1243SendWakeup(void);                   // Send device Wakeup Command

- void ADS1243SendSleep(void);                    // Set device to Sleep

- void ADS1243SendSync(void);                     // Sync device

- void ADS1243SendResetCommand(void);             // Send a device Reset Command

- long ADS1243ReadData(void);                     // Read the data results by command

- void ADS1243ReadRegister(int StartAddress, int NumReg,unsigned * pData);    // Read the register(s) (NumReg is the total number of registers read)

- void ADS1243WriteRegister(int StartAddress, int NumReg,unsigned * pData);   // Write the register(s) (NumReg is the total number of registers written)

- void ADS1243WriteSequence(int StartAddress, int NumReg,unsigned * pData);   // Write the register(s) (NumReg is the total number of registers written)

- void ADS1243SendRDATAC(void);                   // Read data continuous mode

- void ADS1243SendSTOPC(void);                   // Stop read data continuous mode

- void ADS1243SendSYSOCAL(void);                  // System offset calibration

- void ADS1243SendSYSGCAL(void);                  // System gain calibration

- void ADS1243SendSELFOCAL(void);                 // Self offset calibration

- /* Register Set Value Commands */

- // Relate to MUX0

- int ADS1243SetBurnOutSource(int BurnOut);

- int ADS1243SetChannel(void);      // pMux (0=AINP. 1=AINN); vMux (value of mux channel selection)

- // Relate to VBIAS

- int ADS1243SetBias(unsigned char vBias);

- // Relate to MUX1

- int ADS1243SetIntRef(int sRef);

- int ADS1243SetVoltageReference(int VoltageRef);

- int ADS1243SetSystemMonitor(int Monitor);

- // Relate to SYS0

- int ADS1243SetGain(void);

- int ADS1243SetDataRate(int DataRate);

- // Relate to OFC (3 registers)

- int ADS1243SetOFC(long RegOffset);

- // Relate to FSC (3 registers)

- int ADS1243SetFSC(long RegGain);

- // Relate to IDAC0

- int ADS1243SetDRDYMode(int DRDYMode);

- int ADS1243SetCurrentDACOutput(int CurrentOutput);

- // Relate to IDAC1

- int ADS1243SetIDACRouting(int IDACroute, int IDACdir);      // IDACRoute (0 = I1DIR, 1 = I2DIR)

- // Relate to GPIOCFG

- int ADS1243SetGPIOConfig(unsigned char cdata);

- // Relate to GPIODIR

- int ADS1243SetGPIODir(unsigned char cdata);

- // Relate to GPIODAT

- int ADS1243SetGPIO(unsigned char cdata);

- /* Register Get Value Commands */

- // Relate to MUX0

- int ADS1243GetBurnOutSource(void);

- int ADS1243GetChannel(int cMux);

- int ADS1243GetSpeed (void);

- // Relate to VBIAS

- unsigned char ADS1243GetBias(void);

- //Relate to MUX1

- int ADS1243GetCLKSTAT(void);

- int ADS1243GetIntRef(void);

- int ADS1243GetVoltageReference(void);

- int ADS1243GetSystemMonitor(void);

- // Relate to SYS0

- int ADS1243GetGain(void);

- int ADS1243GetDataRate(void);

- // Relate to OFC (3 registers)

- long ADS1243GetOFC(void);

- // Relate to FSC (3 registers)

- long ADS1243GetFSC(void);

- // Relate to GPIOCFG

- unsigned char ADS1243GetGPIOConfig(void);

- // Relate to GPIODIR

- unsigned char ADS1243GetGPIODir(void);

- // Relate to GPIODAT

- unsigned char ADS1243GetGPIO(void);

- /* Miscellaneous Commands */

- long ADS1243RDATACRead(void);       // reads data directly based on RDATAC mode (writes NOP) and 32 SCLKs

- void InitSPI(void);

- void InitDevice(void);

- #endif /* ADS1243_H_ */

- // Relate to GPIODIR

- unsigned char ADS1243GetGPIODir(void);

- // Relate to GPIODAT

- unsigned char ADS1243GetGPIO(void);

- /* Miscellaneous Commands */

- long ADS1243RDATACRead(void);       // reads data directly based on RDATAC mode (writes NOP) and 32 SCLKs

- void InitSPI(void);

- void InitDevice(void);

- #endif /* ADS1243_H_ */