KeyStone 1

Robust C66 CorePac Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | December 8, 2013 | Andy Yin | Initial Version |
| 1.1 | March 20, 2015 | Brighton Feng | Print device information including device type, speed grade, boot mode, required voltage, ID… |

Contents

1	Introduction	3

2	Test Method	4

2.1	Memory Protection	4

2.2	Error Detection and Correction (ED/ECC)	4

2.3	Other Robust Features	4

2.4	Exception location	5

3	Test cases	6

3.1	Memory Protection	6

3.2	EDC/ECC	7

4	Test code/project	7

4.1	CCS project	7

4.2	To run the test program on KeyStone 1 EVM	9

4.3	Test configuration	10

4.4	Migrate the test to custom board	11

References	11

Appendix: Typical test output	12

Figures

Figure 1.	Robust system on Keystone devices	3

Figure 2.	Directory structure of test project	8

Tables

Table 1.	Different memory protection modules	6

Table 2.	EDC/ECC mechanism implement in keystone memory	7

Table 3.	Source files of the example codes	9

- Introduction

The KeyStone devices provide many features, which can support users to build robust application on it. Following figure shows those features.

- Robust system on Keystone devices

With DSP core, EDMA and SRIO as test master respectively, the STK verifies below features:

1. Memory protection for L1D, L1P, LL2, SL2, DDR3, reserved space and peripherals;

2. L1P ED, EDC for L2, SL2;

3. EDMA error handling;

4. Watch-dog error;

5. Interrupt drop detection.

Refer to “KeyStone_1_Self_Test_Kit_User's_Guide(READ ME FIRST).doc” for some common information about K1 STK.

- Test Method

  - Memory Protection

Configure the memory access permission, when a master access the memory, its permission is checked and exception will happen when it is denied. The STK manually lets one of the DSP core, EDMA or SRIO master to access the memory to trigger exception.

The pseudocode for L1 memory protection is:

Configure the L1 memory as all cache and deny any other masters’ access at initialization;

Access the L1 memory to trigger error;

The pseudocode for LL2 memory protection is:

Modify the related MPPA access permission according to the data address;

Access the protected memory to trigger error;

The pseudocode for SL2/DDR3 memory protection is:

Modify the MPAX configuration;

Access the protected memory to trigger error;

The pseudocode for peripherals configuration space protection is:

Modify the related MPU MPPA access permission according to the address;

Access the protected memory to trigger error;

  - Error Detection and Correction (ED/ECC)

L1P only supports error detection (ED), LL2, SL2 and DDR support error detection and correction. STK manually generates bit error to trigger exception with DSP core or DMA as master.

The pseudocode for EDC test is:

Pend the EDC;

Inject error;

Enable EDC;

Access data to trigger the error;

  - Other Robust Features

Besides the memory protection and EDC/ECC, the watch-dog, EDMA error, and interrupt drop error are verified.

Timer 0 through (N-1) are dedicated to each of the N CorePacs as a watchdog. The STK configures the Timer N in watch-dog mode, and stop servicing the dog to generate NMI exception.

To generate EDMA error, a NULL transfer and an EDMA transfer with destination address 0 are configured and triggered.

To test interrupt drop error, Timer N, N is the number of core, is configured in periodic mode to generate interrupt, but as the interrupt service is disabled, the timer N periodic interrupt is not serviced, and then the interrupt drop error event is generated.

  - Exception location

Normally, exception is captured about 10~100 cycles after the access violation, which depends on many factors including the operation type, the target module which generates exception event... For example, for a write operation to a register protected by MPU, the delay includes: the delay of the write instruction from DSP core to the register; and the delay for the error event routing from the MPU to CIC and then to CorePac exception module. So if the test function is written like below:

test_func()

{

illegal access;  //to trigger the exception

return;

}

The NRP captured by the exception module will be in the location of parent function which call this test function.

To make the NRP correlates to the function which triggers the exception more obviously, in STK, the function is implemented like below:

test_func()

{

illegal access;  //to trigger the exception

asm(" nop 5");

asm(" nop 5");

……

asm(" nop 5"); 	/*NRP should be around the address of this instruction*/

asm(" nop 5");

return;

}

- Test cases

  - Memory Protection

As shown in figure 1, there are many memory protection modules, following table summarizes their difference.

- Different memory protection modules

| Memory protection module | Position in System | page/segment/range |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | Number | Size |  |
| L1D memory protection | Slave port | 16 | 2KB | Each CorePac has one set of L1D, L1P, LL2 and XMC. |
| L1P memory protection | Slave port | 16 | 2KB |  |
| LL2 memory protection | Slave port | 32 | (LL2 size)/32 |  |
| XMC MPAX | Master port | 16 | Programmable |  |
| SMS MPAX | Master port | 8 | Programmable | Each Privilege ID has a pair of SMS and SES. |
| SES MPAX | Master port | 8 | Programmable |  |
| MPU | Slave port | 1~16 | Programmable | Refer to device specific data manual for number of ranges and programmable size for each MPU. |

All the memory protection features in table1 are tested with manually generated error.

For L1 memory protection, L1 are configured as all cache and all the memory protection page attribute registers are cleared to deny any other masters’ access. EDMA and DSP core are used as master to access the L1, and then the L1 memory protection exception is triggered.

For LL2 memory protection, EDMA is used as master to trigger exception.

For XMC MPAX, DSP core is used as master to access the protected SL2 and DDR memory to trigger exception.

For SES/SMS MPAX, EDMA and SRIO are used as master to access the protected SL2/DDR to trigger exception.

The MPU is used to protect the configure space of slave peripherals. The STK configures MPU0 to deny all the access to PLL registers, and exception is generated when DSP core access the one of the PLL registers. The STK also configures the MPU1 and MPU2 to prevent some queues from being accessed by DSP cores, and then triggers expectation by DSP core accessing.

  - EDC/ECC

EDC (Error Detection and Correction) or ECC (Error Checking and Correcting) are designed for soft error. EDC or ECC mechanism are implemented through keystone all levels of memory, following table compares these mechanism in different memory modules:

- EDC/ECC mechanism implement in keystone memory

| memory | error detection | Error correction | Segment line size |
| --- | --- | --- | --- |
| L1P | 1 bit | N/A | 64bit |
| CorePac L2 | 2 bit | 1bit | 128bit |
| Shared L2 (MSMC) | 2 bit | 1bit | 256bit |
| DDR | 2 bit | 1bit | 64bit |

For L1P, one bit error is injected into L1P cache, IDMA is used to access it to trigger exception.

For LL2, 1, 2 and 3 bit error are injected into LL2 code and data space, IDMA and DSP core are used to access them to trigger exception.

For SL2, one bit error is injected into SL2 code and data space, EDMA and DSP core are used to trigger the exception.

- Test code/project

This section introduces the implementation of these robust features tests on KeyStone 1 DSP include the C6678, C6670, TCI6614.

  - CCS project

Following figure shows the directory structure of the test project.

- Directory structure of test project

The project files are in “Robust” folder. There is some commonly used initialization and driver code for DDR, EDMA and PLL… in “common” folder. The main source code files are in the “Robust\src” subfolder. Below table describes these source files.

- Source files of the example codes

| Source files | Functions | Descriptions |
| --- | --- | --- |
| Keystone_common.c/h | L1P_EDC_setup LL2_EDC_setup LL2_EDC_scrubbing_timer_setup KeyStone_SL2_EDC_enable Robust_Interrupts_Init KeyStone_MSMC_MP_interrupt_en KeyStone_MPU_interrupts_enable KeyStone_Exception_cfg | Initialization for EDC, ECC and interrupt controller mapping. |
| Robust_xxx.c/h | L1D_MP_test L1P_MP_test LL2_MP_test XMC_SL2_MP_test XMC_DDR_MP_test SMS_SL2_MP_test SES_DDR_MP_test peripheral_MP_test Reserved_Space_Test | Implementation of below memory protection features. L1D, L1P, LL2, XMC_SL2, XMC_DDR, SMS_SL2, SES_DDR, Slave MPU, reserve space protection |
|  | L1P_ED_test LL2_EDC_test SL2_EDC_test DDR_ECC_test | Implementation of EDC/ECC features. L1P ED, LL2 EDC, SL2 EDC, DDR ECC |
|  | watch_dog_test EDMA_ERROR_test INT_drop_test | Implementation of other robust features. Watch-dog error, EDMA error handling, Interrupt drop error. |

  - To run the test program on KeyStone 1 EVM

The test program can be run on EVMs of C6670, C6678, or TCI6614.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- import the project to CCS

- build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_C6678_1_1_2_6\packages\ti\csl

- Set the boot mode of the device on EVM to no boot.

- load the program to core 0 of a DSP.

- run the core, then you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

There are multiple macros defined in the source code to configure the test.

Each robust feature can be configured to be tested or not through following macros in “Robust_System.c”. The value of “1” means enable the test, and “0” means disable the test.

#define L1D_MP_TEST 		1

#define L1P_MP_TEST 		1

#define LL2_MP_TEST 		1

#define XMC_SL2_MP_TEST 	1

#define XMC_DDR_MP_TEST 	1

#define SMS_SL2_MP_TEST 	1

#define SES_DDR_MP_TEST 	1

#define PERIPHERAL_MP_TEST 	1

#define L1P_ED_TEST 		1

#define LL2_EDC_TEST 		1

#define SL2_EDC_TEST 		1

#define DDR_ECC_TEST 		0 	/*only for EVM with ECC memory*/

#define EDMA_ERROR_TEST 	1

#define WATCH_DOG_TEST 		1

/*reserved space test can not safely return, thus other tests after this will not run,

to run following tests, this test must be disabled.*/

#define RESERVED_SPACE_TEST 1

/*interrup drop test can not safely return, thus other tests after this will not run,

to run following tests, this test must be disabled.*/

#define INT_DROP_TEST 		1

#define BIT_PATTERN_FILLING_TEST 		1

#define ADDRESS_TEST 			1

#define BIT_WALKING_TEST 			1

It is possible that the test program stops after one of the test cases with following output information.

Exception happened at a place can not safely return!

This is a normal behavior. To run other test cases after this case, this test case should be disabled and rebuild/rerun the test.

  - Migrate the test to custom board

These tests are implemented based on TI’s EVM boards. In your real system, the DDR configuration may be changed according to your hardware design in the function KeyStone_DDR_init() in “KeyStone_DDR_Init.c”, and it is need to change the input parameters of the function KeyStone_DDR_init.

The DSP core speed may also be changed therefore it is need to change the input parameters of the function KeyStone_main_PLL_init.

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- Memory Protection On Keystone Devices (SPRWIKI9012)

- TMS320C66x DSP CPU and Instruction Set Reference Guide (SPRUGH7)

- TMS320C66x DSP CorePac User Guide (SPRUGW0)

- KeyStone Architecture Multicore Shared Memory Controller User Guide (SPRUGW7)

- KeyStone Architecture Memory Protection Unit (MPU) User Guide (SPRUGW5)

- KeyStone Architecture Enhanced Direct Memory Access (EDMA3) Controller User Guide (SPRUGS5)

- KeyStone Architecture Timer64 User Guide (SPRUGV5)

- KeyStone Architecture Chip Interrupt Controller (CIC) User Guide (SPRUGW4)

- KeyStone Architecture DDR3 Memory Controller’s user guider (SPRUGV8)

- TMS320C6000 Optimizing Compiler User's Guide (SPRU187)

- “Interrupts”, “MPU” and “Memory Map Summary” sections in Device specific Data Manuals

Appendix: Typical test output

Below is the test output on C6678 EVM.

JTAG ID= 0x1009e02f. This is C6678/TCI6608 device, version variant = 1.

DEVSTAT= 0x00010081. little endian, No boot or EMIF16(NOR FLASH) or UART boot, PLL configuration implies the input clock for core is 50MHz.

SmartReflex VID= 47, required core voltage= 1.001V.

Die ID= 0x02011014, 0x04010169, 0x00000000, 0x40680021

Device speed grade = 1000MHz.

Initialize DSP main clock = 100.00MHz/1x10 = 1000MHz

Initialize DDR speed = 66.67MHzx/1x20 = 1333.333MTS

Enable Exception handling...

SRIO path configuration 4xLaneABCD

===================-L1D memory protection test-==================

!!!DMA write to L1D at 0x00F00000...

External exception happened. MEXPFLAG[2]=0x80000000.

External exception happened. MEXPFLAG[3]=0x8000000.

EDMA module 0 error

TC 0 error happened

EDMA3TC has detected an error at source or destination address.

write error (10). TCC= 0. TCINTEN= 1. TCCHEN= 0

Event 123: DMC_DMPA DMA memory protection fault for L1D

memory protection exception caused by master with ID 0 at 0xf00000

Supervisor Write violation

NRP=0xc015138, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xbabfa7

B3=0xc0150e0, A4=0x1070, B4= 0x2701010, B14= 0x83a050, B15= 0x838f88

!!!DSP core write to L1D at 0x00F00100...

External exception happened. MEXPFLAG[3]=0x4000000.

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0xf00100

Supervisor Write violation

NRP=0xc01516a, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xbc4345

B3=0xc015150, A4=0xf00100, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f88

!!!DSP core write to gpCGEM_regs->L1DMPPA[0] register at 0x184ae40 without unlocking...

External exception happened. MEXPFLAG[3]=0x4000000.

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0x184ae40

Supervisor Write violation

NRP=0xc0151aa, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xbd66c1

B3=0xc015194, A4=0x59, B4= 0x184ae40, B14= 0x83a050, B15= 0x838f88

===================-L1P memory protection test-==================

!!!DMA write to L1P at 0x00E00000...

External exception happened. MEXPFLAG[2]=0x80000000.

External exception happened. MEXPFLAG[3]=0x2000000.

EDMA module 0 error

TC 0 error happened

EDMA3TC has detected an error at source or destination address.

write error (10). TCC= 0. TCINTEN= 1. TCCHEN= 0

Event 121: PMC_DMPA DMA memory protection fault for L1P

memory protection exception caused by master with ID 0 at 0xe00000

Supervisor Write violation

NRP=0xc01502a, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xbe57e7

B3=0xc014fd0, A4=0x1070, B4= 0x2701010, B14= 0x83a050, B15= 0x838f90

!!!DSP core write to gpCGEM_regs->L1PMPPA[0] register at 0x184a640 without unlocking...

External exception happened. MEXPFLAG[3]=0x1000000.

Event 120: PMC_CMPA CPU memory protection fault for L1P

memory protection exception caused by local access at 0x184a640

Supervisor Write violation

NRP=0xc015074, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xc00db3

B3=0xc015058, A4=0x59, B4= 0xffffffff, B14= 0x83a050, B15= 0x838f90

===================-LL2 memory protection test-==================

EDMA write to 0x10828000, which is allowed for external access...

!!!EDMA write to 0x10820000, which is protected against external access...

External exception happened. MEXPFLAG[2]=0x80000000.

External exception happened. MEXPFLAG[3]=0x20000000.

EDMA module 0 error

TC 0 error happened

EDMA3TC has detected an error at source or destination address.

write error (10). TCC= 0. TCINTEN= 1. TCCHEN= 0

Event 125: UMC_DMPA DMA memory protection fault for L2

memory protection exception caused by master with ID 0 at 0x820000

Supervisor Write violation

NRP=0xc010084, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xc194ef

B3=0xc01002a, A4=0x1070, B4= 0x2701010, B14= 0x83a050, B15= 0x838f78

!!!DSP core write to gpCGEM_regs->L2MPPA[0] register at 0x184a200 without unlocking...

External exception happened. MEXPFLAG[3]=0x10000000.

Event 124: UMC_CMPA CPU memory protection fault for L2

memory protection exception caused by local access at 0x184a200

Supervisor Write violation

NRP=0xc0100c6, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xc34a45

B3=0xc0100b0, A4=0x58, B4= 0x184a200, B14= 0x83a050, B15= 0x838f78

===================-XMC SL2 memory protection test-==============

DSP core write to 0xc110400 which is write-only...

DSP core read from 0xc12c808 which is read-only...

!!!DSP core read from 0xc11883f which is write-only...

External exception happened. MEXPFLAG[3]=0x4000000.

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0xc11883f

Supervisor Read violation

NRP=0xc016616, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xc9a65a

B3=0xc016608, A4=0x38, B4= 0xc, B14= 0x83a050, B15= 0x838f80

!!!DSP core write to 0xc12d44c which is read-only...

External exception happened. MEXPFLAG[3]=0x4000.

Event 110: MDMAERREVT XMC VBUSM error event

memory protection exception caused by local access at 0xc12d440

Supervisor Write violation

NRP=0xc01664e, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xcab1e5

B3=0xc01663c, A4=0x36, B4= 0x11, B14= 0x83a050, B15= 0x838f80

!!!DSP core write to gpXMC_regs->XMPAX[0].XMPAXL register at 0x8000000 without unlocking...

External exception happened. MEXPFLAG[3]=0x4000.

Event 110: MDMAERREVT XMC VBUSM error event

memory protection exception caused by local access at 0x8000000

Supervisor Write violation

NRP=0xc01668a, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xcbcfc5

B3=0xc016670, A4=0xffffffff, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

===================-XMC DDR memory protection test-==============

DSP core write to 0x81000400 which is write-only...

DSP core read from 0x81020808 which is read-only...

!!!DSP core read from 0x8100883f which is write-only...

External exception happened. MEXPFLAG[3]=0x4000000.

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0x8100883f

Supervisor Read violation

NRP=0xc016d26, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xd4d35e

B3=0xc016d14, A4=0x39, B4= 0xffffff81, B14= 0x83a050, B15= 0x838f80

!!!DSP core write to 0x8102144c which is read-only...

External exception happened. MEXPFLAG[3]=0x4000.

Event 110: MDMAERREVT XMC VBUSM error event

memory protection exception caused by local access at 0x81021440

Supervisor Write violation

NRP=0xc016d62, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xd5e133

B3=0xc016d4c, A4=0x37, B4= 0x8102144c, B14= 0x83a050, B15= 0x838f80

===================-SMS SL2 memory protection test-==============

EDMA copy from 0xc128000 (read-only) to 0xc100000 (write-only)...

!!!EDMA copy from 0xc108000 (write-only) to 0xc129000 (read-only)...

External exception happened. MEXPFLAG[0]=0x400.

External exception happened. MEXPFLAG[2]=0x80000000.

EDMA module 0 error

TC 0 error happened

EDMA3TC has detected an error at source or destination address.

read error (2). TCC= 0. TCINTEN= 1. TCCHEN= 0

Event 10 : MSMC_mpf_error_n MSMC Memory protection fault indicators for local CorePac

memory protection exception caused by master 16 (PrivID= 0) at address 0xc108000

NRP=0xc010a90, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xdc25c1

B3=0xc010a64, A4=0x1070, B4= 0x2701010, B14= 0x83a050, B15= 0x838f30

!!!DSP core write to gpMSMC_regs->SMS_MPAX_PER_PRIVID[0].SMS[0].MPAXL register at 0xbc00200 without unlocking...

External exception happened. MEXPFLAG[3]=0x4000.

Event 110: MDMAERREVT XMC VBUSM error event

MDMA write status error detected

XID (Transaction ID)= 14

Privilege error

NRP=0xc010afa, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xddf8d8

B3=0xc010ad0, A4=0x72, B4= 0xbc00200, B14= 0x83a050, B15= 0x838f30

--------------------SRIO SMS SL2 memory protection test---------------

SRIO copy from 0xc12a000 (read-only) to 0xc108000 (write-only)...

!!!SRIO copy from 0xc12a000 (read-only) to 0xc129000 (read-only)...

External exception happened. MEXPFLAG[2]=0x80000000.

memory protection exception caused by master 54 (PrivID= 9) at address 0xc129000

NRP=0xc010d6c, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xdf3f1b

B3=0xc010d3a, A4=0x838f54, B4= 0xc12a000, B14= 0x83a050, B15= 0x838f30

===================-SES DDR memory protection test-==============

EDMA copy from 0x81024000 (read-only) to 0x81010000 (write-only)...

!!!EDMA copy from 0x81018000 (write-only) to 0x81025000 (read-only)...

External exception happened. MEXPFLAG[0]=0x400.

External exception happened. MEXPFLAG[2]=0x80000000.

EDMA module 0 error

TC 0 error happened

EDMA3TC has detected an error at source or destination address.

read error (1). TCC= 0. TCINTEN= 1. TCCHEN= 0

Event 10 : MSMC_mpf_error_n MSMC Memory protection fault indicators for local CorePac

memory protection exception caused by master 16 (PrivID= 0) at address 0x81018000

NRP=0xc010410, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xe83d3a

B3=0xc0103e4, A4=0x1070, B4= 0x2701010, B14= 0x83a050, B15= 0x838f28

!!!DSP core write to gpMSMC_regs->SES_MPAX_PER_PRIVID[0].SES[0].MPAXL register at 0xbc00600 without unlocking...

External exception happened. MEXPFLAG[3]=0x4000.

Event 110: MDMAERREVT XMC VBUSM error event

MDMA write status error detected

XID (Transaction ID)= 4

Privilege error

NRP=0xc01047a, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xea0dea

B3=0xc010450, A4=0x72, B4= 0xbc00600, B14= 0x83a050, B15= 0x838f28

--------------------SRIO SES DDR3 memory protection test---------------

SRIO copy from 0x81026000 (read-only) to 0x81018000 (write-only)...

!!!SRIO copy from 0x81026000 (read-only) to 0x81025000 (read-only)...

External exception happened. MEXPFLAG[2]=0x80000000.

memory protection exception caused by master 54 (PrivID= 9) at address 0x81025000

NRP=0xc0106ec, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xeb55a3

B3=0xc0106ca, A4=0x838f4c, B4= 0x81026000, B14= 0x83a050, B15= 0x838f28

===================-peripherals protection test-=================

!!!Write to PLL register at 0x02310110 which is protected by MPU0...

External exception happened. MEXPFLAG[2]=0x80000000.

External exception happened. MEXPFLAG[3]=0x80000000.

MPU 0 protection violation error

The MSTID 8 (PRIVID 0) triggered MPU error at 0x2310110 with secure access

Supervisor write fault!

Event 127: EMC_BUSERR Bus Error Interrupt for global configuration space between 0x01C00000 - 0x07FFFFFF

CFG write status error detected

XID (Transaction ID)= 14

Privilege error

NRP=0xc016866, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xec842a

B3=0xc016844, A4=0x45, B4= 0x2310110, B14= 0x83a050, B15= 0x838f90

MPU 2 setup 11 of 16 ranges.

DSP core Pop descriptor from queue 2049 at 0x2a2801c in a read-only range...

!!!DSP core Push descriptor to queue 2049 at 0x2a2801c in a read-only range...

External exception happened. MEXPFLAG[2]=0x80000000.

External exception happened. MEXPFLAG[3]=0x80000000.

MPU 2 protection violation error

The MSTID 8 (PRIVID 0) triggered MPU error at 0x2a2801c with secure access

Supervisor write fault!

Event 127: EMC_BUSERR Bus Error Interrupt for global configuration space between 0x01C00000 - 0x07FFFFFF

CFG write status error detected

XID (Transaction ID)= 12

Privilege error

NRP=0xc0167b0, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xee9045

B3=0xc016794, A4=0x50, B4= 0xc133980, B14= 0x83a050, B15= 0x838f60

DSP core Push descriptor to queue 2049 at 0x3402801c in a writeable range...

====================L1P ED test=================================

!!!manually generate one bit error in L1P cache for function at 0x800860, and then execute it...

internal excpetion happened. IERR=0x9.

Instruction fetch exception

Opcode exception

L1P Cache parity check error caused by program fetch at address 0x800860

NRP=0x800842, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xf0888d

B3=0xc01639c, A4=0x1820100, B4= 0x1820100, B14= 0x83a050, B15= 0x838f60

!!!manually generate one bit error in L1P cache at 0xe00860, and then read it by DMA...

External exception happened. MEXPFLAG[3]=0x20000.

Event 113: PMC_ED Single bit error detected during DMA read

L1P RAM parity check error caused by DMA at address 0xe00860

NRP=0xc016430, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xf1c0eb

B3=0xc01641a, A4=0xe00840, B4= 0x1820100, B14= 0x83a050, B15= 0x838f60

===================-LL2 EDC test-================================

--------------------LL2 data EDC test----------------------------

!!!manually generate 3 bit error in data at 0x83a2e8, and then read it...

External exception happened. MEXPFLAG[3]=0x200000.

Event 117: UMC_ED2 Uncorrected bit error detected

LL2 EDC error (non-correctable) at address 0x83a2e0 caused by L1D access.

total non-correctable error number= 1, total correctable error number= 0.

NRP=0xc00f282, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xf2e3c4

B3=0xc00f1b2, A4=0x1, B4= 0x7, B14= 0x83a050, B15= 0x838f60

!!!manually generate 2 bit error in data at 0x83a2e8, and then read it...

External exception happened. MEXPFLAG[3]=0x200000.

Event 117: UMC_ED2 Uncorrected bit error detected

LL2 EDC error (non-correctable) at address 0x83a2e0 caused by L1D access.

total non-correctable error number= 2, total correctable error number= 0.

NRP=0xc00f354, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xf434b5

B3=0xc00f2a6, A4=0x10001, B4= 0x3, B14= 0x83a050, B15= 0x838f60

!!!manually generate one bit error in data at 0x83a2e8, and then read it...

External exception happened. MEXPFLAG[3]=0x200000.

Event 117: UMC_ED2 Uncorrected bit error detected

LL2 EDC error (non-correctable) at address 0x83a2e0 caused by L1D access.

total non-correctable error number= 3, total correctable error number= 0.

NRP=0xc00f422, NTSR=0x1000d, IRP=0x0, ITSR=0x0, TSCH= 0x0, TSCL= 0xf587b6

B3=0xc00f37a, A4=0x1840044, B4= 0x1, B14= 0x83a050, B15= 0x838f60

!!!scrub the corrupted data to fix the error...

LL2 EDC (correctable) at bit 64 of address 0x83a2e0 caused by DMA access.

total non-correctable error number= 3, total correctable error number= 1.

IRP= 0xc00f4ae, ITSR= 0xd. TSCH= 0x0, TSCL= 0xf6ab73

read the data again...(no error happens again)

--------------------LL2 code EDC test----------------------------

!!!manually generate one bit error in a function at 0x800820, and then execute the function...

LL2 EDC (correctable) at bit 13 of address 0x800820 caused by L1P access.

total non-correctable error number= 3, total correctable error number= 3.

IRP= 0x800806, ITSR= 0xd. TSCH= 0x0, TSCL= 0xf7a2a3

one bit error was corrected with previous execution. Execute the function again...(no error happens again)

===================-SL2 EDC test-================================

!!!manually generate one bit error in a function at 0xc019c80, and then execute the function...

SL2 Correctable error occurred at bit 13 of address 0xc019c80 by PrivID 0 (from C66x CorePacs)

IRP= 0xc019c74, ITSR= 0xd. TSCH= 0x0, TSCL= 0xf8b3f0

one bit error was corrected with previous execution. Execute the function again...(no error happens again)

!!!manually generate one bit error data at 0xc137000, and then read it...

SL2 Correctable error occurred at bit 8 of address 0xc137000 by PrivID 0 (from C66x CorePacs)

IRP= 0xc011050, ITSR= 0xf. TSCH= 0x0, TSCL= 0xf96a4b

!!!manually generate one bit error data at 0xe0000000, and then read it...

SL2 Correctable error occurred at bit 8 of address 0xc137000 by PrivID 0 (from C66x CorePacs)

IRP= 0xc011050, ITSR= 0xf. TSCH= 0x0, TSCL= 0xfa1a3f

===================-EDMA Error test-===========================

!!!start a NULL transfer with EDMA CC0 Channel 0, to trigger event miss error...

External exception happened. MEXPFLAG[2]=0x80000000.

EDMA module 0 error

EDMA Channel 0 event missed.

NRP=0xc016e18, NTSR=0x1000f, IRP=0xc011050, ITSR=0xf, TSCH= 0x0, TSCL= 0xfaa1cd

B3=0xc016da4, A4=0x404, B4= 0x2704004, B14= 0x83a050, B15= 0x838f98

!!!start a transfer to address 0 with EDMA CC1 Channel 2, to trigger EDMA bus error...

External exception happened. MEXPFLAG[2]=0x80000000.

EDMA module 1 error

TC 2 error happened

EDMA3TC has detected an error at source or destination address.

read error (1). TCC= 2. TCINTEN= 1. TCCHEN= 0

NRP=0xc016ed4, NTSR=0x1000f, IRP=0xc011050, ITSR=0xf, TSCH= 0x0, TSCL= 0xfb8db7

B3=0xc016e78, A4=0x1070, B4= 0x2721010, B14= 0x83a050, B15= 0x838f98

===================-watch dog timer test-========================

start watch-dog timer...

service watch-dog 1 times, at time counter = 6

service watch-dog 2 times, at time counter = 3916328

service watch-dog 3 times, at time counter = 3924782

service watch-dog 4 times, at time counter = 3953090

service watch-dog 5 times, at time counter = 3913825

service watch-dog 6 times, at time counter = 3971613

service watch-dog 7 times, at time counter = 3930823

service watch-dog 8 times, at time counter = 4104923

service watch-dog 9 times, at time counter = 3996417

service watch-dog 10 times, at time counter = 3950503

stop servicing watch-dog, it will timeout and trigger NMI after 3000 ms...

NMI exception happened, normally you should reset the DSP to recover from the problem!

NRP=0xc0171d0, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x0, TSCL= 0xb0ae07c6

B3=0xc0171c4, A4=0x4b, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f50

==================-Reserved space access test-=================

--------------------access zero address------------------------

!!!read from reserved address 0x0...

External exception happened. MEXPFLAG[3]=0x4000000.

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0x0

Supervisor Read violation

NRP=0xc015260, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2b9dcc45

B3=0xc015250, A4=0x524005, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

!!!write to reserved address 0x0...

External exception happened. MEXPFLAG[3]=0x10000000.

Event 124: UMC_CMPA CPU memory protection fault for L2

memory protection exception caused by local access at 0x0

Supervisor Write violation

NRP=0xc0151f0, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2b9ed11d

B3=0xc0151e2, A4=0x25, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

--------------------access address exceed LL2------------------

!!!read from reserved address 0xa00000...

External exception happened. MEXPFLAG[3]=0x4000000.

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0xa00000

Supervisor Read violation

NRP=0xc015260, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2b9fd842

B3=0xc015250, A4=0x524005, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

!!!write to reserved address 0xa00000...

External exception happened. MEXPFLAG[3]=0x10000000.

Event 124: UMC_CMPA CPU memory protection fault for L2

memory protection exception caused by local access at 0xa00000

Supervisor Write violation

NRP=0xc0151f0, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2ba0df42

B3=0xc0151e2, A4=0x2a, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

------------access local reserved configuration space----------

!!!read from reserved address 0x1000000...

External exception happened. MEXPFLAG[3]=0x4000000.

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0x1000000

Supervisor Read violation

NRP=0xc015260, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2ba1e7f9

B3=0xc015250, A4=0x524005, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

!!!write to reserved address 0x1000000...

External exception happened. MEXPFLAG[3]=0x40000000.

Event 126: EMC_CMPA CPU memory protection fault for local configuration space between 0x01000000-0x01BFFFFF

memory protection exception caused by local access at 0x1000000

Supervisor Write violation

NRP=0xc0151ee, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2ba2ef86

B3=0xc0151e2, A4=0x2b, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

---access reserved cfg space between INTC and power control----

!!!read from reserved address 0x18001c4...

External exception happened. MEXPFLAG[3]=0x4000000.

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0x18001c4

Supervisor Read violation

NRP=0xc015260, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2ba3fc1e

B3=0xc015250, A4=0xc015338, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

!!!write to reserved address 0x18001c4...

External exception happened. MEXPFLAG[3]=0x800000.

Event 119: SYS_CMPA CPU Memory Protection Fault for local configuration of INTC and power control

NRP=0xc0151f0, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2ba503e7

B3=0xc0151e2, A4=0x2b, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

------------access global reserved configration space----------

!!!read from reserved address 0x1cf0000...

External exception happened. MEXPFLAG[3]=0x84000000.

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0x1cf0000

Supervisor Read violation

Event 127: EMC_BUSERR Bus Error Interrupt for global configuration space between 0x01C00000 - 0x07FFFFFF

CFG read status error detected

XID (Transaction ID)= 12

Addressing error

NRP=0xc01525a, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2ba5db7e

B3=0xc015250, A4=0x524005, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

!!!write to reserved address 0x1cf0000...

External exception happened. MEXPFLAG[3]=0x80000000.

Event 127: EMC_BUSERR Bus Error Interrupt for global configuration space between 0x01C00000 - 0x07FFFFFF

CFG write status error detected

XID (Transaction ID)= 15

Addressing error

NRP=0xc015210, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2ba717f8

B3=0xc0151e2, A4=0x2b, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

-------------access reserved XMC configuration space-----------

!!!read from reserved address 0x8000100...

External exception happened. MEXPFLAG[3]=0x4004000.

Event 110: MDMAERREVT XMC VBUSM error event

MDMA read status error detected

XID (Transaction ID)= 1

Privilege error

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0x8000100

Supervisor Read violation

NRP=0xc01525a, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2ba819fd

B3=0xc015250, A4=0x524005, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

!!!write to reserved address 0x8000100...

External exception happened. MEXPFLAG[3]=0x4000.

Event 110: MDMAERREVT XMC VBUSM error event

memory protection exception caused by local access at 0x8000100

Supervisor Write violation

NRP=0xc0151f2, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2ba950b9

B3=0xc0151e2, A4=0x2b, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

-----------------access XMC unmapped space---------------------

!!!read from reserved address 0x1c000000...

External exception happened. MEXPFLAG[3]=0x4004000.

Event 110: MDMAERREVT XMC VBUSM error event

MDMA read status error detected

XID (Transaction ID)= 9

Privilege error

Event 122: DMC_CMPA CPU memory protection fault for L1D (and other memory read finally goes through the L1D controller)

memory protection exception caused by local access at 0x1c000000

Supervisor Read violation

NRP=0xc01525a, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2baa5834

B3=0xc015250, A4=0x524005, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

!!!write to reserved address 0x1c000000...

External exception happened. MEXPFLAG[3]=0x4000.

Event 110: MDMAERREVT XMC VBUSM error event

memory protection exception caused by local access at 0x1c000000

Supervisor Write violation

NRP=0xc0151f4, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2bab8f54

B3=0xc0151e2, A4=0x2c, B4= 0xc019ac0, B14= 0x83a050, B15= 0x838f80

-----------------execute from reserved space-------------------

!!!execute at reserved address 0x0...

internal excpetion happened. IERR=0x1.

Instruction fetch exception

NRP=0x0, NTSR=0x1000d, IRP=0xc011050, ITSR=0xf, TSCH= 0x1, TSCL= 0x2bac95ad

B3=0xc0153f0, A4=0x26, B4= 0x0, B14= 0x83a050, B15= 0x838f90

Exception happened at a place can not safely return!