#!/bin/sh

# timer_run.sh

#

# This script configures Timer4 to be run

# on the VP1 Board. After configured,

# the timer runs indefinitely at a fixed

# freq and duty cycle, fed from the

# internal CLK_M_OSC clock

#

echo "Configuring Timer 7...\n"

#configurations changed for Timer6

# Enable Timer7's clock in CM_PER

# This allows us to access it's control regs

#devmem2 0x44e00088 w 0x2

# Set the Pinmux for Timer7 (0x2 because its mode 2 of this pin)

# Using the pin mux linux driver from the SDK

# echo 0x22 > /sys/kernel/debug/omap_mux/xdma_event_intr0

#Select TIMER7 CLOCK SOURCE

#devmem2 0x44e0051c w 0x1

devmem2 0x4804a004 w 0x1

#Enable timer to access its ctrl regs

#devmem2 0x44e000f0 w 0x2

devmem2 0x4804a07c w 0x2

# Timer configurations.. Stop Timer.

# Auto-Reload

# Compare Mode Enabled

# Trigger set on overflow

# Toggle Mode

#devmem2 0x48044038 w 0x1842

#devmem2 0x48048038 w 0x68

devmem2 0x4804a038 w 0x1443

# Write to the CM_DPLL registers to select the clk source

# Selected the clk source for the Timer7

# This should be &= (default 0x12) (writing a 0x2 for 32kHz)

#0x1 = SEL2 : Select CLK_M_OSC clock

#devmem2 0x44e00510 w 0x1

#devmem2 0x44e0051c w 0x1

# Initial count value

#devmem2 0x4804403C w 0xFFFFAFFF

# Set the re-load value

#devmem2 0x48044040 w 0xFFFFDFFF

#devmem2  0x48048040 w 0xFFFFFD39

devmem2  0x4804a040 w 0xFFFFFD39

# Setting a duty cycle to 50.7%

#devmem2 0x4804404C w 0xFFFFFAF0

#devmem2 0x4804804c w 0xFFFFFE9C

devmem2 0x4804a04c w 0xFFFFFE9C

#set trigger

#devmem2 0x48048044 w 0x00000000

devmem2 0x4804a044 w 0x00000000

# Force the Timer to Smart-Idle

#devmem2 0x48044010 w 0x8

#devmem2 0x48048010 w 0x8

devmem2 0x4804a010 w 0x8

# Start Timer

#devmem2 0x48048038 w 0x186B

devmem2 0x4804a038 w 0x186B

# Verify that the timer is running,

# by polling the count register a

# few times. We should we up counting.

for i in {1..2}

do

echo "Timer Value $i"

#devmem2 0x4804803C

devmem2 0x4804a03C

# End

echo "Timer is Running!"

echo ""

#echo "See TP31 on the EVM"

echo ""

# Delay added to allow time for the first timer

# overflow to occur, before any consecutive memory

# accesses are made. (Just for standby testing).

sleep 1

done

# Disable MODULEMODE of CM_PER_TIMER4_CLKCTRL register

# Only if this value is 0, are we able to wake-up from standby??

#devmem2 0x44e000f0 w 0x0

devmem2 0x4804a07c w 0x0

#END