Code Changes to NIMU and Explanation

- Updated NIMUDeviceTable to include an initialization function for a second interface

- EMACInit_Core:

  - Create static global variable called PhysIdx which is used to set SGMII physical index.  Also used to gate which interface is initialized

  - MAC address needs to be unique for separate interface to prevent packet misdirection.  LSB of the MAC is incremented to make it unique

  - When setting driver interface functions, pointed the second interface to a different send function.

  - PA should only have to be initialized once

- Setup_Rx

  - Each interface needed its own receive queue, so gRxQHnd was made into an array, using PhysIdx as the index

  - Changed accumulator channel number so each interface has a separate one.  The +8 comes from usable accumulator channels on each core

  - Registered a new interrupt/event for the second interface

- EmacSendAMC

  - Needed to change physical index that send is using (in regular EmacSend, this PhysIdx is set to 0)

- EmacRxPktAmcISR

  - Same as ISR that is used for first port except event and accumulator numbers are changed to match those that the second interface uses.

- Qmss_QueueHnd gPaTxQHnd [NUM_PA_TX_QUEUES], gTxReturnQHnd[2], gTxFreeQHnd[2], gRxFreeQHnd[2], gRxQHnd[2], gTxCmdReturnQHnd[2], gTxCmdFreeQHnd[2];

  - The above variables were changed into arrays with two entries (one for each interface).  This requires several other parts in the code to change as well

  - Setup_Rx

    - When these queues are used in Setup_Rx, they are indexed by [ptr_pvt_data->pdi.PhysIdx] which indicates the interface

  - Setup_Tx

    - Loop over the number of interfaces, indexing queues such as gTxFreeQHnd[index], thus setting up the queues for each interface

  - EMACSend/EMACSendAMC

    - Since each one of these is exclusive to an interface, the number used to index these queues is simply the one I know it is supposed to be (0 or 1)

  - EmacRxPktISR/EmacRxPktAMCISR

    - Once again index by [ptr_pvt_data->pdi.PhysIdx]

  - Add_MACAddress

    - Queues are indexed by [ethInfo->inport] which describes the appropriate interface

  - EmacStop

    - Queues are indexed by [ptr_pvt_data->pdi.PhysIdx] since each interface will call EmacStop