Step 1: Follow the http://processors.wiki.ti.com/index.php/AM572x_GP_EVM_Hardware_Setup to setup the AM572x GP EVM.

Step 2: Connect CortexA15_0, below should be seen for the SOC initialization by GEL file from CCS console:

IcePick_D: GEL Output: Ipu RTOS is released from Wait-In-Reset.

IcePick_D: GEL Output: Ipu SIMCOP is released from Wait-In-Reset.

IcePick_D: GEL Output: IVAHD C66 is released from Wait-In-Reset.

IcePick_D: GEL Output: IVAHD ICONT1 is released from Wait-In-Reset.

IcePick_D: GEL Output: IVAHD ICONT2 is released from Wait-In-Reset.

CS_DAP_DebugSS: GEL Output: --->>> CONFIGURE DEBUG DPLL settings to 1.9 GHZs  <<<---

CS_DAP_DebugSS: GEL Output: > Setup DebugSS 1.9GHz in progress...

CS_DAP_DebugSS: GEL Output: < Done with Setup DebugSS Trace export clock (TPIU) to 97MHz

CS_DAP_DebugSS: GEL Output: < Done with Setup DebugSS PLL Clocking 1.9GHz

CS_DAP_DebugSS: GEL Output: < Done with Setup DebugSS ATB Clocking 380MHz

CS_DAP_DebugSS: GEL Output: < Done with Setup DebugSS Trace export clock (TPIU) to 97MHz

CS_DAP_DebugSS: GEL Output: --->>> TURNING ON L3_INSTR and L3_3 clocks required for debug instrumention <<<<<<----

CS_DAP_DebugSS: GEL Output: ---<<< L3 instrumentation clocks are enabled >>>> ---

CS_DAP_DebugSS: GEL Output: --->>> Mapping TIMER supsend sources to default cores <<<<<<----

CS_DAP_PC: GEL Output: Cortex-A15 1 is not in WIR mode so nothing to do.

CortexA15_0: GEL Output: --->>> AM572x GP EVM <<<---

CortexA15_0: GEL Output: --->>> AM572x Target Connect Sequence Begins ... <<<---

CortexA15_0: GEL Output: --->>> I2C Init <<<---

CortexA15_0: GEL Output: --->>> AM572x Begin MMC2 Pad Configuration <<<---

CortexA15_0: GEL Output: --->>> AM572x End MMC2 Pad Configuration <<<---

CortexA15_0: GEL Output: --->>> AM572x PG2.0 GP device <<<---

CortexA15_0: GEL Output: --->>> PRCM Clock Configuration for OPPNOM in progress... <<<---

CortexA15_0: GEL Output: 	Cortex A15 DPLL OPP 0 clock config is in progress...

CortexA15_0: GEL Output: 	Cortex A15 DPLL is already locked, now unlocking...

CortexA15_0: GEL Output: 	Cortex A15 DPLL OPP 0 is DONE!

CortexA15_0: GEL Output: 	IVA DPLL OPP 0 clock config is in progress...

CortexA15_0: GEL Output: 	IVA DPLL OPP 0 is DONE!

CortexA15_0: GEL Output: 	PER DPLL OPP 0 clock config in progress...

CortexA15_0: GEL Output: 	PER DPLL already locked, now unlocking

CortexA15_0: GEL Output: 	PER DPLL OPP 0 is DONE!

CortexA15_0: GEL Output: 	CORE DPLL OPP 0 clock config is in progress...

CortexA15_0: GEL Output: 	CORE DPLL OPP  already locked, now unlocking....

CortexA15_0: GEL Output: 	CORE DPLL OPP 0 is DONE!

CortexA15_0: GEL Output: 	ABE DPLL OPP 0 clock config in progress...

CortexA15_0: GEL Output: 	ABE DPLL OPP 0 is DONE!

CortexA15_0: GEL Output: 	GMAC DPLL OPP 0 clock config is in progress...

CortexA15_0: GEL Output: 	GMAC DPLL OPP 0 is DONE!

CortexA15_0: GEL Output: 	GPU DPLL OPP 0 clock config is in progress...

CortexA15_0: GEL Output: 	GPU DPLL OPP 0 is DONE!

CortexA15_0: GEL Output: 	DSP DPLL OPP 0 clock config is in progress...

CortexA15_0: GEL Output: 	DSP DPLL OPP 0 is DONE!

CortexA15_0: GEL Output: 	PCIE_REF DPLL OPP 0 clock config is in progress...

CortexA15_0: GEL Output: 	PCIE_REF DPLL OPP 0 is DONE!

CortexA15_0: GEL Output: --->>> PRCM Clock Configuration for OPP 0 is DONE! <<<---

CortexA15_0: GEL Output: --->>> PRCM Configuration for all modules in progress... <<<---

CortexA15_0: GEL Output: --->>> PRCM Configuration for all modules is DONE! <<<---

CortexA15_0: GEL Output: --->>> DDR3 Initialization is in progress ... <<<---

CortexA15_0: GEL Output: 	DDR DPLL clock config for 532MHz is in progress...

CortexA15_0: GEL Output: 	DDR DPLL clock config for 532MHz is in DONE!

CortexA15_0: GEL Output:        Launch full leveling

CortexA15_0: GEL Output:        Updating slave ratios in PHY_STATUSx registers

CortexA15_0: GEL Output:        as per HW leveling output

CortexA15_0: GEL Output:        HW leveling is now disabled. Using slave ratios from

CortexA15_0: GEL Output:        PHY_STATUSx registers

CortexA15_0: GEL Output:        Launch full leveling

CortexA15_0: GEL Output:        Updating slave ratios in PHY_STATUSx registers

CortexA15_0: GEL Output:        as per HW leveling output

CortexA15_0: GEL Output:        HW leveling is now disabled. Using slave ratios from

CortexA15_0: GEL Output:        PHY_STATUSx registers

CortexA15_0: GEL Output:        Two EMIFs in interleaved mode - (2GB total)

CortexA15_0: GEL Output: --->>> DDR3 Initialization is DONE! <<<---

CortexA15_0: GEL Output: --->>> Mapping TIMER suspend sources to default cores <<<<<<----

CortexA15_0: GEL Output: --->>> IPU1SS Initialization is in progress ... <<<---

CortexA15_0: GEL Output: --->>> IPU1SS Initialization is DONE! <<<---

CortexA15_0: GEL Output: --->>> IPU2SS Initialization is in progress ... <<<---

CortexA15_0: GEL Output: --->>> IPU2SS Initialization is DONE! <<<---

CortexA15_0: GEL Output: --->>> DSP1SS Initialization is in progress ... <<<---

CortexA15_0: GEL Output: DEBUG: Clock is active ...

CortexA15_0: GEL Output: DEBUG: Checking for data integrity in DSPSS L2RAM ...

CortexA15_0: GEL Output: DEBUG: Data integrity check in GEM L2RAM is sucessful!

CortexA15_0: GEL Output: --->>> DSP1SS Initialization is DONE! <<<---

CortexA15_0: GEL Output: --->>> DSP2SS Initialization is in progress ... <<<---

CortexA15_0: GEL Output: DEBUG: Clock is active ...

CortexA15_0: GEL Output: DEBUG: Checking for data integrity in DSPSS L2RAM ...

CortexA15_0: GEL Output: DEBUG: Data integrity check in GEM L2RAM is sucessful!

CortexA15_0: GEL Output: --->>> DSP2SS Initialization is DONE! <<<---

CortexA15_0: GEL Output: --->>> IVAHD Initialization is in progress ... <<<---

CortexA15_0: GEL Output: DEBUG: Clock is active ...

CortexA15_0: GEL Output: --->>> IVAHD Initialization is DONE! ... <<<---

CortexA15_0: GEL Output: --->>> PRUSS 1 and 2 Initialization is in progress ... <<<---

CortexA15_0: GEL Output: --->>> PRUSS 1 and 2 Initialization is in complete ... <<<---

CortexA15_0: GEL Output: --->>> AM572x Target Connect Sequence DONE !!!!!  <<<---

CortexA15_0: GEL Output: --->>> Mapping TIMER suspend sources to default cores <<<<<<----

CortexA15_0: GEL Output: --->>> IPU1SS Initialization is in progress ... <<<---

CortexA15_0: GEL Output: --->>> IPU1SS Initialization is DONE! <<<---

CortexA15_0: GEL Output: --->>> IPU2SS Initialization is in progress ... <<<---

CortexA15_0: GEL Output: --->>> IPU2SS Initialization is DONE! <<<---

CortexA15_0: GEL Output: --->>> DSP1SS Initialization is in progress ... <<<---

CortexA15_0: GEL Output: DEBUG: Clock is active ...

CortexA15_0: GEL Output: DEBUG: Checking for data integrity in DSPSS L2RAM ...

CortexA15_0: GEL Output: DEBUG: Data integrity check in GEM L2RAM is sucessful!

CortexA15_0: GEL Output: --->>> DSP1SS Initialization is DONE! <<<---

CortexA15_0: GEL Output: --->>> DSP2SS Initialization is in progress ... <<<---

CortexA15_0: GEL Output: DEBUG: Clock is active ...

CortexA15_0: GEL Output: DEBUG: Checking for data integrity in DSPSS L2RAM ...

CortexA15_0: GEL Output: DEBUG: Data integrity check in GEM L2RAM is sucessful!

CortexA15_0: GEL Output: --->>> DSP2SS Initialization is DONE! <<<---

CortexA15_0: GEL Output: --->>> IVAHD Initialization is in progress ... <<<---

CortexA15_0: GEL Output: DEBUG: Clock is active ...

CortexA15_0: GEL Output: --->>> IVAHD Initialization is DONE! ... <<<---

CortexA15_0: GEL Output: --->>> PRUSS 1 and 2 Initialization is in progress ... <<<---

CortexA15_0: GEL Output: --->>> PRUSS 1 and 2 Initialization is in complete ... <<<---

Step 3: Load vps_examples_displayDss_a15_0_debug/release.xa15fg and run:

CCS console:

[CortexA15_0] DISP_DSS_APP: Sample Application - STARTS !!!

*** VPDMA Firmware Loading... ***

VPDMA Firmware Address = 0xa0011fc0

VPDMA Load Address     = 0x4897d004

VPDMA Firmware Version = 0x4d0001b8

VPDMA List Busy Status = 0x00000000

*** VPDMA Firmware Load Success ***

*** VPDMA Firmware Loading... ***

VPDMA Firmware Address = 0xa0011fc0

VPDMA Load Address     = 0x4899d004

VPDMA Firmware Version = 0x4d0001b8

VPDMA List Busy Status = 0x00000000

*** VPDMA Firmware Load Success ***

*** VPDMA Firmware Loading... ***

VPDMA Firmware Address = 0xa0011fc0

VPDMA Load Address     = 0x489bd004

VPDMA Firmware Version = 0x4d0001b8

VPDMA List Busy Status = 0x00000000

*** VPDMA Firmware Load Success ***

*** VPDMA Firmware Loading... ***

VPDMA Firmware Address = 0xa0011fc0

VPDMA Load Address     = 0x489dd004

VPDMA Firmware Version = 0x4d0001b8

VPDMA List Busy Status = 0x00000000

*** VPDMA Firmware Load Success ***

Build time            : [17:02:58 May 25 2017]

FVID2 Version         : [FVID_02_01_00_01]

BSP Version           : [BSP_01_08_00_xx]

Platform              : [EVM]

SOC                   : [AM572X]

SOC Revision          : [ES2.0]

FT Revision           : [3]

Package               : [UNKNOWN]

Core                  : [A15]

Board Detected        : [AM572X GP EVM]

EEPROM Base Board Name: [AM572PM_]

Base Board Revision   : [REV A]

Daughter Card Revision: [REV A]

Load 4 YUV422I_YUYV video frames of size 1920x1080 to location: 0x0x81600000

Trying to Load buffer from SD card.

Couldnot Load buffer from SD card.

waiting for 1 min to load buffer from scripting consolePress any key (in UART console) after loading buffer

loadRaw(0x81600000, 0, "C:\\display_yuyv422_prog_packed_1920_1080.tigf", 32, false);

Similar printout from UART console:

Step 4: Use View----- Scripting Console to open the console:

Step 5: Use Alt+F8 to halt A15_0 core, it can’t load during run

Step 6: Load the first video till finish:

Step 7: Run A15_0 for UART to accept input

Step 8: Press “enter” in UART console, it asks you to enter the second video:

Step 9: Use Alt+F8 to halt A15_0 core again

Step 10: Load the second video till finish:

Step 11: Run A15_0 again for UART to accept input

Step 12: Press “enter” in UART console (same info printed in CCS console):

Step 13: select the test to run by enter the test number (E.g. 0) in UART:

Video shall be displayed on LCD (due to AM572x GP EVM uses 800x480 pixel LCD, the display is partial)

Step 14:

After the test done, statistics are printed and a new test can be selected: