/*==============================================================================

*   Copyright (c) Texas Instruments Inc 2002, 2003, 2004

*

*   Use of this software is controlled by the terms and conditions found in the

*   license agreement under which this software has been supplied.

*==============================================================================

*/

/**=============================================================================

*   @file  Emif_example.c

*

*

*   @desc  Example of EMIF module of CSL

*

*==============================================================================

*   @n <b> Test Description </b>

*   @n The example shows how to test the EMIF.

*      1. Initilizing the EMIF Hardware with required values for SDRAM

*      2. Write a particular pattern of data to SDRAM

*      3. Read verify the pattern is succefully written to SDRAM

*==============================================================================

*

*   <b> Test Procedure </b>

*   @verbatim

*      1. Configure the CCS setup to work with the emulator being used.

*         Refer CCS manual for setup configuration

*      2. Load required GEL file depending on the board used

*      3. Launch CCS window

*      4. Open project emif_example.pjt

*      5. Build the project and load the .out file of the project.

*      6. The test can be executed from the main().

*

*   @endverbatim

*

*

*==============================================================================

*/

/*==============================================================================

*  Revision History

*  ===============

*  12-may-2005 Ganesh K  File created.

*

*==============================================================================

*/

#include "Emif_example.h"

/* Variable to count the test failures */

Uint32                   emifTestFail = EMIF_TEST_PASSED;

/* Emif Hardware setup structure */

CSL_EmifHwSetup          hwSetup;

/* Handle for the module */

CSL_EmifHandle           hEmif = NULL;

/*

*==============================================================================

*  @func   main

*

*  @desc

*     This is the main routine for the file.

*

*  @arg

*      NONE

*

*  @return

*      NONE

*==============================================================================

*/

void

main (

void

)

{

/* Perform the test */

emif_csl_example_test ();

/* Check for errors in test */

if (emifTestFail == EMIF_TEST_PASSED) {

printf ("\nEmif CSL Example Test PASSED\n");

}

else {

printf ("\nEmif CSL Example Test FAILED\n");

}

}

/*

*==============================================================================

*  @func   emif_csl_example_test

*

*  @desc

*     This is the test routine which SDRAM write, read and verify data.

*

*  @arg

*      NONE

*

*  @return

*      NONE

*==============================================================================

*/

void

emif_csl_example_test (

void

)

{

CSL_EmifObj              emifObj;

CSL_Status               status;

Uint32                  *arg;

Uint8                    pattern;

Uint32                   i, j, k;

CSL_sysInit();

/* Initialize the emif CSL module */

status = CSL_emifInit (NULL);

if (status != CSL_SOK) {

printf ("EMIF: Initialization... Failed.\n");

printf ("\tReason: CSL_emifInit failed [Status: %d] \n ", status);

emifTestFail++;

return;

}

else {

printf ("EMIF: Initialization... Passed.\n");

//printf ("\tReason: CSL_emifInit failed [Status: %d] \n ", status);

}

/* Open the emif CSL module */

hEmif = CSL_emifOpen (&emifObj, CSL_EMIF, NULL, &status);

if ((hEmif == NULL) || (status != CSL_SOK)) {

printf ("EMIF: Open invalid instance... Failed\n");

printf ("\tReason: CSL_emifOpen failed [Status: %d] \n ", status);

emifTestFail++;

return;

}

else {

printf ("EMIF: Invalid instance... Passed.\n");

}

/* Do the required setup for SDRAM. Refer SDRAM data sheet for values */

/* Setting SDRAM timimng register SDTIMR */

hwSetup.emClkCycles.tras = EMIF_TEST_EM_CLK_CYCLES_3;

hwSetup.emClkCycles.trc = EMIF_TEST_EM_CLK_CYCLES_5;

hwSetup.emClkCycles.trcd = EMIF_TEST_EM_CLK_CYCLES_1;

hwSetup.emClkCycles.trfc = EMIF_TEST_EM_CLK_CYCLES_6;

hwSetup.emClkCycles.trp = EMIF_TEST_EM_CLK_CYCLES_1;

hwSetup.emClkCycles.trrd = EMIF_TEST_EM_CLK_CYCLES_1;

hwSetup.emClkCycles.twr = EMIF_TEST_EM_CLK_CYCLES_1;

/* Setting required refresh rate */

hwSetup.refreshRate = EMIF_TEST_SDRAM_REFRESH;

hwSetup.sdramPageSize = CSL_EMIF_512WORD_9COL_ADDR;

hwSetup.sdramIntBankSetup = CSL_EMIF_4_SDRAM_BANKS;

hwSetup.sdramCasLaten = CSL_EMIF_CAS_LATENCY_SET_TO_2;

hwSetup.narrowMode = CSL_EMIF_32BIT_USAGE;

hwSetup.sdramSelfRefreshEn = CSL_EMIF_SELF_REFRESH_DISABLE;

/* Setting values in SDRAM Self reresh exit timing register SDSRETR */

hwSetup.txs = EMIF_TEST_ECLKOUT_CYCLES_6;

/* Setup EMIF hardware parameters */

status = CSL_emifHwSetup (hEmif, &hwSetup);

if (status != CSL_SOK) {

printf ("EMIF: Hardware setup... Failed.\n");

emif_error_exit ();

emifTestFail++;

}

else {

printf ("EMIF: Hardware setup... Passed.\n");

}

/*

* Write a Pattern to SDRAM. The pattern written consist of clock of

* pattern written to SDRAM. The pattern is nth block starts with value

* 'n' and ends with value 'n-1'. for eg block0 start with 0x00 and ends

* with 0xFF. Block 0xF starts with 0xF ends with 0xE

*

*/

arg = (Uint32 *) EMIF_TEST_SDRAM_START_ADDR;

for (j = 0; j <= 15; j++) {

pattern = j;

for (i = 0; i <= 50; i++) {

*arg = pattern++;

arg++;

}

}

k = EMIF_TEST_PASSED;

/* Read and verify Pattern From SDRAM */

arg = (Uint32 *) EMIF_TEST_SDRAM_START_ADDR;

for (j = 0; j <= 15; j++) {

pattern = j;

for (i = 0; i <= 50; i++) {

if (*arg != pattern++) {

/* If pattern doesnt match set error */

k = 1;

break;

}

arg++;

}

/* On error exit */

if (k != EMIF_TEST_PASSED) {

break;

}

}

/* Print the test result */

if (k != EMIF_TEST_PASSED) {

emifTestFail++;

printf ("EMIF:SDRAM Read verify ......Failed\n");

}

else {

printf ("EMIF:SDRAM Read verify ......Passed\n");

}

/* Check closing the module */

status = CSL_emifClose (hEmif);

if (status != CSL_SOK) {

emifTestFail++;

printf ("EMIF: Close Emif instance... Failed\n");

}

else {

printf ("EMIF: Close Emif instance... Passed\n");

}

return;

}

/*

* =============================================================================

*  @func   emif_error_exit

*

*  @desc

*     This calls the module close function in case of error

*

*  @arg

*      NONE

*

*  @return

*      NONE

* =============================================================================

*/

void

emif_error_exit (

void

)

{

CSL_Status               status;

/* Check closing the module */

status = CSL_emifClose (hEmif);

if (status != CSL_SOK) {

emifTestFail++;

printf ("EMIF: Close Emif instance... Failed\n");

}

else {

printf ("EMIF: Close Emif instance... Passed\n");

}

}