There are some questions about KeyStone II 10 Gigabit Ethernet(3-port) itself QoS capabilities, list as blow:

- Packet strict priority scheduling

From "KeyStone II Architecture 10 Gigabit Ethernet Subsystem User Guide.pdf", there are description about packet priority handling for this 10G switch:

Basically, there are two level registers for priority mapping per each port:

- packet priority to header packet priority mapping register

- header priority to switch priority mapping register

But from register list, such two registers only available for P0(host port), for other two ethernet ports(P1&P2), there is only one register (TX_PRI_MAP):

Some questions are:

- Why missing another (RX_PRI_MAP) for P1&P2?

- Do these two Ethernet ports(P1&P2) support packet priority handling?

- What’s packet priority handling algorism? Does it mean strict priority scheduling？

- Queue size per priority

Based on above statement, my understanding this switch doesn’t support queue size per priority, but can change TX_BLK_CNT field to limit the maximum queue size for all eight priority in total.

From register dump:

Both two ports(1&2) value are 0x00000302, the strange thing is the bits field definition is very weird, RX_BLK_CNT has 6bits while TX_BLK_CNT has 5bits, my understanding is both should have 8bits so that can provide range 0-255 for count setting which should similar as port 0 definition:

But port0 still has inconsistent definition, check above tagged red color part.

Some questions are:

- Since TX_BLK_CNT bit fields are BIT9-BIT13, and from register memory dump, the value is 0x00000302, then it looks both P1&P2 TX_BLK_CNT are set to “1”:

  - What’s the memory block size?

  - The number of memory blocks is “1”, what’s the amount of memory represent for?

  - Are BIT9-BIT13 correct for TX_BLK_CNT bit fields