The following steps can be used in a command shell to generate the CCS projects for PDK low level drivers. Users are expected to complete component discovery in CCS before using these steps.

Setup environment from root directory:

<SDK_INSTALL_PATH>\processor_sdk_rtos_am65xx_5_01_00_11

setupenv.bat

Optional step: SDK contains prebuilt binaries for driver and CSL libraries so there is no need to rebuild the libraries but if you need to rebuild the libraries, you can do it using gmake utility  using the following options.

gmake <target>_clean

gmake <target>

<target>: Refer to makefile in the pdk_am65xx_x_x_x\packages

all csl board emac fvid2 gpio i2c mcasp mmcsd nimu pm profiling pruss sciclient spi uart udma usb sbl sa osal can be used as <target>

This can take a while to build. To limit the build to A53 core, use the additional arguments as

gmake <target> LIMIT_BOARDS=am65xx_evm LIMIT_CORES=mpu1_0 <ARGUMENTS>

| ARGUMENTS | Options | Description |
| --- | --- | --- |
| LIMIT_BOARDS | am65xx_evm, am65xx_idk | Supported evaluation platform |
| LIMIT_SOC | am65xx | Supported SOC options |
| LIMIT_CORES | mcu1_0, mpu1_0 | Support for different cores |
| DISABLE_RECURSE_DEPS | yes, no | Disable for building dependencies |
| BUILD_PROFILE | release, debug | Build profile |

To apply the options globally, set the environment variables:

set LIMIT_BOARDS=am65xx_evm

set LIMIT_SOC=am65xx

set DISABLE_RECURSE_DEPS=yes

set BUILD_PROFILE=release