GS80 OMAP5/J6 Eye improvement techniques

Version: 7

Dt 10/05/15

We are targeting the termination resistance tuning to improve the eye.

Fundamental idea would be to reduce termination resistance by a few counts. We would recommend tuning termination resistance first and then if that doesn‘t work it is recommended to optimize the channel USB cable.

Address of registers

| Register | Address in Vayu/OMAP5 |
| --- | --- |
| TERMINATION_CONTROL | 0x4a084000 (J6 USB2PHY2 - 0x4a085000) |
| USB2PHYCM_TRIM | 0x4a084034 (J6 USB2PHY2 – 0x4a085034) |

The register descriptions are below this text

Adjusting termination resistance

- Write 0 to TERMINATION_CONTROL.MEM_FS_CODE_SEL bits 27:24 in this register. This offset may be otherwise internally applied when the test pkt is being emitted. Make sure to do this step first because the value read in 5:0 will change once 0 is written to 27:24.

- Read TERMINATION_CONTROL.RTERM_RMX (offset 0x00) bit 5:0.

- Increase this value by 8-9 codes from the read value. 10 codes will provide ~2ohm decrease in output impedance. If untrimmed, you can use 21 as default. Depending on the exact system, you may have to go higher.

- Set TERMINATION_CONTROL.MEM_USE_RTERM_RMX_REG bit 21 as 1 and set the value from step 2 to MEM_RTERM_RMX (bit 20:15).

Table 40. USB2PHYCM_TRIM

| Address Offset |  |  |  |  |  | 0x34 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  | contains trim bit overrides for the USB2PHYCM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | 30 | 29 | 28 | 27 | 26 |  | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 |  | 14 | 13 | 12 | 11 | 10 |  | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| mem_USEBGTRIM | mem_BGTRIM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reserved |  |  | mem_USECURTRIM | mem_CURTRIM |  |  |  |  |  | Reserved |  |  |  |  | Reserved |  |  |

| Bits | Field Name | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| 31 | MEM_USEBGTRIM | When set to ‘1’ bits 30 to 16 are used as the trim value for the bandgap | RW | 0 |
| 30:16 | MEM_BGTRIM | Override value for the mem_BGTRIM value | RW | 0x0000 |
| 15:14 | Reserved |  | RO Rreturns0s | 0x0 |
| 13 | MEM_USECURTRIM | Override enable for Cuurent trim. | RW | 0 |
| 12:8 | MEM_CURTRIM | Current trim code. This is used if mem_use_CURTRIM is set. | RW | 0x00 |
| 7:3 | Reserved |  | RO Rreturns0s | 0x00 |
| 2:0 | Reserved |  | RW | 0x0 |

Table 27. TERMINATION_CONTROL

| Address Offset |  |  |  |  |  | 0x00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  | contains bits related to control of terminations in USB2PHY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  | RW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | 30 | 29 | 28 | 27 | 26 |  | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 |  | 14 | 13 | 12 | 11 | 10 |  | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Reserved |  | mem_ALWAYS_UPDATE | Reserved | mem_FS_CODE_SEL |  |  |  |  | Reserved |  | mem_USE_RTERM_RMX_REG | mem_RTERM_RMX |  |  |  |  |  |  | Reserved | mem_HS_CODE_SEL |  |  | Reserved |  | Reserved | mem_DISABLE_TEMP_TRACK | Reserved |  | RTERM_RMX |  |  |  |  |  |

| Bits | Field Name | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| 31:30 | Reserved |  | RW | 0x0 |
| 29 | MEM_ALWAYS_UPDATE | When set to ‘1’, the calibration code is updated immediately after a code computation without waiting for idle periods. | RW | 0 |
| 28 | Reserved |  | RO Rreturns0s | 0 |
| 27:24 | MEM_FS_CODE_SEL | FS Code selection control | RW | 0x7 |
| 23:22 | Reserved |  | RW | 0x0 |
| 21 | MEM_USE_RTERM_RMX_REG | Override termination resistor trim code with mem_RTERM_RMX from this register | RW | 0 |
| 20:15 | MEM_RTERM_RMX | The value written to this field is used as Termination resistor trim code if bit 21 is set to ‘1’. Bits 5:2 of this are also used as rpu trim code if bit 21 is set. | RW | 0x00 |
| 14 | Reserved |  | RO Rreturns0s | 0 |
| 13:11 | MEM_HS_CODE_SEL | HS Code selection control | RW | 0x0 |
| 10 | Reserved |  | RO Rreturns0s | 0 |
| 9 | Reserved |  | RO Rreturns0s | 0 |
| 8 | MEM_DISABLE_TEMP_TRACK | Disables the temperature tracking function of the termination calibration | RW | 0 |
| 7:6 | Reserved |  | RO Rreturns0s | 0x0 |
| 5:0 | RTERM_RMX | Returns the current value of RTERM_RMX | RO Rreturns0s | 0x00 |