;******************************************************************************

;* BOOT  v4.6.4                                                               *

;* Copyright (c) 1996-2010 Texas Instruments Incorporated                     *

;******************************************************************************

;****************************************************************************

;* BOOT.ASM

;*

;* THIS IS THE INITAL BOOT ROUTINE FOR TMS470 C++ PROGRAMS.

;* IT MUST BE LINKED AND LOADED WITH ALL C++ PROGRAMS.

;*

;* THIS MODULE PERFORMS THE FOLLOWING ACTIONS:

;*   1) ALLOCATES THE STACK AND INITIALIZES THE STACK POINTER

;*   2) CALLS AUTO-INITIALIZATION ROUTINE

;*   3) CALLS THE FUNCTION MAIN TO START THE C++ PROGRAM

;*   4) CALLS THE STANDARD EXIT ROUTINE

;*

;* THIS MODULE DEFINES THE FOLLOWING GLOBAL SYMBOLS:

;*   1) __stack     STACK MEMORY AREA

;*   2) _c_int00    BOOT ROUTINE

;*

;****************************************************************************

.if  __TI_TMS470_V7M3__

.thumbfunc _c_int00

.else

.armfunc _c_int00

.endif

;****************************************************************************

; Accomodate different lowerd names in different ABIs

;****************************************************************************

.if   __TI_EABI_ASSEMBLER

.asg	_args_main,   ARGS_MAIN_RTN

.asg	exit,         EXIT_RTN

.asg    main_func_sp, MAIN_FUNC_SP

.elseif __TI_ARM9ABI_ASSEMBLER | .TMS470_32BIS

.asg	__args_main,   ARGS_MAIN_RTN

.asg	_exit,         EXIT_RTN

.asg    _main_func_sp, MAIN_FUNC_SP

.else

.asg	$_args_main,   ARGS_MAIN_RTN

.asg	$exit,         EXIT_RTN

.asg    _main_func_sp, MAIN_FUNC_SP

.endif

.if .TMS470_16BIS

;****************************************************************************

;*  16 BIT STATE BOOT ROUTINE                                               *

;****************************************************************************

.if __TI_TMS470_V7M3__

.state16

.else

.state32

.endif

.global	__stack

;***************************************************************

;* DEFINE THE USER MODE STACK (DEFAULT SIZE IS 512)

;***************************************************************

__stack:.usect	".stack", 0, 4

.global	_c_int00

;***************************************************************

;* FUNCTION DEF: _c_int00

;***************************************************************

_c_int00: .asmfunc

.if !__TI_TMS470_V7M3__

.if __TI_NEON_SUPPORT__ | __TI_VFP_SUPPORT__

;*------------------------------------------------------

;* SETUP PRIVILEGED AND USER MODE ACCESS TO COPROCESSORS

;* 10 AND 11, REQUIRED TO ENABLE NEON/VFP

;* COPROCESSOR ACCESS CONTROL REG

;* BITS [23:22] - CP11, [21:20] - CP10

;* SET TO 0b11 TO ENABLE USER AND PRIV MODE ACCESS

;*------------------------------------------------------

MRC      p15,#0x0,r0,c1,c0,#2

MOV      r3,#0xf00000

ORR      r0,r0,r3

MCR      p15,#0x0,r0,c1,c0,#2

;*------------------------------------------------------

; SET THE EN BIT, FPEXC[30] TO ENABLE NEON AND VFP

;*------------------------------------------------------

MOV      r0,#0x40000000

FMXR     FPEXC,r0

.endif

;------------------------------------------------------

;* SET TO USER MODE

;*------------------------------------------------------

MRS     r0, cpsr

BIC     r0, r0, #0x1F  ; CLEAR MODES

ORR     r0, r0, #0x10  ; SET USER MODE

MSR     cpsr_cf, r0

;*------------------------------------------------------

;* CHANGE TO 16 BIT STATE

;*------------------------------------------------------

ADD	r0, pc, #1

BX	r0

.state16

.endif

;*------------------------------------------------------

;* INITIALIZE THE USER MODE STACK

;*------------------------------------------------------

LDR     r0, c_stack

MOV	sp, r0

LDR     r0, c_STACK_SIZE

ADD	sp, r0

;*-----------------------------------------------------

;* ALIGN THE STACK TO 64-BITS IF EABI.

;*-----------------------------------------------------

.if __TI_EABI_ASSEMBLER

MOV	r7, sp

MOV	r0, #0x07

BIC     r7, r0         ; Clear upper 3 bits for 64-bit alignment.

MOV	sp, r7

.endif

;*-----------------------------------------------------

;* SAVE CURRENT STACK POINTER FOR SDP ANALYSIS

;*-----------------------------------------------------

LDR	r0, c_mf_sp

MOV	r7, sp

STR	r7, [r0]

;*------------------------------------------------------

;* Perform all the required initilizations:

;*   - Process BINIT Table

;*   - Perform C auto initialization

;*   - Call global constructors

;*------------------------------------------------------

BL      __TI_auto_init

;*------------------------------------------------------

;* CALL APPLICATION

;*------------------------------------------------------

BL      ARGS_MAIN_RTN

;*------------------------------------------------------

;* IF APPLICATION DIDN'T CALL EXIT, CALL EXIT(1)

;*------------------------------------------------------

MOV     r0, #1

BL      EXIT_RTN

;*------------------------------------------------------

;* DONE, LOOP FOREVER

;*------------------------------------------------------

L1:     B	L1

.endasmfunc

.else           ; !.TMS470_16BIS

;****************************************************************************

;*  32 BIT STATE BOOT ROUTINE                                               *

;****************************************************************************

.global	__stack

;***************************************************************

;* DEFINE THE USER MODE STACK (DEFAULT SIZE IS 512)

;***************************************************************

__stack:.usect	".stack", 0, 4

.global	_c_int00

;***************************************************************

;* FUNCTION DEF: _c_int00

;***************************************************************

_c_int00: .asmfunc

.if __TI_NEON_SUPPORT__ | __TI_VFP_SUPPORT__

;*------------------------------------------------------

;* SETUP PRIVILEGED AND USER MODE ACCESS TO COPROCESSORS

;* 10 AND 11, REQUIRED TO ENABLE NEON/VFP

;* COPROCESSOR ACCESS CONTROL REG

;* BITS [23:22] - CP11, [21:20] - CP10

;* SET TO 0b11 TO ENABLE USER AND PRIV MODE ACCESS

;*------------------------------------------------------

MRC      p15,#0x0,r0,c1,c0,#2

MOV      r3,#0xf00000

ORR      r0,r0,r3

MCR      p15,#0x0,r0,c1,c0,#2

;*------------------------------------------------------

; SET THE EN BIT, FPEXC[30] TO ENABLE NEON AND VFP

;*------------------------------------------------------

MOV      r0,#0x40000000

FMXR     FPEXC,r0

.endif

;*------------------------------------------------------

;* SET TO USER MODE

;*------------------------------------------------------

MRS     r0, cpsr

BIC     r0, r0, #0x1F  ; CLEAR MODES

ORR     r0, r0, #0x13  ; SET USER MODE

MSR     cpsr_cf, r0

;*------------------------------------------------------

;* INITIALIZE THE USER MODE STACK

;*------------------------------------------------------

LDR     sp, c_stack

LDR     r0, c_STACK_SIZE

ADD	sp, sp, r0

;*-----------------------------------------------------

;* ALIGN THE STACK TO 64-BITS IF EABI.

;*-----------------------------------------------------

.if __TI_EABI_ASSEMBLER

BIC     sp, sp, #0x07  ; Clear upper 3 bits for 64-bit alignment.

.endif

;*-----------------------------------------------------

;* SAVE CURRENT STACK POINTER FOR SDP ANALYSIS

;*-----------------------------------------------------

LDR	r0, c_mf_sp

STR	sp, [r0]

;*------------------------------------------------------

;* Perform all the required initilizations:

;*   - Process BINIT Table

;*   - Perform C auto initialization

;*   - Call global constructors

;*------------------------------------------------------

BL      __TI_auto_init

;*------------------------------------------------------

;* CALL APPLICATION

;*------------------------------------------------------

BL      ARGS_MAIN_RTN

;*------------------------------------------------------

;* IF APPLICATION DIDN'T CALL EXIT, CALL EXIT(1)

;*------------------------------------------------------

MOV     R0, #1

BL      EXIT_RTN

;*------------------------------------------------------

;* DONE, LOOP FOREVER

;*------------------------------------------------------

L1:     B	L1

.endasmfunc

.endif    ; !.TMS470_16BIS

;***************************************************************

;* CONSTANTS USED BY THIS MODULE

;***************************************************************

c_stack		.long    __stack

c_STACK_SIZE  	.long    __STACK_SIZE

c_mf_sp	        .long    MAIN_FUNC_SP

.if __TI_EABI_ASSEMBLER

.data

.align 4

_stkchk_called:

.field          0,32

.else

.sect   ".cinit"

.align  4

.field          4,32

.field          _stkchk_called+0,32

.field          0,32

.bss    _stkchk_called,4,4

.symdepend ".cinit", ".bss"

.symdepend ".cinit", ".text"

.symdepend ".bss", ".text"

.endif

;******************************************************

;* UNDEFINED REFERENCES                               *

;******************************************************

.global _stkchk_called

.global	__STACK_SIZE

.global ARGS_MAIN_RTN

.global MAIN_FUNC_SP

.global	EXIT_RTN

.global __TI_auto_init

.end