int FLASH_WRITE()

{

spiDAT1_t dataconfig1_t;

uint16 i;

dataconfig1_t.CS_HOLD = FALSE;

dataconfig1_t.WDEL = TRUE;

dataconfig1_t.DFSEL = SPI_FMT_0;

dataconfig1_t.CSNR = 0xfe;

gioSetBit(hetPORT1, 8, 0);

spiTransmitData( spiREG4, &dataconfig1_t, 1, &PAGE_PROG[0]);                                 // send the page programing command

spiTransmitData( spiREG4, &dataconfig1_t, 1, &memory_address_RW[2]);                         // send the 24 - 16 bit of the address

spiTransmitData( spiREG4, &dataconfig1_t, 1, &memory_address_RW[1]);                         // send the 15 - 8 bit of the address

spiTransmitData( spiREG4, &dataconfig1_t, 1, &memory_address_RW[0]);                         // send the 7 - 0 bit of the address

for (i = 0; i < 20; i++) {

spiTransmitData( spiREG4, &dataconfig1_t, 1, &val);                                      // send the write value to the flash

}

gioSetBit(hetPORT1, 8, 1);

val++;

if (val > 150)

val = 100;

return i ;

}

-------------------------------------------------------------------

int FLASH_READ()

{

spiDAT1_t dataconfig1_t;

uint16 i;

dataconfig1_t.CS_HOLD = FALSE;

dataconfig1_t.WDEL = TRUE;

dataconfig1_t.DFSEL = SPI_FMT_0;

dataconfig1_t.CSNR = 0xfe;

gioSetBit(hetPORT1, 8, 0);

spiTransmitData( spiREG4, &dataconfig1_t, 1, &READ[0]);                                     // send the read command

spiTransmitData( spiREG4, &dataconfig1_t, 1, &memory_address_RW[2]);                        // send 24 - 16 bit of the address

spiTransmitData( spiREG4, &dataconfig1_t, 1, &memory_address_RW[1]);                        // send 15 - 8 bit of the address

spiTransmitData( spiREG4, &dataconfig1_t, 1, &memory_address_RW[0]);                        // send 7 - 0 bit of the address

for (i = 0; i < 20; i++) {

spiReceiveData( spiREG4, &dataconfig1_t, 1, &read_data[1]);                             // read the data from send address

sciSendByte(sciREG, 'A');

Debug_Display_Val(read_data[1]);                                                        // display the read value from ADC

sciSendByte(sciREG, ' ');

}

gioSetBit(hetPORT1, 8, 1);

sciSendByte(sciREG, '\r');

sciSendByte(sciREG, '\n');

return i ;

}

---------------------------------------------------------------------------------

void ERASE_OPERATION()

{

spiDAT1_t dataconfig1_t;

uint16 SECTOR_ER = 0x20  ;

dataconfig1_t.CS_HOLD = FALSE;

dataconfig1_t.WDEL    = TRUE;

dataconfig1_t.DFSEL   = SPI_FMT_0;

dataconfig1_t.CSNR    = 0xfe;

gioSetBit(hetPORT1 ,   8 , 0 );

spiTransmitData  ( spiREG4, &dataconfig1_t , 1, &SECTOR_ER   ) ;                                        //    send the sector errase command to the device

spiTransmitData  ( spiREG4, &dataconfig1_t , 1, &memory_address_RW[2]   ) ;                             //   send the 24 - 16 bit of the address

spiTransmitData  ( spiREG4, &dataconfig1_t , 1, &memory_address_RW[1]   ) ;                             //  send the 15 - 8 bit of the  address

spiTransmitData  ( spiREG4, &dataconfig1_t , 1, &memory_address_RW[0]   ) ;                             //  send the 7 - 0 bit of the address

gioSetBit(hetPORT1 ,   8 , 1 );

while( (READ_STATUS_REGISTER() & 0x02) != 0 );                                                           //  wait until the WEL bit reset in the status register

}