/*

* ads7142registermap.h

*

*  Created on: Mar 23, 2018

*      Author: a0232450

*/

#define ADS7142_I2C_ADDRESS                                         0x1C

//***************************************************************************************************************************************************************************************************************************************

//

//  Command Opcodes

//

//***************************************************************************************************************************************************************************************************************************************

#define SINGLE_READ                                                 0x10        //Use for Single Register Read

#define SINGLE_WRITE                                                0x08        //Use for Single Register Write

#define SET_BIT                                                     0x18

#define CLEAR_BIT                                                   0x20

#define BLOCK_READ                                                  0x30        //Use for Block Register Read

#define BLOCK_WRITE                                                 0x28        //Use for Block Register Write

//***************************************************************************************************************************************************************************************************************************************

//

//  Register Map

//

//***************************************************************************************************************************************************************************************************************************************

/* Reset Registers */

#define ADS7142_REG_WKEY                                            0x17        //Write Key for writing into DEVICE_RESET Register

#define ADS7142_VAL_WKEY_RESET                                      0x00        //Write 00h to WKEY after device reset to prevent erroneous reset

#define ADS7142_VAL_WKEY_DEVICE_RESET                               0x0A        //Write 0Ah for write access to DEVICE_RESET

#define ADS7142_REG_DEVICE_RESET                                    0x14        //DEVICE_RESET Register

#define ADS7142_VAL_DEVICE_RESET_RESET                              0x01        //Write 01h to reset device

//***************************************************************************************************************************************************************************************************************************************

/* Functional Mode Select Registers */

#define ADS7142_REG_OFFSET_CAL                                      0x15        //Write to this register initiates internal offset calibration cycle, perform offset calibration periodically to nullify offset error

//Write 01h to OFFSET_CAL to begin offset calibration

#define ADS7142_REG_OPMODE_SEL                                      0x1C        //Write to OPMODE_SEL to set the functional mode of the device

#define ADS7142_VAL_OPMODE_SEL_I2C_CMD_MODE_W_CHANNEL_0             0x00        //00h - Manual Mode with Channel 0 only

#define ADS7142_VAL_OPMODE_SEL_I2C_CMD_MODE_W_AUTO_SEQ_EN           0x04        //04h - Manual Mode with AUTO Sequencing enabled

#define ADS7142_VAL_OPMODE_SEL_AUTONOMOUS_MONITORING_MODE           0x06        //06h - Autonomous Monitoring Mode with AUTO Sequencing enabled

#define ADS7142_VAL_OPMODE_SEL_HIGH_PERCISION_MODE                  0x07        //07h - High Precision Mode with AUTO Sequencing enabled

#define ADS7142_REG_OPMODE_I2CMODE_STATUS                           0x00        //Read only register that provides the present operation mode and I2C mode configuration

//00h - Device is not in High Speed I2C mode and is in Manual Mode

//01h - Device is not in High Speed I2C mode and is in Autonomous Monitoring Mode

//03h - Device is not in High Speed I2C mode and is in High Precision Mode (16-bit Resolution)

//04h - Device is in High Speed I2C mode & Manual Mode

//06h - Device is in High Speed I2C mode & Autonomous Monitoring Mode

//07h - Device is in High Speed I2C mode & High Precision Mode (16-bit Resolution)

//***************************************************************************************************************************************************************************************************************************************

/* Input Configuration Registers */

#define ADS7142_REG_CHANNEL_INPUT_CFG                               0x24        //Configures the analog input pins of the device

#define ADS7142_VAL_CHANNEL_INPUT_CFG_2_CHANNEL_SINGLE_ENDED        0x03        //Two-Channel, Single-Ended Configuration

#define ADS7142_VAL_CHANNEL_INPUT_CFG_1_CHANNEL_SINGLE_ENDED        0x01        //Single-Channel, Single-Ended Configuration

#define ADS7142_VAL_CHANNEL_INPUT_CFG_1_CHANNEL_PSEUDO_DIFF         0x02        //Single-Channel, Pseudo-Differential Configuration

//***************************************************************************************************************************************************************************************************************************************

/* Analog MUX and Sequencer Registers */

#define ADS7142_REG_AUTO_SEQ_CHEN                                   0x20        //This register selects the channels that are scanned when auto-sequencing is enabled

//By default, both channels are selected at power up

#define ADS7142_VAL_AUTO_SEQ_CHENAUTO_SEQ_CH0                       0x01        //Scan Ch0 only when auto-sequencing is enabled

#define ADS7142_VAL_AUTO_SEQ_CHENAUTO_SEQ_CH1                       0x02        //Scan Ch1 only when auto-sequencing is enabled

#define ADS7142_REG_START_SEQUENCE                                  0x1E        //A write to this register starts the channel scanning sequence

#define ADS7142_VAL_START_SEQUENCE_SEQ_START                        0x01        //Sets the BUSY/~RDY pin high and starts the first conversion in the sequence

#define ADS7142_REG_ABORT_SEQUENCE                                  0x1F        //A write to this register aborts the channel scanning sequence

/* Once sequence is aborted using this register, it is recommended to read the DATA_BUFFER_STATUS register to know the number of entries filled in the data buffer

or ACCUMULATOR_STATUS register to know the number of accumulations finished before the abort */

#define ADS7142_REG_SEQUENCE_STATUS                                 0x04        //Read-only register that provides the status of sequence in the device

//00h - Auto Sequencing disabled, no error

//02h - Auto Sequencing enabled, no error

//04h - Not Used

//06h - Auto Sequencing enabled, device in error

//***************************************************************************************************************************************************************************************************************************************

/* Oscillator and Timing control Registers */

#define ADS7142_REG_OSC_SEL                                         0x18        //A write to this register selects the oscillator used for the conversion process

#define ADS7142_OSC_SEL_HSZ_LP                                      0x01        //A read or write of this value indicates use or enable of the Low Power Oscillator

#define ADS7142_OSC_SEL_HSZ_HSO                                     0x00        //A read or write of this value indicates use or enable of the High Speed Oscillator

#define ADS7142_REG_nCLK_SEL                                        0x19        //This register controls the cycle time for a single conversion by setting the nCLK parameter

//nCLK is the number of clocks of the selected oscillator that the device uses for one conversion cycle

/* When using the High Speed Oscillator: for a value x written into the nCLK register

* if x <= 21, nCLK is set to 21 (15h or 00010101b)

* if x > 21, nCLK is set to x

* When using the Low Power Oscillator: for a value x written into the nCLK register:

* if x <= 18, nCLK is set to 18 (12h or 00010010b)

* if x > 18, nCLK is set to x

*/

//***************************************************************************************************************************************************************************************************************************************

/* Data Buffer Control Register */

#define ADS7142_REG_DATA_BUFFER_OPMODE                              0x2C         //Selects Data Buffer Operation Mode

#define ADS7142_DATA_BUFFER_STARTSTOP_CNTRL_NBURST                  0x00         //Stop Burst Mode

#define ADS7142_DATA_BUFFER_STARTSTOP_CNTRL_BURST                   0x01         //Start Burst Mode, default

#define ADS7142_DATA_BUFFER_STARTSTOP_CNTRL_PREALERT                0x04         //Pre Alert Data Mode

#define ADS7142_DATA_BUFFER_STARTSTOP_CNTRL_POSTALERT               0x06         //Post Alert Data Mode

/*Output Data Format Configuration*/

#define ADS7142_REG_DOUT_FORMAT_CFG                                 0x28         //This read/write register configures the data output format for the data buffer

#define ADS7142_REG_DOUT_FORMAT_CFG_DOUT_FORMAT0                    0x00         //12-bit conversion result followed by 0000b or 0h

#define ADS7142_REG_DOUT_FORMAT_CFG_DOUT_FORMAT1                    0x01         //12-bit conversion result followed by 3-bit Channel ID (000b for CH0, 001b for CH1)

#define ADS7142_REG_DOUT_FORMAT_CFG_DOUT_FORMAT2                    0x02         //12-bit conversion result followed by 3-bit Channel ID(000b for CH0, 001b for CH1) followed by DATA_VALID bit

#define ADS7142_REG_DOUT_FORMAT_CFG_DOUT_FORMAT3                    0x03         //12-bit conversion result followed by 0000b or 0h

#define ADS7142_REG_DATA_BUFFER_STATUS                              0x01         //Read-Only register that provides the number of entries filled in the data buffer until the last conversion

//Read will vary from 00h to 10h

//Bits 4-0 of this 8-bit register represent DATA_WORDCOUNT [00000] to [10000] = Number of entries filled in data buffer (0 to 16)

//***************************************************************************************************************************************************************************************************************************************

/*Accumulator Control Registers */

#define ADS7142_REG_ACC_EN                                          0x30         //Write to this register to enable the Accumulator

#define ADS7142_VAL_ACC_EN_ON                                       0x0F         //Turns on the accumulator

#define ADS7142_VAL_ACC_EN_OFF                                      0x00         //Turns off the accumulator

#define ADS7142_REG_ACC_CH0_LSB                                     0x08         //Provides the LSB of accumulated data for CH0

#define ADS7142_REG_ACC_CH0_MSB                                     0x09         //Provides the MSB of accumulated data for CH0

#define ADS7142_REG_ACC_CH1_LSB                                     0x0A         //Provides the LSB of accumulated data for CH1

#define ADS7142_REG_ACC_CH1_MSB                                     0x0B         //Provides the MSB of accumulated data for CH1

#define ADS7142_REG_ACCUMULATOR_STATUS                              0x02         /*Provides the present status of Accumulator (Read only)

Bits 3-0 of this register yield the number of accumulations completed until the last finished conversion*/

//***************************************************************************************************************************************************************************************************************************************

/*Digital Window Comparator Registers */

#define ADS7142_REG_ALERT_DWC_EN                                    0x37         //Write to this register enables the Alert and Digital Window Comparator block

#define ADS7142_REG_ALERT_DWC_BLOCK_ENABLE                          0x01         //Enable the Digital Window Comparator

#define ADS7142_REG_ALERT_DWC_BLOCK_DISABLE                         0x00         //Disable the Digital Window Comparator

#define ADS7142_REG_ALERT_CHEN                                      0x34         //Enables the alert functionality for individual channels

#define ADS7142_REG_DWC_HTH_CH0_MSB                                 0x39         //Sets the four most significant bits of high threshold for CH0

#define ADS7142_REG_DWC_HTH_CH0_LSB                                 0x38         //Sets the eight least significant bits of high threshold for CH0

#define ADS7142_REG_DWC_LTH_CH0_MSB                                 0x3B         //Sets the four most significant bits of low threshold for CH0

#define ADS7142_REG_DWC_LTH_CH0_LSB                                 0x3A         //Sets the eight least significant bits of high threshold for CH0

#define ADS7142_REG_DWC_HYS_CH0                                     0x40         //Sets the hysteresis for both comparators on CH0

#define ADS7142_REG_DWC_HTH_CH1_MSB                                 0x3D         //Sets the four most significant bits of high threshold for CH1

#define ADS7142_REG_DWC_HTH_CH1_LSB                                 0x3C         //Sets the eight least significant bits of high threshold for CH1

#define ADS7142_REG_DWC_LTH_CH1_MSB                                 0x3F         //Sets the four most significant bits of low threshold for CH1

#define ADS7142_REG_DWC_LTH_CH1_LSB                                 0x3E         //Sets the eight least significant bits of low threshold for CH1

#define ADS7142_REG_DWC_HYS_CH1                                     0x41         //Sets the hysteresis for both comparators for CH1

#define ADS7142_REG_PRE_ALERT_EVENT_COUNT                           0x36         //Sets the Pre-Alert Event Count for both high and low comparators, for both channels

#define ADS7142_REG_ALERT_TRIG_CHID                                 0x03         //Provides the channel ID of the channel which was first to set the alert output (Read only)

//00h ---> Channel 0

//10h ---> Channel 1

#define ADS7142_REG_ALERT_LOW_FLAGS                                 0x0C         //Provides the read/write status of latched flags for low alert on both channels

#define ADS7142_REG_ALERT_HIGH_FLAGS                                0x0E         //Provides the read/write status of latched flags for high alert on both channels