I’ve been having an issue with GPTM Timer 0 and I’m not sure what I’m doing wrong.

I have a 2 board system.  Board #1 is producing a Synch pulse and Board #2 board is accepting the Synch pulse.  This Synch pulse will produce an edge ~ every 4-6ms.  When the edge occurs, a CAN message is being sent by Board #2.  In the middle of the Synch pulse, another operation needs to occur on Board #2 so Timer0 is being setup as an Up Counter that will interrupt in the middle of the Synch Pulse.  (~ every 2-3ms)

The problem I’m seeing is the process will operate from 10 – 20 seeconds and then go off in the weeds.  When I am emulating on Board #2, I can’t get the emulator to give me any information.  After I see the processor is not working correctly and I try to Halt the emulation.  I receive an Error -2062 and I’m not able to Halt the process.

I’ve done some debugging and I believe the issue has to do with Timer 0.

Here are the two routines that I think are causing the problem:

//---------- ENABLE MEASUREMENT TIMER ------------

void Enable_MeasTimer(uint32_t dwTimerIntStatus)

{

volatile uint32_t i;

//Ensure peripheral is enabled

if(ROM_SysCtlPeripheralReady(SYSCTL_PERIPH_TIMER0))

{

// Reset the Timer Value

while(HWREG(MEASUREMENT_TIMER + TIMER_O_TAV) != 0x00000001)

{

Main_delay(12);

HWREG(MEASUREMENT_TIMER + TIMER_O_TAV) = 0x00000001;

}

// Determine if Interrupt needs to be enabled.

if(dwTimerIntStatus == MEASTIMER_INT_ON)

{

//Enable Timer0A interrupt

IntEnable(INT_TIMER0A);

}

//Enables Timer0A

TimerEnable(MEASUREMENT_TIMER,TIMER_A);

#if DIRECTIVE_DELTA_MEAS_TEST

GPIOPinWrite_Output(GPIO_PORTM_BASE,ISO_SPARE2,0x00000000);

#endif

//---------- DISABLE MEASUREMENT TIMER ------------

void Disable_MeasTimer(void)

{

//Ensure peripheral is enabled

if(ROM_SysCtlPeripheralReady(SYSCTL_PERIPH_TIMER0))

{

//Disable Timer0A interrupt

IntDisable(INT_TIMER0A);

//Disable Timer0A

TimerDisable(MEASUREMENT_TIMER,TIMER_A);

#if DIRECTIVE_DELTA_MEAS_TEST

GPIOPinWrite_Output(GPIO_PORTM_BASE,ISO_SPARE2,0x00000002);

#endif

//Ensure the Timer Interrupt Flag is cleared.

TimerIntClear(MEASUREMENT_TIMER, TIMER_TIMA_TIMEOUT);

}

}

Inside the routines, you will see a Directive that is used to toggle an I/O line.  As I was troubleshooting the problem, I used these directives to see where the code would go off in the weeds.  Based on my trouble shooting, it looks like the problem occurs in the Enable_MeasTimer() routine.  When the Directive is moved to the first line of the routine, I can see the line toggle correctly before the code hangs up.  When I move the Directive to the last line of code, I can see the processor never hits that line.

I also noticed that when I added the Main_delay() before I tried to reset the TAV value, the code would operate much longer.  If I remove the delay it goes off in the weeds much faster.

The only interrupts running during this operation are the Synch pulse coming in on one of the I/O and Timer3 which run continuously.  The interrupt routine for both of these sources simply clears the interrupt and sets a flag.