//*****************************************************************************

// Copyright (C) 2014 Texas Instruments Incorporated

//

// All rights reserved. Property of Texas Instruments Incorporated.

// Restricted rights to use, duplicate or disclose this code are

// granted through contract.

// The program may not be used without the written permission of

// Texas Instruments Incorporated or against the terms and conditions

// stipulated in the agreement under which this program has been supplied,

// and under no circumstances can it be used with non-TI connectivity device.

//

//*****************************************************************************

//*****************************************************************************

//

// Application Name     -   MQTT Client

// Application Overview -   This application acts as a MQTT client and connects

//                          to the IBM MQTT broker, simultaneously we can

//                          connect a web client from a web browser. Both

//                          clients can inter-communicate using appropriate

//                          topic names.

//

// Application Details  -

// http://processors.wiki.ti.com/index.php/CC32xx_MQTT_Client

// or

// docs\examples\CC32xx_MQTT_Client.pdf

//

//*****************************************************************************

//*****************************************************************************

//

//! \addtogroup mqtt_client

//! @{

//

//*****************************************************************************

// Standard includes

#include <stdlib.h>

// simplelink includes

#include "simplelink.h"

// driverlib includes

#include "hw_types.h"

#include "hw_ints.h"

#include "hw_memmap.h"

#include "interrupt.h"

#include "rom_map.h"

#include "prcm.h"

#include "uart.h"

#include "timer.h"

// common interface includes

#include "network_if.h"

#ifndef NOTERM

#include "uart_if.h"

#endif

#include "button_if.h"

#include "gpio_if.h"

#include "timer_if.h"

#include "common.h"

#include "utils.h"

#include "sl_mqtt_client.h"

// application specific includes

#include "pinmux.h"

#define APPLICATION_VERSION 	"1.1.1"

/*Operate Lib in MQTT 3.1 mode.*/

#define MQTT_3_1_1              false /*MQTT 3.1.1 */

#define MQTT_3_1                true /*MQTT 3.1*/

#define WILL_TOPIC              "Client"

#define WILL_MSG                "Client Stopped"

#define WILL_QOS                QOS2

#define WILL_RETAIN             false

/*Defining Broker IP address and port Number*/

#define SERVER_ADDRESS          "broker.hivemq.com"

#define PORT_NUMBER             1883

#define MAX_BROKER_CONN         1

#define SERVER_MODE             MQTT_3_1

/*Specifying Receive time out for the Receive task*/

#define RCV_TIMEOUT             30

/*Background receive task priority*/

#define TASK_PRIORITY          3

/* Keep Alive Timer value*/

#define KEEP_ALIVE_TIMER        2

/*Clean session flag*/

#define CLEAN_SESSION           true

/*Retain Flag. Used in publish message. */

#define RETAIN                  1

/*Defining Publish Topic*/

#define PUB_TOPIC_FOR_SW3       "/cc3200/ButtonPressEvtSw3"

#define PUB_TOPIC_FOR_SW2       "data"

/*Defining Number of topics*/

#define TOPIC_COUNT             3

/*Defining Subscription Topic Values*/

#define TOPIC1                  "/cc3200/ToggleLEDCmdL1"

#define TOPIC2                  "/cc3200/ToggleLEDCmdL2"

#define TOPIC3                  "/cc3200/ToggleLEDCmdL3"

/*Defining QOS levels*/

#define QOS0                    0

#define QOS1                    1

#define QOS2                    2

/*Spawn task priority and OSI Stack Size*/

#define OSI_STACK_SIZE         2048

#define UART_PRINT              Report

typedef struct connection_config{

SlMqttClientCtxCfg_t broker_config;

void *clt_ctx;

unsigned char *client_id;

unsigned char *usr_name;

unsigned char *usr_pwd;

bool is_clean;

unsigned int keep_alive_time;

SlMqttClientCbs_t CallBAcks;

int num_topics;

char *topic[TOPIC_COUNT];

unsigned char qos[TOPIC_COUNT];

SlMqttWill_t will_params;

bool is_connected;

}connect_config;

typedef enum

{

PUSH_BUTTON_SW2_PRESSED,

PUSH_BUTTON_SW3_PRESSED,

BROKER_DISCONNECTION

}events;

typedef struct

{

void * hndl;

events event;

}event_msg;

//*****************************************************************************

//                      LOCAL FUNCTION PROTOTYPES

//*****************************************************************************

static void

Mqtt_Recv(void *app_hndl, const char  *topstr, long top_len, const void *payload,

long pay_len, bool dup,unsigned char qos, bool retain);

static void sl_MqttEvt(void *app_hndl,long evt, const void *buf,

unsigned long len);

static void sl_MqttDisconnect(void *app_hndl);

void pushButtonInterruptHandler2();

void pushButtonInterruptHandler3();

void ToggleLedState(ledEnum LedNum);

void TimerPeriodicIntHandler(void);

void LedTimerConfigNStart();

void LedTimerDeinitStop();

void BoardInit(void);

static void DisplayBanner(char * AppName);

void MqttClient(void *pvParameters);

void BrokerDisconnect(void *pvParameters);

void Reconnect();

int n;

//*****************************************************************************

//                 GLOBAL VARIABLES -- Start

//*****************************************************************************

#ifdef USE_FREERTOS

#if defined(ewarm)

extern uVectorEntry __vector_table;

#endif

#if defined(ccs)

extern void (* const g_pfnVectors[])(void);

#endif

#endif

unsigned short g_usTimerInts;

/* AP Security Parameters */

SlSecParams_t SecurityParams = {0};

/*Message Queue*/

OsiMsgQ_t g_PBQueue;

int globalConnectedFlag = false;

int ReconnectTask = false;

int globalDisconnectEventFlag = false;

/* connection configuration */

connect_config usr_connect_config[] =

{

{

{

{

SL_MQTT_NETCONN_URL,

SERVER_ADDRESS,

PORT_NUMBER,

0,

0,

0,

NULL

},

SERVER_MODE,

true,

},

NULL,

"user1",

NULL,

NULL,

true,

KEEP_ALIVE_TIMER,

{Mqtt_Recv, sl_MqttEvt, sl_MqttDisconnect},

TOPIC_COUNT,

{TOPIC1, TOPIC2, TOPIC3},

{QOS2, QOS2, QOS2},

{WILL_TOPIC,WILL_MSG,WILL_QOS,WILL_RETAIN},

false

}

};

/* library configuration */

SlMqttClientLibCfg_t Mqtt_Client={

0,

TASK_PRIORITY,

30,

true,

(long(*)(const char *, ...))UART_PRINT

};

/*Publishing topics and messages*/

const char *pub_topic_sw2 = PUB_TOPIC_FOR_SW2;

const char *pub_topic_sw3 = PUB_TOPIC_FOR_SW3;

unsigned char *data_sw2={"0123654789"};

unsigned char *data_sw3={"Push button sw3 is pressed on CC32XX device"};

void *app_hndl = (void*)usr_connect_config;

//*****************************************************************************

//                 GLOBAL VARIABLES -- End

//*****************************************************************************

//****************************************************************************

//! Defines Mqtt_Pub_Message_Receive event handler.

//! Client App needs to register this event handler with sl_ExtLib_mqtt_Init

//! API. Background receive task invokes this handler whenever MQTT Client

//! receives a Publish Message from the broker.

//!

//!\param[out]     topstr => pointer to topic of the message

//!\param[out]     top_len => topic length

//!\param[out]     payload => pointer to payload

//!\param[out]     pay_len => payload length

//!\param[out]     retain => Tells whether its a Retained message or not

//!\param[out]     dup => Tells whether its a duplicate message or not

//!\param[out]     qos => Tells the Qos level

//!

//!\return none

//****************************************************************************

static void

Mqtt_Recv(void *app_hndl, const char  *topstr, long top_len, const void *payload,

long pay_len, bool dup,unsigned char qos, bool retain)

{

char *output_str=(char*)malloc(top_len+1);

memset(output_str,'\0',top_len+1);

strncpy(output_str, (char*)topstr, top_len);

output_str[top_len]='\0';

if(strncmp(output_str,TOPIC1, top_len) == 0)

{

ToggleLedState(LED1);

}

else if(strncmp(output_str,TOPIC2, top_len) == 0)

{

ToggleLedState(LED2);

}

else if(strncmp(output_str,TOPIC3, top_len) == 0)

{

ToggleLedState(LED3);

}

UART_PRINT("\n\rPublish Message Received");

UART_PRINT("\n\rTopic: ");

UART_PRINT("%s",output_str);

free(output_str);

UART_PRINT(" [Qos: %d] ",qos);

if(retain)

UART_PRINT(" [Retained]");

if(dup)

UART_PRINT(" [Duplicate]");

output_str=(char*)malloc(pay_len+1);

memset(output_str,'\0',pay_len+1);

strncpy(output_str, (char*)payload, pay_len);

output_str[pay_len]='\0';

UART_PRINT("\n\rData is: ");

UART_PRINT("%s",(char*)output_str);

UART_PRINT("\n\r");

free(output_str);

return;

}

//****************************************************************************

//! Defines sl_MqttEvt event handler.

//! Client App needs to register this event handler with sl_ExtLib_mqtt_Init

//! API. Background receive task invokes this handler whenever MQTT Client

//! receives an ack(whenever user is in non-blocking mode) or encounters an error.

//!

//! param[out]      evt => Event that invokes the handler. Event can be of the

//!                        following types:

//!                        MQTT_ACK - Ack Received

//!                        MQTT_ERROR - unknown error

//!

//!

//! \param[out]     buf => points to buffer

//! \param[out]     len => buffer length

//!

//! \return none

//****************************************************************************

static void

sl_MqttEvt(void *app_hndl, long evt, const void *buf,unsigned long len)

{

int i;

switch(evt)

{

case SL_MQTT_CL_EVT_PUBACK:

UART_PRINT("PubAck:\n\r");

UART_PRINT("%s\n\r",buf);

break;

case SL_MQTT_CL_EVT_SUBACK:

UART_PRINT("\n\rGranted QoS Levels are:\n\r");

for(i=0;i<len;i++)

{

UART_PRINT("QoS %d\n\r",((unsigned char*)buf)[i]);

}

break;

case SL_MQTT_CL_EVT_UNSUBACK:

UART_PRINT("UnSub Ack \n\r");

UART_PRINT("%s\n\r",buf);

break;

default:

break;

}

}

//****************************************************************************

//

//! callback event in case of MQTT disconnection

//!

//! \param app_hndl is the handle for the disconnected connection

//!

//! return none

//

//****************************************************************************

static void

sl_MqttDisconnect(void *app_hndl)

{

// connect_config *local_con_conf;

// local_con_conf = app_hndl;

connect_config *local_con_conf = (connect_config *)app_hndl;

//    UART_PRINT("disconnect from broker %s\r\n",

//       (local_con_conf->broker_config).server_info.server_addr);

local_con_conf->is_connected = false;

globalDisconnectEventFlag = true;

}

//****************************************************************************

//

//! Push Button Handler1(GPIOS2). Press push button2 (GPIOSW2) Whenever user

//! wants to publish a message. Write message into message queue signaling the

//!    event publish messages

//!

//! \param none

//!

//! return none

//

//****************************************************************************

void pushButtonInterruptHandler2()

{

event_msg msg;

msg.event = PUSH_BUTTON_SW2_PRESSED;

msg.hndl = NULL;

//

// write message indicating publish message

//

osi_MsgQWrite(&g_PBQueue,&msg,OSI_NO_WAIT);

}

//****************************************************************************

//

//! Push Button Handler3(GPIOS3). Press push button3 (GPIOSW3) Whenever user

//! wants to publish a message. Write message into message queue signaling the

//!    event publish messages

//!

//! \param none

//!

//! return none

//

//****************************************************************************

void pushButtonInterruptHandler3()

{

event_msg msg;

msg.event = PUSH_BUTTON_SW3_PRESSED;

msg.hndl = NULL;

//

// write message indicating exit from sending loop

//

osi_MsgQWrite(&g_PBQueue,&msg,OSI_NO_WAIT);

}

//****************************************************************************

//

//!    Toggles the state of GPIOs(LEDs)

//!

//! \param LedNum is the enumeration for the GPIO to be toggled

//!

//!    \return none

//

//****************************************************************************

void ToggleLedState(ledEnum LedNum)

{

unsigned char ledstate = 0;

switch(LedNum)

{

case LED1:

ledstate = GPIO_IF_LedStatus(MCU_RED_LED_GPIO);

if(!ledstate)

{

GPIO_IF_LedOn(MCU_RED_LED_GPIO);

}

else

{

GPIO_IF_LedOff(MCU_RED_LED_GPIO);

}

break;

case LED2:

ledstate = GPIO_IF_LedStatus(MCU_ORANGE_LED_GPIO);

if(!ledstate)

{

GPIO_IF_LedOn(MCU_ORANGE_LED_GPIO);

}

else

{

GPIO_IF_LedOff(MCU_ORANGE_LED_GPIO);

}

break;

case LED3:

ledstate = GPIO_IF_LedStatus(MCU_GREEN_LED_GPIO);

if(!ledstate)

{

GPIO_IF_LedOn(MCU_GREEN_LED_GPIO);

}

else

{

GPIO_IF_LedOff(MCU_GREEN_LED_GPIO);

}

break;

default:

break;

}

}

//*****************************************************************************

//

//! Periodic Timer Interrupt Handler

//!

//! \param None

//!

//! \return None

//

//*****************************************************************************

void

TimerPeriodicIntHandler(void)

{

unsigned long ulInts;

//

// Clear all pending interrupts from the timer we are

// currently using.

//

ulInts = MAP_TimerIntStatus(TIMERA0_BASE, true);

MAP_TimerIntClear(TIMERA0_BASE, ulInts);

//

// Increment our interrupt counter.

//

g_usTimerInts++;

if(!(g_usTimerInts & 0x1))

{

//

// Off Led

//

GPIO_IF_LedOff(MCU_RED_LED_GPIO);

}

else

{

//

// On Led

//

GPIO_IF_LedOn(MCU_RED_LED_GPIO);

}

}

//****************************************************************************

//

//! Function to configure and start timer to blink the LED while device is

//! trying to connect to an AP

//!

//! \param none

//!

//! return none

//

//****************************************************************************

void LedTimerConfigNStart()

{

//

// Configure Timer for blinking the LED for IP acquisition

//

Timer_IF_Init(PRCM_TIMERA0,TIMERA0_BASE,TIMER_CFG_PERIODIC,TIMER_A,0);

Timer_IF_IntSetup(TIMERA0_BASE,TIMER_A,TimerPeriodicIntHandler);

Timer_IF_Start(TIMERA0_BASE,TIMER_A,100);

}

//****************************************************************************

//

//! Disable the LED blinking Timer as Device is connected to AP

//!

//! \param none

//!

//! return none

//

//****************************************************************************

void LedTimerDeinitStop()

{

//

// Disable the LED blinking Timer as Device is connected to AP

//

Timer_IF_Stop(TIMERA0_BASE,TIMER_A);

Timer_IF_DeInit(TIMERA0_BASE,TIMER_A);

}

//*****************************************************************************

//

//! Board Initialization & Configuration

//!

//! \param  None

//!

//! \return None

//

//*****************************************************************************

void BoardInit(void)

{

/* In case of TI-RTOS vector table is initialize by OS itself */

#ifndef USE_TIRTOS

//

// Set vector table base

//

#if defined(ccs)

IntVTableBaseSet((unsigned long)&g_pfnVectors[0]);

#endif

#if defined(ewarm)

IntVTableBaseSet((unsigned long)&__vector_table);

#endif

#endif

//

// Enable Processor

//

MAP_IntMasterEnable();

MAP_IntEnable(FAULT_SYSTICK);

PRCMCC3200MCUInit();

}

//*****************************************************************************

//

//! Application startup display on UART

//!

//! \param  none

//!

//! \return none

//!

//*****************************************************************************

static void

DisplayBanner(char * AppName)

{

UART_PRINT("\n\n\n\r");

UART_PRINT("\t\t *************************************************\n\r");

UART_PRINT("\t\t    CC3200 %s Application       \n\r", AppName);

UART_PRINT("\t\t *************************************************\n\r");

UART_PRINT("\n\n\n\r");

}

extern volatile unsigned long g_ulStatus;

//*****************************************************************************

//

//! Task implementing MQTT client communication to other web client through

//!    a broker

//!

//! \param  none

//!

//! This function

//!    1. Initializes network driver and connects to the default AP

//!    2. Initializes the mqtt library and set up MQTT connection configurations

//!    3. set up the button events and their callbacks(for publishing)

//!    4. handles the callback signals

//!

//! \return None

//!

//*****************************************************************************

void MqttClient(void *pvParameters)

{

long lRetVal = -1;

int iCount = 0;

int iNumBroker = 0;

int iConnBroker = 0;

event_msg RecvQue;

unsigned char policyVal;

connect_config *local_con_conf = (connect_config *)app_hndl;

//

// Configure LED

//

GPIO_IF_LedConfigure(LED1|LED2|LED3);

GPIO_IF_LedOff(MCU_RED_LED_GPIO);

GPIO_IF_LedOff(MCU_GREEN_LED_GPIO);

//

// Reset The state of the machine

//

Network_IF_ResetMCUStateMachine();

//

// Start the driver

//

lRetVal = Network_IF_InitDriver(ROLE_STA);

if(lRetVal < 0)

{

UART_PRINT("Failed to start SimpleLink Device\n\r",lRetVal);

LOOP_FOREVER();

}

// switch on Green LED to indicate Simplelink is properly up

GPIO_IF_LedOn(MCU_ON_IND);

// Start Timer to blink Red LED till AP connection

LedTimerConfigNStart();

// Initialize AP security params

SecurityParams.Key = (signed char *)SECURITY_KEY;

SecurityParams.KeyLen = strlen(SECURITY_KEY);

SecurityParams.Type = SECURITY_TYPE;

//

// Connect to the Access Point

//

/* lRetVal = Network_IF_ConnectAP(SSID_NAME, SecurityParams);

if(lRetVal < 0)

{

UART_PRINT("Connection to an AP failed\n\r");

LOOP_FOREVER();

}*/

lRetVal = sl_WlanProfileAdd(SSID_NAME,strlen(SSID_NAME),0,&SecurityParams,0,1,0);

//set AUTO policy

lRetVal = sl_WlanPolicySet(SL_POLICY_CONNECTION,

SL_CONNECTION_POLICY(1,0,0,0,0),

&policyVal, 1 /*PolicyValLen*/);

while(!(IS_CONNECTED(g_ulStatus)) || !(IS_IP_ACQUIRED(g_ulStatus)))

{

UART_PRINT("Waiting to Connect to AP ...\n\r");

UtilsDelay(20000000); // delay for testing, but better to use a more power efficient delay.

}

//

// Disable the LED blinking Timer as Device is connected to AP

//

LedTimerDeinitStop();

//

// Switch ON RED LED to indicate that Device acquired an IP

//

GPIO_IF_LedOn(MCU_IP_ALLOC_IND);

UtilsDelay(20000000);

GPIO_IF_LedOff(MCU_RED_LED_GPIO);

GPIO_IF_LedOff(MCU_ORANGE_LED_GPIO);

GPIO_IF_LedOff(MCU_GREEN_LED_GPIO);

//

// Register Push Button Handlers

//

Button_IF_Init(pushButtonInterruptHandler2,pushButtonInterruptHandler3);

//

// Initialze MQTT client lib

//

lRetVal = sl_ExtLib_MqttClientInit(&Mqtt_Client);

if(lRetVal != 0)

{

// lib initialization failed

UART_PRINT("MQTT Client lib initialization failed\n\r");

LOOP_FOREVER();

}

/******************* connection to the broker ***************************/

iNumBroker = sizeof(usr_connect_config)/sizeof(connect_config);

if(iNumBroker > MAX_BROKER_CONN)

{

UART_PRINT("Num of brokers are more then max num of brokers\n\r");

LOOP_FOREVER();

}

//while(1)

{

connect_to_broker:

while(iCount < iNumBroker)

{

//create client context

local_con_conf[iCount].clt_ctx =

sl_ExtLib_MqttClientCtxCreate(&local_con_conf[iCount].broker_config,

&local_con_conf[iCount].CallBAcks,

&(local_con_conf[iCount]));

//

// Set Client ID

//

sl_ExtLib_MqttClientSet((void*)local_con_conf[iCount].clt_ctx,

SL_MQTT_PARAM_CLIENT_ID,

local_con_conf[iCount].client_id,

strlen((char*)(local_con_conf[iCount].client_id)));

//

// Set will Params

//

if(local_con_conf[iCount].will_params.will_topic != NULL)

{

sl_ExtLib_MqttClientSet((void*)local_con_conf[iCount].clt_ctx,

SL_MQTT_PARAM_WILL_PARAM,

&(local_con_conf[iCount].will_params),

sizeof(SlMqttWill_t));

}

//

// setting username and password

//

if(local_con_conf[iCount].usr_name != NULL)

{

sl_ExtLib_MqttClientSet((void*)local_con_conf[iCount].clt_ctx,

SL_MQTT_PARAM_USER_NAME,

local_con_conf[iCount].usr_name,

strlen((char*)local_con_conf[iCount].usr_name));

if(local_con_conf[iCount].usr_pwd != NULL)

{

sl_ExtLib_MqttClientSet((void*)local_con_conf[iCount].clt_ctx,

SL_MQTT_PARAM_PASS_WORD,

local_con_conf[iCount].usr_pwd,

strlen((char*)local_con_conf[iCount].usr_pwd));

}

}

//

// connectin to the broker

//

if((sl_ExtLib_MqttClientConnect((void*)local_con_conf[iCount].clt_ctx,

local_con_conf[iCount].is_clean,

local_con_conf[iCount].keep_alive_time) & 0xFF) != 0)

{

UART_PRINT("\n\rBroker connect fail for conn no. %d \n\r",iCount+1);

// GPIO_IF_LedOn(MCU_GREEN_LED_GPIO);

//delete the context for this connection

sl_ExtLib_MqttClientCtxDelete(local_con_conf[iCount].clt_ctx);

continue;

// break;

}

else

{

UART_PRINT("\n\rSuccess: conn to Broker no. %d\n\r ", iCount+1);

local_con_conf[iCount].is_connected = true;

//   GPIO_IF_LedOff(MCU_GREEN_LED_GPIO);

//  globalConnectedFlag = false;

iConnBroker++;

}

//

// Subscribe to topics

//

if(sl_ExtLib_MqttClientSub((void*)local_con_conf[iCount].clt_ctx,

local_con_conf[iCount].topic,

local_con_conf[iCount].qos, TOPIC_COUNT) < 0)

{

UART_PRINT("\n\r Subscription Error for conn no. %d\n\r", iCount+1);

UART_PRINT("Disconnecting from the broker\r\n");

sl_ExtLib_MqttClientDisconnect(local_con_conf[iCount].clt_ctx);

local_con_conf[iCount].is_connected = false;

//delete the context for this connection

sl_ExtLib_MqttClientCtxDelete(local_con_conf[iCount].clt_ctx);

iConnBroker--;

continue;

//break;

}

else

{

int iSub;

UART_PRINT("Client subscribed on following topics:\n\r");

for(iSub = 0; iSub < local_con_conf[iCount].num_topics; iSub++)

{

UART_PRINT("%s\n\r", local_con_conf[iCount].topic[iSub]);

}

}

iCount++;

}

if(iConnBroker < 1)

{

//

// no succesful connection to broker

//

goto connect_to_broker;

//break;

}

iCount = 0;

globalConnectedFlag = true;

GPIO_IF_LedOff(MCU_GREEN_LED_GPIO);

for(;;)

{

sl_ExtLib_MqttClientSend((void*)local_con_conf[iCount].clt_ctx,

pub_topic_sw2,data_sw2,strlen((char*)data_sw2),QOS2,RETAIN);

}

}

}

void BrokerDisconnect(void *pvParameters)

{

long lRetVal = -1;

int iCount = 0;

int iNumBroker = 0;

int iConnBroker = 0;

event_msg RecvQue;

unsigned char policyVal;

connect_config *local_con_conf = (connect_config *)app_hndl;

for(;;)

{

if(globalDisconnectEventFlag == true)

{

GPIO_IF_LedOn(MCU_GREEN_LED_GPIO);

globalDisconnectEventFlag = false;

globalConnectedFlag = false;

while(!(globalConnectedFlag ==true))

{

sl_ExtLib_MqttClientCtxDelete(local_con_conf[iCount].clt_ctx);

Reconnect();

}

}

osi_Sleep(100);

}

}

void Reconnect()

{

long lRetVal = -1;

int iCount = 0;

int iNumBroker = 0;

int iConnBroker = 0;

unsigned char policyVal;

connect_config *local_con_conf = (connect_config *)app_hndl;

/******************* connection to the broker ***************************/

iNumBroker = sizeof(usr_connect_config)/sizeof(connect_config);

if(iNumBroker > MAX_BROKER_CONN)

{

UART_PRINT("Num of brokers are more then max num of brokers\n\r");

LOOP_FOREVER();

}

connect_to_broker:

//   while(iCount < iNumBroker)

{

//create client context

local_con_conf[iCount].clt_ctx =

sl_ExtLib_MqttClientCtxCreate(&local_con_conf[iCount].broker_config,

&local_con_conf[iCount].CallBAcks,

&(local_con_conf[iCount]));

//

// Set Client ID

//

sl_ExtLib_MqttClientSet((void*)local_con_conf[iCount].clt_ctx,

SL_MQTT_PARAM_CLIENT_ID,

local_con_conf[iCount].client_id,

strlen((char*)(local_con_conf[iCount].client_id)));

//

// Set will Params

//

if(local_con_conf[iCount].will_params.will_topic != NULL)

{

sl_ExtLib_MqttClientSet((void*)local_con_conf[iCount].clt_ctx,

SL_MQTT_PARAM_WILL_PARAM,

&(local_con_conf[iCount].will_params),

sizeof(SlMqttWill_t));

}

//

// setting username and password

//

if(local_con_conf[iCount].usr_name != NULL)

{

sl_ExtLib_MqttClientSet((void*)local_con_conf[iCount].clt_ctx,

SL_MQTT_PARAM_USER_NAME,

local_con_conf[iCount].usr_name,

strlen((char*)local_con_conf[iCount].usr_name));

if(local_con_conf[iCount].usr_pwd != NULL)

{

sl_ExtLib_MqttClientSet((void*)local_con_conf[iCount].clt_ctx,

SL_MQTT_PARAM_PASS_WORD,

local_con_conf[iCount].usr_pwd,

strlen((char*)local_con_conf[iCount].usr_pwd));

}

}

//

// connectin to the broker

//

if((sl_ExtLib_MqttClientConnect((void*)local_con_conf[iCount].clt_ctx,

local_con_conf[iCount].is_clean,

local_con_conf[iCount].keep_alive_time) & 0xFF) != 0)

{

UART_PRINT("\n\rBroker connect fail for conn no. %d \n\r",iCount+1);

//delete the context for this connection

sl_ExtLib_MqttClientCtxDelete(local_con_conf[iCount].clt_ctx);

// break;

// continue;

}

else

{

UART_PRINT("\n\rSuccess: conn to Broker no. %d\n\r ", iCount+1);

local_con_conf[iCount].is_connected = true;

globalConnectedFlag = true;

GPIO_IF_LedOff(MCU_GREEN_LED_GPIO);

iConnBroker++;

}

//

// Subscribe to topics

//

if(sl_ExtLib_MqttClientSub((void*)local_con_conf[iCount].clt_ctx,

local_con_conf[iCount].topic,

local_con_conf[iCount].qos, TOPIC_COUNT) < 0)

{

UART_PRINT("\n\r Subscription Error for conn no. %d\n\r", iCount+1);

UART_PRINT("Disconnecting from the broker\r\n");

//delete the context for this connection

sl_ExtLib_MqttClientCtxDelete(local_con_conf[iCount].clt_ctx);

iConnBroker--;

///    continue;

//   break;

}

else

{

int iSub;

UART_PRINT("Client subscribed on following topics:\n\r");

for(iSub = 0; iSub < local_con_conf[iCount].num_topics; iSub++)

{

UART_PRINT("%s\n\r", local_con_conf[iCount].topic[iSub]);

}

}

iCount++;

}

iCount = 0;

}

//*****************************************************************************

//

//! Main

//!

//! \param  none

//!

//! This function

//!    1. Invokes the SLHost task

//!    2. Invokes the MqttClient

//!

//! \return None

//!

//*****************************************************************************

void main()

{

long lRetVal = -1;

//

// Initialize the board configurations

//

BoardInit();

//

// Pinmux for UART

//

PinMuxConfig();

//

// Configuring UART

//

InitTerm();

//

// Display Application Banner

//

DisplayBanner("MQTT_Client");

//

// Start the SimpleLink Host

//

lRetVal = VStartSimpleLinkSpawnTask(SPAWN_TASK_PRIORITY);

if(lRetVal < 0)

{

ERR_PRINT(lRetVal);

LOOP_FOREVER();

}

//

// Start the MQTT Client task

//

osi_MsgQCreate(&g_PBQueue,"PBQueue",sizeof(event_msg),10);

lRetVal = osi_TaskCreate(MqttClient,

(const signed char *)"Mqtt Client App",

OSI_STACK_SIZE, NULL, 2, NULL );

if(lRetVal < 0)

{

ERR_PRINT(lRetVal);

LOOP_FOREVER();

}

lRetVal = osi_TaskCreate(BrokerDisconnect,

(const signed char *)"BrokerDisconnect App",

OSI_STACK_SIZE, NULL, 2, NULL );

if(lRetVal < 0)

{

ERR_PRINT(lRetVal);

LOOP_FOREVER();

}

//

// Start the task scheduler

//

osi_start();

}